# ΣΧΟΛΗ ΗΛΕΚΤΡΟΛΟΓΩΝ ΜΗΧΑΝΙΚΩΝ

# ΚΑΙ ΜΗΧΑΝΙΚΩΝ ΥΠΟΛΟΓΙΣΤΩΝ

ΤΟΜΕΑΣ ΤΕΧΝΟΛΟΓΙΑΣ ΠΛΗΡΟΦΟΡΙΚΗΣ ΚΑΙ ΥΠΟΛΟΓΙΣΤΩΝ

ΕΘΝΙΚΟ ΜΕΤΣΟΒΙΟ ΠΟΛΥΤΕΧΝΕΙΟ

# Διερεύνηση πρωτοκόλλου J1850 PWM και σχεδίαση διεπαφής επικοινωνίας με επεξεργαστή οχήματος

# Διπλωματική Εργασία

# Σπύρογλου Γεώργιος

# Επιβλέπων : Ιωάννης Αβαριτσιώτης

# Καθηγητής Ε.Μ.Π

# Αθήνα, Ιούλιος 2008

# ΣΧΟΛΗ ΗΛΕΚΤΡΟΛΟΓΩΝ ΜΗΧΑΝΙΚΩΝ

# ΚΑΙ ΜΗΧΑΝΙΚΩΝ ΥΠΟΛΟΓΙΣΤΩΝ

ΤΟΜΕΑΣ ΤΕΧΝΟΛΟΓΙΑΣ ΠΛΗΡΟΦΟΡΙΚΗΣ ΚΑΙ ΥΠΟΛΟΓΙΣΤΩΝ

ΕΘΝΙΚΟ ΜΕΤΣΟΒΙΟ ΠΟΛΥΤΕΧΝΕΙΟ

# Διερεύνηση πρωτοκόλλου J1850 PWM και σχεδίαση διεπαφής επικοινωνίας με επεξεργαστή οχήματος

# Διπλωματική Εργασία

Σπύρογλου Γεώργιος

# Επιβλέπων : Ιωάννης Αβαριτσιώτης

Καθηγητής Ε.Μ.Π

# Εγκρίθηκε από την τριμελή εξεταστική επιτροπή την ...... Ιουλίου 2008

.........                     .........                   .........

Ιωάννης Αβαριτσιώτης             Ελευθέριος Καγιάφας              Βασίλειος Λούμος

Καθηγητής Ε.Μ.Π.                    Καθηγητής Ε.Μ.Π.                 Καθηγητής Ε.Μ.Π.

# Αθήνα, Ιούλιος 2008

# Copyright © Σπύρογλου Γεώργιου 2008

Με επιφύλαξη παντός δικαιώματος. All rights reserved.

Απαγορεύεται η αντιγραφή, αποθήκευση και διανομή της παρούσας εργασίας, εξ’ ολοκλήρου ή τμήματος αυτής, για εμπορικό σκοπό. Επιτρέπεται η ανατύπωση, αποθήκευση και διανομή για σκοπό μη κερδοσκοπικό, εκπαιδευτικής ή ερευνητικής φύσης, υπό την προϋπόθεση να αναφέρεται η πηγή προέλευσης και να διατηρείται το παρόν μήνυμα. Ερωτήματα που αφορούν τη χρήση της εργασίας για κερδοσκοπικό σκοπό πρέπει να απευθύνονται προς τον συγγραφέα.

Οι απόψεις και τα συμπεράσματα που περιέχονται σε αυτό το έγγραφο εκφράζουν τον συγγραφέα και δεν πρέπει να ερμηνευθεί ότι αντιπροσωπεύουν τις επίσημες θέσεις του Εθνικού Μετσόβιου Πολυτεχνείου.

# Περίληψη

Η παρούσα αναφορά περιγράφει τη διερεύνηση του πρωτοκόλλου J1850 PWM καθώς και το σχεδιασμό και την υλοποίηση δύο σταδίων:

Α) Το πρώτο είναι ένα ηλεκτρονικό κύκλωμα το οποίο ανακτά OBD (On – Board Diagnostics) δεδομένα συνδεόμενο με αντάπτορα στο αυτοκίνητο και με σειριακή θύρα RS-232 στον υπολογιστή.

Β) Το δεύτερο είναι ένα γραφικό περιβάλλον (GUI – Graphic User Interface) το οποίο υλοποιήθηκε σε γλώσσα Java, με τη βοήθεια του οποίου αποτυπώνονται γραφικά τα αποτελέσματα που ανακτώνται από τη σύνδεση της πλακέτας που περιγράφηκε παραπάνω, με το ηλεκτρονικό σύστημα του αυτοκινήτου.

Συνοπτικά η διάρθρωση της αναφοράς είναι η ακόλουθη:

- Το 1ο Κεφάλαιο αποτελεί μία συνολική επισκόπηση του αντικειμένου που πραγματεύεται η συγκεκριμένη εργασία.

- Στο 2ο Κεφάλαιο πραγματοποιείται μία ιστορική αναδρομή της οποίας σκοπός είναι να εξοικειωθεί ο αναγνώστης με τις διαδικασίες που προηγήθηκαν μέχρι να φτάσουμε στα σημερινά πρωτόκολλα και όλα εκείνα τα γεγονότα που ώθησαν στην εξέλιξη των πρωτοκόλλων και την ανάγκη δημιουργίας νέων συστημάτων υλοποίησης τους.

- Στο 3ο Κεφάλαιο περιγράφεται αναλυτικά το πρωτόκολλο SAE J1850 PWM για το οποίο σχεδιάστηκε το ηλεκτρονικό κύκλωμα στο οποίο έχει γίνει ήδη αναφορά. Αξίζει να αναφερθεί ότι το κύκλωμα υποστηρίζει και άλλα πρωτόκολλα τα οποία όμως δεν είναι αντικείμενο της παρούσας αναφοράς.

- Στο 4ο Κεφάλαιο πραγματοποιείται η περιγραφή της διαδικασίας που ακολουθήθηκε για την υλοποίηση του ηλεκτρονικού κυκλώματος και του γραφικού περιβάλλοντος και παρουσιάζονται τα αποτελέσματα από την εφαρμογή της διάταξης.

- Στο 5ο Κεφάλαιο αξιολογείται η ακρίβεια των αποτελεσμάτων και η ακρίβεια και η λειτουργικότητα τόσο του κυκλώματος όσο και του γραφικού περιβάλλοντος που αναπτύχθηκε. Παράλληλα, προτείνονται τρόποι βελτίωσης και παρατίθενται προτάσεις για μελλοντικές εξελίξεις.

- Ακολουθεί παράρτημα όπου αναφέρονται οι βασικότεροι κωδικοί σφαλμάτων με την επεξήγησή τους και τέλος γίνεται αναφορά στη βιβλιογραφία που χρησιμοποιήθηκε.

Λέξεις-Κλειδιά: εργαλείο σάρωσης, πρωτόκολλο, διάγνωση, κωδικοί σφάλματος, εκπομπές καυσαερίων, αυτοκίνητο, γραφικό περιβάλλον.

# Abstract

This essay describes the design procedure and the implementation in two basic stages:

- Firstly an electronic circuit that obtains OBD (On – Board Diagnostics) data from the car connector and transmits it to the PC through an RS-232 port.

- Secondly a GUI (Graphic User Interface), implemented in Java, which plots the results that are obtained from the vehicle through the board.

The structure of the essay is the following:

- The 1st Chapter is an overall review of the main subject theme of this essay.

- In the 2nd Chapter a flashback is made to familiarize the reader with the procedures that led to the today protocols and created the necessity for new methods for data acquisition in vehicles.

- In the 3rd Chapter the SAE J1850 PWM protocol is described for which the electronic circuit, which was mentioned above, was designed. At this point it should be noticed that this circuit can easily support other OBD protocols which are beyond the scope of this thesis.

- The 4th Chapter includes the description of the procedure that was followed in order to implement the electronic circuit and the GUI. The reader can also find the results which derived from the testing of the applications.

- In the 5th Chapter an evaluation of the results is made and there are also some conclusions as well as suggestions for possible future evolution.

- In the Appendix the most fundamental Trouble Codes are included with their explanation which is followed by the bibliography that was used for this report.

Keywords: OBD 2, scan tool, SAE J1850, PWM, diagnostics, automotive, protocol, trouble codes, MIL, gas emission, graphic user interface.

ΠΙΝΑΚΑΣ ΠΕΡΙΕΧΟΜΕΝΩΝ

ΚΕΦΑΛΑΙΟ 1 – ΠΡΟΛΟΓΟΣ

1.1 Εισαγωγή 12

ΚΕΦΑΛΑΙΟ 2 – ΙΣΤΟΡΙΚΗ ΑΝΑΔΡΟΜΗ

2.1 On Board Diagnostics – Generation 1 15

2.1.1 Ενδεικτική λυχνία βλαβών (MIL) 16

2.1.2 Κωδικοί διάγνωσης βλαβών 16

2.1.3 Ακολουθίες σειριακών δεδομένων 17

2.2 On Board Diagnostics – Generation 2 (OBD II) 18

2.2.1 Χαρακτηριστικά του OBD II 19

2.2.1.1 Διάγνωση που αφορά τον αισθητήρα οξυγόνου 19

2.2.1.2 Σύστημα καταγραφής καυσίμου19

2.2.1.3 Καταγραφή αφλογιστίας 19

2.2.1.4 Απεικόνιση του καταλύτη 21

2.2.1.5 ΕGR Σύστημα καταγραφής 21

2.2.1.6 Καταγραφή συστήματος καθαρισμού εξάτμισης 22

2.2.1.7 Δευτερεύον σύστημα καταγραφής αέρα 22

2.2.1.8 Ενεργοποίηση Ενδεικτικής Λυχνίας Δυσλειτουργιών (MIL) 22

2.2.1.9 Έλεγχος ετοιμότητας 23

2.2.1.10 Αποθηκευμένα δεδομένα του κινητήρα 24

2.2.1.11 Τυποποίηση των πληροφοριών επιδιόρθωσης και των κωδικών ανίχνευσης βλαβών25

ΚΕΦΑΛΑΙΟ 3 – ΤΟ ΠΡΩΤΟΚΟΛΛΟ SAE J1850 PWM

3.1 Γενικά 29

3.2 Τοπολογία Δικτύου – Data Bus Topology 29

3.3 Έλεγχος του δίαυλου 29

3.4 Διαστρωμάτωση σε επίπεδα 30

3.4.1 Επίπεδο Εφαρμογής – Application Layer 30

3.4.2 Ενδιάμεσο Επίπεδο Μετατροπής – Data Link Layer 33

3.4.2.1 Ανίχνευση Σφάλματος 38

3.4.2.1.1 Cyclic Redundancy Check (CRC) 38

3.4.2.1.2 Μήκος Πλαισίου/Μηνύματος 40

3.4.2.1.3 Δεδομένα εκτός Ορίων – Out-of-Range – Data 40

3.4.2.1.4 Ανίχνευση Μη Έγκυρων Bits 41

3.4.2.1.5 Ανίχνευση Μη Έγκυρης Δομής Πλαισίου 41

3.4.2.2 Αντιμετώπιση Σφαλμάτων 41

3.4.3  Φυσικό Επίπεδο – Physical Layer 41

3.4.3.1 Υλικό Φυσικού Επιπέδου 42

3.4.3.2 Προδιαγραφές Μονάδας Φορτίου 42

3.4.3.3 Μέγιστος Αριθμός Κόμβων 42

3.4.3.4 Μέγιστο Μήκος Δικτύου 42

3.4.3.5 Bit Δεδομένων 43

3.4.3.6 Απαιτήσεις Χρονισμού των Συμβόλων 47

3.4.3.7 Αφύπνιση Κόμβων μέσω του Φυσικού Επιπέδου 51

3.4.3.8 Σφάλματα του Φυσικού Επιπέδου 52

3.5  Υλοποίηση Δικτύου – Network Implementation 54

ΚΕΦΑΛΑΙΟ 4 – ΠΕΡΙΓΡΑΦΗ ΤΗΣ ΔΙΑΔΙΚΑΣΙΑΣ

4.1 Σχεδιασμός και ανάλυση του ηλεκτρονικού κυκλώματος 57

4.1.1	Μικροελεγκτής 57

4.1.2	Διασύνδεση J1850 PWM 58

4.1.3	Διασύνδεση RS232 59

4.1.4	Διασύνδεση CAN και ISO-9141 60

4.1.5	Διασύνδεση με το όχημα 60

4.1.6	Συνολικό Κύκλωμα 62

4.2 Υλοποίηση του γραφικού περιβάλλοντος (GUI) 65

ΚΕΦΑΛΑΙΟ 5 – ΑΠΟΤΕΛΕΣΜΑΤΑ, ΣΥΜΠΕΡΑΣΜΑΤΑ ΚΑΙ ΜΕΛΛΟΝΤΙΚΕΣ ΕΞΕΛΙΞΕΙΣ

5.1 Δοκιμή και Αποτελέσματα 73

5.2 Προτάσεις για μελλοντική εξέλιξη 73

ΠΑΡΑΡΤΗΜΑ Α 77

ΠΑΡΑΡΤΗΜΑ Β 105

ΒΙΒΛΙΟΓΡΑΦΙΑ 155

ΠΡΟΛΟΓΟΣ

  - Εισαγωγή

Στην παρούσα εργασία υλοποιήθηκε ένα ηλεκτρονικό κύκλωμα το οποίο συνδέεται μέσω αντάπτορα στο Data Link Connector του οχήματος και μέσω σειριακής θύρας RS – 232 σε υπολογιστή. Παράλληλα, υλοποιήθηκε σε περιβάλλον NetBeans χρησιμοποιώντας Java, ένα γραφικό περιβάλλον με τη χρήση του οποίου μπορεί ο χρήστης να ανακτήσει τις βλάβες που έχουν καταγραφεί στο σύστημα καθώς επίσης να δει γραφικά τις στροφές του κινητήρα σε πραγματικό χρόνο. Επιπρόσθετα, ο χρήστης μπορεί να διαγράψει κωδικούς σφαλμάτων που έχουν αποθηκευθεί, είτε αυτοί έχουν επιδιορθωθεί είτε όχι αν και είναι ασφαλέστερο οι κωδικοί να μη διαγράφονται πριν αποκατασταθεί η βλάβη. Την υλοποίηση όλων αυτών των λειτουργιών την επιτρέπει το ενσωματωμένο πρωτόκολλο SAE J1850 PWM το οποίο απαντάται σε πολλά οχήματα προηγμένης τεχνολογίας.

Τα πλεονεκτήματα της συγκεκριμένης υλοποίησης σε σχέση με τις παραδοσιακές μεθόδους ανίχνευσης βλαβών στα οχήματα είναι τα ακόλουθα:

- Η συλλογή των απαραίτητων πληροφοριών είναι μία διαδικασία απλή, σύντομη, ακριβής που δε χρειάζεται εξειδικευμένες γνώσεις. Και αυτό γιατί με τη χρήση ενός υπολογιστή και χωρίς πολύμετρα και συνεχείς μετρήσεις που απαιτούσαν παλιότερες μέθοδοι, γίνεται καταγραφή των βλαβών οι οποίες έχουν προηγουμένως ανιχνευθεί απλά και έγκαιρα με το φωτισμό μίας ενδεικτικής λυχνίας η οποία ενεργοποιείται σε περίπτωση βλάβης.

- Η αντιστοίχηση των βλαβών σε σφάλματα βοηθάει στην καταγραφή τους και την τυποποίηση κάποιου στάνταρντ εγχειριδίου για την αντιμετώπισή τους. Παράλληλα ο χρήστης έχει την επιλογή να μη διαγραφεί τον κωδικό του σφάλματος από τη μνήμη μετά την αντιμετώπισή του προκειμένου να κρατήσει ιστορικό για το συγκεκριμένο όχημα.

- Όλη η διαδικασία γίνεται σε πραγματικό χρόνο πάνω στο όχημα δίνοντας έτσι τη δυνατότητα στο χρήστη να συλλέξει και άλλες πληροφορίες που πιθανόν να τον ενδιαφέρουν όσο το όχημα είναι σε λειτουργία. Οι πληροφορίες αυτές μπορεί να αφορούν στροφές, ταχύτητα κλπ.

- Λόγω της ευκολίας εφαρμογής της μεθόδου οι βλάβες εντοπίζονται γρηγορότερα και συνεπώς αποφεύγονται όλες οι δυσάρεστες συνέπειες που έχει η μη έγκαιρη διάγνωση και αντιμετώπισή τους όπως είναι για παράδειγμα η μόλυνση του περιβάλλοντος σε περιπτώσεις που υπάρχει βλάβη σε οποιοδήποτε σύστημα σχετίζεται με την εκπομπή καυσαερίων. Αυτός ο τελευταίος λόγος και τα σύγχρονα περιβαλλοντικά προβλήματα υπήρξαν πολλές φορές κινητήριος δύναμη για περαιτέρω έρευνα και βελτίωση.

Στη συνέχεια της αναφοράς περιγράφονται τόσο οι διαδικασίες που μας οδήγησαν στην εφαρμογή και την εξέλιξη των σημερινών πρωτοκόλλων όσο και οι διαδικασίες που ακολουθήθηκαν για την ανάπτυξη των εφαρμογών που υλοποιήσαμε και περιγράφηκαν περιληπτικά παραπάνω.

# ΙΣΤΟΡΙΚΗ ΑΝΑΔΡΟΜΗ

2.1	On Board Diagnostics – Generation 1

Τον Απρίλιο του 1985, το Συμβούλιο Διαχείρισης Εναέριων Πόρων της Καλιφόρνια, ενέκρινε το On-Board system regulations (σύστημα κανόνων εφαρμοσμένο πάνω στο όχημα) (OBD) το οποίο εφαρμόστηκε σε όλα σχεδόν τα αυτοκίνητα και τα ελαφρά οχήματα που κυκλοφόρησαν στην αγορά της Καλιφόρνια μετά το 1988. Οι συγκεκριμένοι κανονισμοί απαιτούν το Engine Control Module (ECM) (μονάδα ελέγχου κινητήρα)  να καταγράφει τη λειτουργία  των εξαρτημάτων που σχετίζονται με την εκπομπή καυσαερίων και να ελέγχει τη φυσιολογική λειτουργία τους, ενεργοποιώντας μία ενδεικτική λυχνία στον πίνακα των οργάνων όταν ανιχνεύεται κάποια βλάβη. Το σύστημα αυτό παρέχει επιπρόσθετα και ένα σύνολο κωδικών διάγνωσης βλαβών, καθώς και πίνακες για την ταυτοποίηση βλαβών στο εγχειρίδιο των επισκευών για να βοηθάει τους τεχνικούς να προσδιορίσουν την πιθανή αιτία για τη βλάβη που παρουσιάστηκε στον έλεγχο του κινητήρα και στο σύστημα εκπομπών. Οι βασικοί στόχοι αυτού του συστήματος κανόνων είναι οι ακόλουθοι:

- Να βελτιώσει τις εκπομπές κατά τη λειτουργία του οχήματος ειδοποιώντας το χειριστή του όταν υπάρχει κάποια βλάβη.

- Να βοηθήσει του τεχνικούς των οχημάτων να εντοπίσουν και να επισκευάσουν κυκλώματα που έχουν υποστεί βλάβη στο σύστημα ελέγχου εκπομπών.

Το διαγνωστικό σύστημα OBD Ι εφαρμόζεται στα συστήματα αυτά που έχουν αυξημένη πιθανότητα να προκαλέσουν σημαντική αύξηση στις εκπομπές των καυσαερίων τους αν συμβεί κάποια βλάβη. Συνοπτικά αυτό περιλαμβάνει:

- Όλους του βασικούς αισθητήρες του κινητήρα

- Το σύστημα μέτρησης καυσίμου

- Λειτουργία (Exhaust gas recirculation - EGR)

Παρακάτω φαίνονται σχηματικά τα βασικότερα χαρακτηριστικά του OBD Ι και στη συνέχεια αναλύεται καθένα από αυτά ξεχωριστά:

Ενδεικτική λυχνία βλαβών

Κωδικοί διάγνωσης βλαβών

Διαγνωστική απεικόνιση για:

Βασικούς αισθητήρες εισόδου

Μέτρηση καυσίμου

Λειτουργία EGR

Έλεγχος συστημάτων για ανοιχτοκυκλώματα και βραχυκυκλώματα

2.1.1	Ενδεικτική λυχνία βλαβών (MIL):

Όταν προκύψει μία βλάβη, η ενδεικτική λυχνία βλαβών παραμένει φωτισμένη για όσο διάστημα ανιχνεύεται η βλάβη και σβήνει όταν τα πράγματα ξαναγίνουν φυσιολογικά, αφήνοντας ένα κωδικό διάγνωσης βλάβης αποθηκευμένο στην ECM (Engine Control Module) μνήμη. Τα κυκλώματα ελέγχονται ως προς τη συνέχειά τους, για βραχυκυκλώματα και σε μερικές περιπτώσεις για το φυσιολογικό εύρος των παραμέτρων τους.

Η ενδεικτική λυχνία βλαβών είναι επίσης ένα όργανο οπτικής διερεύνησης για τα περισσότερα προγράμματα ελέγχου εκπομπών και συντήρησης, επιτρέποντας στον ελεγκτή των εκπομπών να αποφασίσει γρήγορα για το εάν το σύστημα ελέγχου εκπομπών λειτουργεί φυσιολογικά. Κατά τη διάρκεια αυτού του οπτικού ελέγχου ο ελεγκτής πρέπει να παρατηρήσει την ενδεικτική λυχνία καθώς ο κινητήρας είναι εκτός λειτουργίας, με ενεργοποιημένα τα ηλεκτρικά συστήματα του οχήματος – προκαταρκτικός έλεγχος (Key on bulb) και μετά καθώς ο κινητήρας είναι σε πλήρη λειτουργία. Η λυχνία θα πρέπει να είναι αναμμένη κατά τη διάρκεια του προκαταρκτικού ελέγχου και σβηστή καθώς ο κινητήρας είναι σε λειτουργία. Όταν ένα όχημα περάσει αυτό τον έλεγχο, είναι πολύ πιθανόν το σύστημα ελέγχου του κινητήρα να λειτουργεί φυσιολογικά.

2.1.2	Κωδικοί διάγνωσης βλαβών

Οι κωδικοί διάγνωσης βλάβης δημιουργούνται από το OBD Ι διαγνωστικό σύστημα και αποθηκεύονται στην ECM μνήμη υποδεικνύοντας το κύκλωμα στο οποίο ανιχνεύθηκε η βλάβη. Οι κωδικοί και οι πληροφορίες που εμπεριέχουν, παραμένουν αποθηκευμένοι στη μνήμη μεγάλης διάρκειας του ECM ανεξάρτητα από το αν ο κωδικός προέκυψε από μία συνεχή και σοβαρή βλάβη ή από κάποια λιγότερο σοβαρή. Σημειώνεται ότι τα προϊόντα της Toyota με σύστημα κωδικών βλάβης θα συνεχίσουν να αποθηκεύουν έναν τέτοιο κωδικό στη μνήμη μακράς διαρκείας του ΕCM μέχρι ο κωδικός να καθαριστεί αποσυνδέοντας την τροφοδοσία από το ECM BATT τερματικό.

2.1.3	Ακολουθίες σειριακών δεδομένων

Κάποιοι κατασκευαστές εισήγαγαν τη χρήση σειριακών δεδομένων προσβάσιμων με ειδικά εργαλεία σάρωσης, παρόλο που κάτι τέτοιο δεν απαιτείται από τους κανονισμούς του OBD Ι. Τα σειριακά δεδομένα είναι ηλεκτρονικές πληροφορίες σχετικές με αισθητήρες, actuators (έμβολα) και ECM στρατηγική διαχείρισης καυσίμου/ψεκασμού, οι οποίες προσπελαύνονται με τη βοήθεια ενός καλωδίου που έρχεται από το ECM. Ο όρος ‘σειριακά δεδομένα’ υποδηλώνει ότι οι πληροφορίες κωδικοποιούνται ψηφιακά και μεταφέρονται σε σειρές ‘λέξεων δεδομένων’. Οι λέξεις δεδομένων αποκωδικοποιούνται και απεικονίζονται με ένα εργαλείο σάρωσης.

Πχ. Η τυπική OBD Ι σειριακή ακολουθία δεδομένων της Toyota συνίσταται από 20 λέξεις δεδομένων συμπεριλαμβανομένων τιμών για αισθητήρες, καταστάσεις διακοπτών και άλλα δεδομένα λειτουργίας του κινητήρα.

Παρακάτω (Εικόνα 2.1) απεικονίζεται σχηματικά ο τρόπος διασύνδεσης του εργαλείου σάρωσης για την απόκτηση των δεδομένων από το όχημα:

Εικόνα 2.1

2.2	On Board Diagnostics – Generation 2 (OBD II)

Παρόλο που το OBD Ι παρέχει πολύτιμες πληροφορίες σχετικά με τον πλήθος των κρίσιμων εκπομπών που σχετίζονται με συστήματα και εξαρτήματα, υπάρχουν και πολλά άλλα σημαντικά στοιχεία που δεν είναι ενσωματωμένα στο OBD Ι λόγω κάποιων τεχνικών περιορισμών την εποχή που το σύστημα μπήκε στη φάση της παραγωγής (κατά τη χρονιά 1988). Από τότε η τεχνολογική εξέλιξη έχει να δείξει πολλά νέα τεχνικά επιτεύγματα. Για παράδειγμα, η τεχνολογία για την καταγραφή της αφλογιστίας του κινητήρα και της αποδοτικότητας του καταλύτη έχει αναπτυχθεί και εφαρμοστεί σε οχήματα που έχουν μπει στην παραγωγική διαδικασία.

Ως αποτέλεσμα αυτών των τεχνικών επιτευγμάτων, και επειδή τα Ι/Μ προγράμματα αποδείχθηκε ότι ήταν λιγότερο αποδοτικά από το επιθυμητό στην ανίχνευση κρίσιμων εκπομπών κατά τη διάρκεια της φυσιολογικής λειτουργίας στο δρόμο, ένα καλύτερο OBD σύστημα αναπτύχθηκε υπό την καθοδήγηση του CARB.

Το OBD II που ενσωματώνεται στα μοντέλα των ετών 1994 -1996, προσθέτει καταγραφή και απεικόνιση της αποδοτικότητας του καταλύτη, ανίχνευση της αφλογιστίας του κινητήρα, καταγραφή του συστήματος καθαρισμού του κανίστρου (canister purge system monitoring), (secondary air system monitoring) καταγραφή του δευτερεύοντος συστήματος αέρα καθώς επίσης και του ρυθμού ροής (flow rate) του EGR συστήματος. Επιπρόσθετα, μία ακολουθία σειριακών δεδομένων που αποτελείται από είκοσι βασικές παραμέτρους δεδομένων και διαγνωστικούς κωδικούς βλαβών είναι αναπόσπαστο κομμάτι του συγκεκριμένου διαγνωστικού συστήματος.

Επιπλέον της βασικής απαίτησης του OBD II για ακολουθία σειριακών δεδομένων, διάφορες αυτοκινητοβιομηχανίες, χρησιμοποίησαν μία εμπλουτισμένη ακολουθία δεδομένων που αποτελείται από επιπλέον λέξεις δεδομένων. Η πρόσβαση σε όλα τα δεδομένα του OBD II γίνεται συνδέοντας ένα γενικής χρήσης αντικείμενο σάρωσης στο Data Link Connector (DLC) που είναι τοποθετημένος εντός της καμπίνας του οχήματος και σε απόσταση όχι μεγαλύτερης του ενός μέτρου από τον οδηγό. Η ποιότητα των δεδομένων, το εργαλείο σάρωσης, ο τρόπος διεξαγωγής του διαγνωστικού ελέγχου, οι διαγνωστικοί κωδικοί βλαβών και οτιδήποτε σχετίζεται με την χρήση του συστήματος OBD II, ορίζεται από την Κοινότητα των Μηχανικών Μηχανοκίνητων Οχημάτων (Society of Automotive Engineers – SAE).

Ο στόχος του OBD II είναι να παρέχει στο όχημα ένα διαγνωστικό σύστημα επί του οχήματος που είναι ικανό να καταγράφει διαρκώς την αποδοτικότητα του συστήματος ελέγχου εκπομπών και να βελτιώνει την απόδοση της διάγνωσης και της επισκευής όταν το σύστημα αποτυγχάνει. Ουσιαστικά, ένας Ι/Μ σταθμός εκπομπών θα προγραμματίζεται σε κάθε όχημα που είναι εφοδιασμένο με OBD II.

2.2.1	Χαρακτηριστικά του OBD II

Οι ακόλουθες πληροφορίες θα βοηθήσουν τον αναγνώστη να εξοικειωθεί με τα σημαντικότερα χαρακτηριστικά του OBD II συστήματος.

2.2.1.1	Διάγνωση που αφορά τον αισθητήρα οξυγόνου

Το σύστημα παρακολουθεί τον (ή τους) αισθητήρα/ες οξυγόνου συμπεριλαμβανομένης της καταγραφής για μόλυνση-νόθευση και υποβάθμιση της ποιότητάς του καταγράφοντας τη switching συχνότητα και το lean-rich, rich-lean switch time.

2.2.1.2	Σύστημα καταγραφής καυσίμου

Τα περισσότερα συστήματα καυσίμου αλλάζουν συνεχώς τη βάση βαθμονόμησής τους για να αντισταθμίσουν τις μεταβολές στην ατμοσφαιρική πίεση, τη θερμοκρασία, τη σύνθεση του καυσίμου, τις ποικιλίες των εξαρτημάτων και άλλους παράγοντες. Η προσαρμοστική συμπεριφορά είναι φυσιολογική όσο παραμένει μέσα στα προδιαγεγραμμένα όρια του συστήματος.

Όταν συνθήκες, όπως για παράδειγμα, λανθασμένη πίεση καυσίμου, ασύμμετρη ροή αέρα ή άλλα μηχανικά προβλήματα, κάνουν το σύστημα καυσίμου να λειτουργήσει εκτός των προδιαγεγραμμένων τιμών, το OBD II σύστημα είναι σχεδιασμένο έτσι ώστε να ανιχνεύει αυτή τη μη φυσιολογική λειτουργία. Εάν αυτές οι συνθήκες λαμβάνουν χώρα για μεγαλύτερο χρονικό διάστημα από κάποιο προκαθορισμένο, ένα κωδικός ανίχνευσης βλάβης (DTC) θα αποθηκευτεί. Όταν αποθηκευτεί ο κωδικός, η ταχύτητα του κινητήρα και η καταστάσεις θέρμανσης και φορτίου αποθηκεύονται σε ένα ανακτήσιμο, στιγμιότυπο (freeze frame) σειριακών δεδομένων.

2.2.1.3 Καταγραφή αφλογιστίας

Χρησιμοποιώντας ένα υψίσυχνο σήμα θέσης για το στροφαλοφόρο άξονα, ο EMC μπορεί να καταγράψει με μεγάλη ακρίβεια τις διακυμάνσεις στην ταχύτητα του στροφαλοφόρου άξονα κατά τη διάρκεια κάθε κύκλου του κινητήρα. Όταν ένας κινητήρας πυροδοτεί σωστά όλους του κυλίνδρους, ο στροφαλοφόρος άξονας επιταχύνει δε κάθε κύκλο. Όταν συμβαίνει αφλογιστία, δηλαδή ο κινητήρας δεν πυροδοτεί κάποιον (ή όλους του κυλίνδρους), η αύξηση της ταχύτητας του στροφαλοφόρου άξονα επηρεάζεται από αυτόν (ή αυτούς) τον κύλινδρο.

Οι κινητήρες που ελέγχονται με το OBD II χρησιμοποιούν έναν αισθητήρα ο οποίος μετράει απευθείας την ταχύτητα και τη θέση του στροφαλοφόρου άξονα. Αυτές οι πληροφορίες επεξεργάζονται από την ECΜ για να αποφασιστεί εάν συμβαίνει αφλογιστία, σε ποιο κύλινδρο λαμβάνει χώρα και σε ποιο βαθμό.

Όταν ανιχνεύεται κάποια αφλογιστία σε οποιοδήποτε βαθμό, ένα κωδικός ανίχνευσης βλάβης αποθηκεύεται και το ίδιο γίνεται για την ταχύτητα του κινητήρα, το φορτίο και τη θερμοκρασία κατά τη στιγμή της αφλογιστίας. Επιπρόσθετα, ο χειριστής του οχήματος θα ειδοποιηθεί για την κατάσταση μέσω μίας λυχνίας που θα αναβοσβήνει όταν συμβαίνει σοβαρό περιστατικό αφλογιστίας.

Στο παρακάτω σχηματικό διάγραμμα (Εικόνα 2.2) φαίνεται η ταχύτητα του στροφαλοφόρου άξονα κατά την ανίχνευση της αφλογιστίας:

Εικόνα 2.2: Το ECM καταγράφει την ταχύτητα και τη θέση του στροφαλοφόρου άξονα διαμέσου των Ne και G εισόδων. Επειδή η ταχύτητα του στροφαλοφόρου άξονα φυσιολογικά αυξάνεται όταν συμβαίνει ανάφλεξη, το ECM μπορεί να καταγράψει αυτή την αλλαγή από το ένα γεγονός ανάφλεξης στο άλλο για να καταγράψει την παρουσία και το βαθμό της αφλογιστίας.

Όταν συμβαίνει μερική αφλογιστία, ο ρυθμός αύξησης της ταχύτητας του στροφαλοφόρου άξονα μειώνεται. Εάν συμβεί πλήρης αφλογιστία, δε θα υπάρξει καμία αύξηση.

2.2.1.4 Απεικόνιση του καταλύτη

Ένας υπο-αισθητήρας οξυγόνου (sub-oxygen) (2) που είναι τοποθετημένος στην κατεύθυνση του ρεύματος στην έξοδο του καταλυτικού μετατροπέα, καταγράφει τη switching συχνότητα η οποία συγκρίνεται με τη switching συχνότητα του βασικού αισθητήρα οξυγόνου (1) που είναι τοποθετημένος αντίθετα με το ρεύμα του καταλύτη. Η αποδοτικότητα της οξείδωσης του καταλύτη μπορεί να αποφασιστεί συγκρίνοντας τις switching συχνότητες αυτών των δύο αισθητήρων. Καθώς η απόδοση μετατροπής του καταλύτη μειώνεται, η switching συχνότητα του αισθητήρα (2) αυξάνεται πλησιάζοντας αυτή του αισθητήρα (1). Επιπλέον, ο αισθητήρας (2) μπορεί να χρησιμοποιηθεί πέρα από τη διαδικασία διάγνωσης και για τη διατήρηση του ιδανικού ελέγχου καυσίμου όταν ο καταλύτης αρχίζει να φθείρεται.

Στο παρακάτω σχηματικό διάγραμμα (Εικόνα 2.3) απεικονίζεται η καταγραφή της αποδοτικότητας του καταλύτη:

Εικόνα 2.3: Το ECM καταγράφει την αποδοτικότητα του καταλύτη συγκρίνοντας τη switching δραστηριότητα του βασικού αισθητήρα οξυγόνου με αυτή του υπο-αισθητήρα οξυγόνου. Όταν το switching του υπό-αισθητήρα οξυγόνου αρχίζει να πλησιάζει αυτό του βασικού αισθητήρα, υποδεικνύεται η φθορά του καταλύτη και αποθηκεύεται ένας κωδικός βλάβης.

2.2.1.5	ΕGR Σύστημα καταγραφής

Η εμπλουτισμένη καταγραφή των EGR χαρακτηριστικών ρυθμών ροής (flow rate) συμπεριλαμβάνει την ικανότητα εντοπισμού ρυθμών ροής (flow rate) που είναι πάνω ή κάτω από τον προκαθορισμένο ρυθμό παροχής για δοσμένες συνθήκες λειτουργίας του κινητήρα. Μία μέθοδος για να γίνει κάτι τέτοιο, είναι απλά να καταγράφεται η αλλαγή στη θερμοκρασία της εσωτερικής πλευράς του περάσματος του EGR. Μία άλλη μέθοδος είναι η μέτρηση της αναλογίας καυσίμου-αέρα στο σύστημα παροχής καυσίμου καθώς η EGR ροή αναστέλλεται στιγμιαία.

2.2.1.6	Εvaporative Purge System monitoring (Καταγραφή συστήματος καθαρισμού εξάτμισης)

Καταγράφοντας τον αισθητήρα οξυγόνου και το πλάτος του παλμού ψεκασμού καθώς το κάνιστρο καθαρίζεται, το ECM μπορεί να εντοπίσει τη μείωση του οξυγόνου στο περιεχόμενο της εξάτμισης και την αντίστοιχη μείωση του πλάτους του παλμού ψεκασμού για να αντισταθμιστεί αυτή η στιγμιαία κατάσταση. Με αυτόν τον τρόπο, το ECM μπορεί να ανιχνεύσει μία αποτυχία στο σύστημα ελέγχου καθαρισμού του κανίστρου και να αποθηκεύσει έναν κωδικό διάγνωσης σφάλματος ώστε να ειδοποιήσει τον χειριστή του οχήματος για τη δυσλειτουργία. Η συγκεκριμένη λειτουργία υπάρχει μόνο σε μοντέλα που έχουν ενσωματωμένο το ΟBD II μετά το 1995.

2.2.1.7	Δευτερεύον σύστημα καταγραφής αέρα (Secondary Air System Monitoring)

Χρησιμοποιώντας ένα ρεύμα αέρος αντίθετης φοράς, από αυτή του ρεύματος του αισθητήρα οξυγόνου, στιγμιαία, κατά τη διάρκεια ενός κλειστού κύκλου λειτουργίας, το ECM μπορεί να καταγράψει την απόκριση του αισθητήρα οξυγόνου και την αύξηση του πλάτους του παλμού ψεκασμού για να αποφασίσει εάν το δευτερεύον σύστημα αέρα λειτουργεί κανονικά.

2.2.1.8	Ενεργοποίηση Ενδεικτικής Λυχνίας Δυσλειτουργιών (MIL)

Όταν συμβαίνει μία δυσλειτουργία, η ενδεικτική λυχνία θα φωτιστεί και θα παραμείνει φωτισμένη εφόσον η βλάβη δεν επιδιορθώνεται. Η λυχνία θα παραμένει φωτισμένη και μετά από διαδοχικές επανεκκινήσεις ακόμα και αν η βλάβη έχει επιδιορθωθεί. Το OBD II μπορεί να ‘σβήσει’ τη λυχνία αν έπειτα από τρεις διαδοχικές επανεκκινήσεις του κινητήρα το πρόβλημα δεν επανεμφανιστεί.

Το OBD II θα σβήσει έναν κωδικό σφάλματος μόνο αν η συγκεκριμένη βλάβη στην οποία αναφέρεται δεν επανεμφανιστεί κατά τη διάρκεια σαράντα διαδοχικών επανεκκινήσεων του κινητήρα. Κάποιες αυτοκινητοβιομηχανίες έχουν επιλέξει να μη σβήνουν τους κωδικούς αλλά τοποθετούν κάποια σήμανση (σημαία) σε κάθε κωδικό που δεν επανεμφανίζεται σε σαράντα διαδοχικές επανεκκινήσεις.

Οι διαγνωστικοί κωδικοί σφαλμάτων μπορούν να διαγραφούν χρησιμοποιώντας ένα γενικό εργαλείο σάρωσης είτε αποσυνδέοντας την τροφοδοσία από το τερματικό ECM BATT.

2.2.1.9	Έλεγχος ετοιμότητας

Το διαγνωστικό σύστημα OBD II κάνει συνεχείς καταγραφές για αφλογιστία και βλάβες στο σύστημα τροφοδοσίας καυσίμου. Επίσης πραγματοποιεί έναν λειτουργικό έλεγχο στον καταλύτη, στο σύστημα EGR και στους αισθητήρες οξυγόνου μία φορά σε κάθε ‘κύκλο οδήγησης’. Συγκεκριμένες οδηγικές συνθήκες πρέπει να λαμβάνονται υπόψιν πριν επιβεβαιωθεί ότι αυτά τα συστήματα λειτουργούν σωστά. Για παράδειγμα, ο κινητήρας θα πρέπει να είναι σε θερμοκρασία κανονικής λειτουργίας, η γωνία στην οποία είναι πατημένο το γκάζι πρέπει να υπερβαίνει μία προκαθορισμένη τιμή, ο κινητήρας θα πρέπει να έχει ένα συγκεκριμένο φορτίο και ούτω καθεξής.

Όταν αυτές οι συνθήκες δεν πληρούνται το ECM δε θα ολοκληρώσει τον έλεγχο ετοιμότητας και δε θα μπορεί να απεικονίσει τα δεδομένα που θα προέκυπταν αν ο έλεγχος ολοκληρωνόταν. Υπό αυτές τις συνθήκες στο εργαλείο σάρωσης θα εμφανίσει μήνυμα που θα υποδηλώνει ότι ‘ο έλεγχος ετοιμότητας δεν ολοκληρώθηκε’, προειδοποιώντας το χειριστή ότι τα δεδομένα ελέγχου δεν είναι διαθέσιμα.

Ο έλεγχος ετοιμότητας είναι μία σημαία η οποία χρησιμοποιείται κατά τη διάρκεια Ι/Μ επιθεωρήσεων (ελέγχων) για να υποδηλώσει ότι το OBD σύστημα του οχήματος δεν μπορεί να παράξει τις πληροφορίες που χρειάζεται κατά τη διάρκεια του ελέγχου. Σε αυτή την περίπτωση το όχημα πρέπει να συνεχίσει να λειτουργεί μέχρι να ικανοποιούνται όλες οι συνθήκες που απαιτούνται για τον έλεγχο ετοιμότητας. Η συγκεκριμένη διαδικασία αναπαρίσταται στην παρακάτω εικόνα (Εικόνα 2.4) η οποία έχει ληφθεί από εργαλείο σάρωσης:

Εικόνα 2.4: Προειδοποίηση που εμφανίζεται κατά τη διεξαγωγή του ελέγχου ετοιμότητας: Καθώς εισάγονται οι CARB OBD II διαγνωστικές συναρτήσεις, η παραπάνω εικόνα θα εμφανιστεί εάν το ΕCM δεν έχει ολοκληρώσει όλους τους δυναμικούς του ελέγχους (και εκείνους που απαιτούν συνεχή καταγραφή και εκείνους που απαιτούν καταγραφή μίας φοράς σε κάθε ‘κύκλο ταξιδιού’).

2.2.1.10 Αποθηκευμένα δεδομένα του κινητήρα

Όταν ανιχνευθεί μία δυσλειτουργία, το σύστημα OBD II θα αποθηκεύσει όλα τα δεδομένα της χρονικής στιγμής στην οποία έγινε η καταγραφή του κωδικού βλάβης. Αυτό το πλαίσιο των αποθηκευμένων δεδομένων μπορεί να ανακτηθεί οποιαδήποτε στιγμή χρησιμοποιώντας το γενικό εργαλείο σάρωσης όπως απεικονίζεται και παρακάτω (Εικόνα 2.5):

Εικόνα 2.5: Όταν κάποιος κωδικός διάγνωσης βλάβης αποθηκεύεται στη μνήμη, οι συνθήκες λειτουργίας του κινητήρα κατά τη στιγμή της αποθήκευσης, αποθηκεύονται στο ECM ως ‘στιγμιότυπο δεδομένων’ (freeze frame data). H ανάκτηση και επισκόπηση αυτών των δεδομένων μπορεί να γίνει κατά τη διάρκεια της διάγνωσης.

2.2.1.11 Τυποποίηση των πληροφοριών επιδιόρθωσης και των κωδικών ανίχνευσης βλαβών

Σύμφωνα με τους κανονισμούς του OBD II οι πληροφορίες διάγνωσης και επισκευής που σχετίζονται με τις εκπομπές καυσαερίων θα είναι άμεσα διαθέσιμες από τον κατασκευαστή του οχήματος στο συνεργείο που θα πραγματοποιεί την επισκευή. Αυτές οι πληροφορίες περιλαμβάνουν διαδικασίες και διευκρινίσεις απαραίτητες για τη διάγνωση του συστήματος ελέγχου του κινητήρα. Παρόλο που πολλά διαγνωστικά στοιχεία μπορούν να είναι διαθέσιμα χρησιμοποιώντας ειδικό εξοπλισμό και διαδικασίες, ειδικές διαδικασίες επισκευής θα γραφούν χρησιμοποιώντας ένα ειδικό εργαλείο σάρωσης και άλλο συνηθισμένο εξοπλισμό ελέγχου όπως πολύμετρα και παλμογράφοι.

Σε μία προσπάθεια να απλοποιηθούν τα διαγνωστικά δεδομένα, το OBD II απαιτεί όλοι οι κατασκευαστές να τυποποιήσουν τους κωδικούς ανίχνευσης βλάβης σε όλα τα οχήματα που είναι εφοδιασμένα με το OBD II. Τελικά, όλες οι πληροφορίες επιδιόρθωσης που σχετίζονται με τις εκπομπές, θα αποκτήσουν τυποποιημένη μορφή και θα είναι διαθέσιμες με ηλεκτρονικά μέσα.

Παρακάτω φαίνονται σχηματικά τα σημαντικότερα χαρακτηριστικά του συστήματος OBD II:

Συνεχόμενη καταγραφή και λειτουργικοί έλεγχοι σε κάθε ταξίδι (once per trip)

Ενσωματωμένη διάγνωση που αφορά τον αισθητήρα οξυγόνου

Ενσωματωμένη διάγνωση που αφορά την αναλογία του καυσίμου

Ανίχνευση αφλογισίας του κινητήρα

Καταγραφή και απεικόνιση της αποδοτικότητας του καταλύτη

Καταγραφή και απεικόνιση της EGR ροής

Εvaporative Purge Flow Meter (Για μοντέλα μετά το ’95)

Δευτερεύουσα καταγραφή αέρα (Lexus)

Νέοι κανόνες που αφορούν το φωτισμό της ενδεικτικής λυχνίας

Τυποποίηση

Κωδικοί διάγνωσης σφαλμάτων

Ακολουθία σειριακών δεδομένων

Εργαλείο σάρωσης

2.3	Τροποποίηση του 1990 (περιορισμός καυσαερίων)

Στις 15 Νοεμβρίου του 1990, προστέθηκε η βελτίωση περιορισμού των καυσαερίων ωθώντας την Οργάνωση Προστασίας του Περιβάλλοντος να προωθήσει νέους κανονισμούς απαιτώντας από τους κατασκευαστές αυτοκινήτων να εγκαταστήσουν OBD συστήματα ικανά να:

- Εντοπίζουν δυσλειτουργίες βασικών εξαρτημάτων του σχετίζονται με τις εκπομπές οι οποίες είχαν ως αποτέλεσμα την αποτυχία του οχήματος να συμμορφωθεί με τα ομοσπονδιακά επιτρεπτά επίπεδα για τις εκπομπές.

- Ειδοποιούν το χειριστή του οχήματος για την ανάγκη συντήρησης ή επισκευής των εξαρτημάτων που σχετίζονται με τις εκπομπές.

- Αποθηκεύουν κωδικούς ανίχνευσης σφαλμάτων και παρέχουν πρόσβαση σε αυτές τις πληροφορίες από το όχημα.

Επιπλέον οι κατασκευαστές θα έχουν διαθέσιμες προς κάθε ενδιαφερόμενο όλες τις πληροφορίες που αφορούν τη συντήρηση και την επισκευή του συστήματος και των εξαρτημάτων εκπομπών.

Η υιοθέτηση αυτών των κανόνων προωθήθηκε από το γεγονός ότι το 1990, 96 αστικές περιοχές στις Ηνωμένες Πολιτείες παραβίαζαν τα διεθνή πρότυπα για την ποιότητα της ατμόσφαιρας αναφορικά με το όζον και άλλες 41 τα παραβίαζαν αναφορικά με το μονοξείδιο του άνθρακα.

Η σταδιακή διαδικασία εισαγωγής του προτύπου OBD II στην Καλιφόρνια και σε όλη την ομοσπονδία απεικονίζεται παρακάτω (Εικόνα 2.6):

Εικόνα 2.6

Ένα πρωτόκολλο OBD II το οποίο υιοθετήθηκε από πολλούς κατασκευαστές οχημάτων είναι και το J1850 PWM, όπως αυτό περιγράφεται στο SAE J1850 Class B. Το συγκεκριμένο πρωτόκολλο αναλύεται στο κεφάλαιο που ακολουθεί.

# ΤΟ ΠΡΩΤΟΚΟΛΛΟ

# SAE J1850 PWM

3.1	Γενικά

Το πρωτόκολλο J1850 PWM (Pulse Width Modulation – Διαμόρφωση Πλάτους Παλμού) αναπτύχθηκε με προσέγγιση ανοικτής αρχιτεκτονικής (open architecture). Ένα δίκτυο που αναπτύσσεται με την αρχιτεκτονική αυτή έχει τη δυνατότητα προσθήκης ή κατάργησης κόμβων με ελάχιστο αντίκτυπο στο υλικό και το λογισμικό τον υπολοίπων κόμβων. Προκειμένου να επιτευχθεί αυτή η προσέγγιση το δίκτυο χρησιμοποιεί Carrier Sense Multiple Access (CSMA) με non-destructive contention resolution. Επιπλέον το δίκτυο αποδίδει προτεραιότητα στα πλαίσια με τέτοιο τρόπο ώστε σε περίπτωση σύγκρουσης μεταξύ κάποιων πλαισίων, τα πλαίσια υψηλότερης προτεραιότητας να εκτελούνται πρώτα σε βάρος αυτών που έχουν μικρότερη προτεραιότητα.

Παρακάτω αναλύονται τα χαρακτηριστικά ενός δικτύου που υλοποιείται με το πρωτόκολλο αυτό.

3.2	Τοπολογία Δικτύου – Data Bus Topology

Περιλαμβάνει όλες τις φυσικές συνδέσεις μεταξύ των κόμβων και του δίαυλου, καθώς και όλους τους κόμβους και δίαυλους που αποτελούν το δίκτυο. Η απλούστερη τοπολογία δίαυλου δεδομένων είναι η τοπολογία μονού επιπέδου (simple- level bus topology), η οποία εφαρμόζεται ευρέως στην αυτοκινητοβιομηχανία. Σε μία τέτοια τοπολογία υπάρχει ένας μόνο δίαυλος και όλοι οι κόμβοι συνδέονται μέσω αυτού. Οι ειδικές ανάγκες κάποιας εφαρμογής μπορεί να απαιτούν μια τοπολογία μονού επιπέδου να υλοποιείται με πολλαπλά καλώδια διασύνδεσης τα οποία να λειτουργούν σε διαφορετικές καταστάσεις (ενεργητική – active ή παθητική – passive). Για το λόγο αυτό πρέπει να οριστούν κάποια κριτήρια ώστε μια τοπολογία να θεωρείται μονού επιπέδου τα οποία παρατίθενται ακολούθως:

α. Όλοι οι κόμβοι στέλνουν και λαμβάνουν δεδομένα μέσω του ιδίου δρόμου.

β. Όλοι οι κόμβοι λαμβάνουν όλα τα πλαίσια ταυτόχρονα.

γ. Η επικοινωνία σε κάθε δίαυλο δεδομένων είναι μοναδική.

3.3 Έλεγχος του δίαυλου

Παρά το γεγονός ότι μπορούν να χρησιμοποιηθούν διάφορες μέθοδοι ελέγχου του δίαυλου, το πρωτόκολλο J1850 έχει σχεδιαστεί να χρησιμοποιεί έλεγχο δίαυλου στον οποίο δεν υπάρχει κόμβος - αφέντης (masterless bus control). Το κυριότερο πλεονέκτημα αυτής της μορφής ελέγχου είναι ότι μπορεί να παρέχει τη βάση για την ανάπτυξη ενός συστήματος επικοινωνιών ανοικτής αρχιτεκτονικής.  Από τη στιγμή που δεν υπάρχει κόμβος – αφέντης κάθε κόμβος έχει ίσες ευκαιρίες να ξεκινήσει μετάδοση δεδομένων εφόσον ο δίαυλος είναι διαθέσιμος (ανενεργός).  Όμως επειδή όλοι οι κόμβοι δεν είναι εξίσου  σημαντικοί  γίνεται  διάκριση  προτεραιότητας  μεταξύ  των πλαισίων

και το πλαίσιο μέγιστης προτεραιότητας ολοκληρώνεται πάντα και διασφαλίζεται ότι δε θα χαθούν δεδομένα εξαιτίας του ανταγωνισμού που αναπτύσσεται κατά τη μετάδοση των πλαισίων.

3.4	Διαστρωμάτωση σε επίπεδα

Το πρωτόκολλο J1850 έχει μία δομή οργανωμένη σε επίπεδα για σαφή διαχωρισμό των λειτουργιών και για βέλτιστη λειτουργικότητα. Παρακάτω αναλύεται η οργάνωση των επιπέδων αυτών, καθώς και οι λειτουργίες τους.

3.4.1 	Επίπεδο Εφαρμογής – Application Layer

Αυτό το επίπεδο καθορίζει τις σχέσεις μεταξύ των διαφόρων συσκευών εισόδου και εξόδου, λαμβάνοντας υπόψιν και την ενδεχόμενη παρέμβαση από κάποιο χειριστή. Επίσης το επίπεδο αυτό περιγράφει σε γλώσσα υψηλού επιπέδου τις διάφορες λειτουργίες και τους αλγόριθμους ελέγχου, αν απαιτούνται τέτοιοι.  Ένα παράδειγμα της λειτουργικότητας του επιπέδου εφαρμογής θα μπορούσε να είναι το εξής: "Πιέζοντας το κουμπί των φώτων πρέπει να ανάψουν τα μικρά φώτα και τα πίσω φώτα”. Το επίπεδο εφαρμογής χρησιμοποιείται επίσης για την περιγραφή των θεσμοθετημένων διαγνωστικών λειτουργιών.

Αναλυτικότερα το επίπεδο αυτό υποστηρίζει:

- Μετάδοση πληροφοριών από έναν κόμβο του δικτύου σε άλλους. Η μετάδοση αυτή εξυπηρετεί τόσο λειτουργικούς όσο και διαγνωστικούς σκοπούς.

- Μηνύματα Κανονικής Λειτουργίας	: Πρόκειται για τα μηνύματα που αποστέλλονται κατά τη διάρκεια μη διαγνωστικών λειτουργιών και χρησιμοποιούνται για την επικοινωνία μεταξύ ενός εκπομπού και ενός η περισσότερων παραληπτών.

- Διαγνωστικά Μηνύματα	: Το δίκτυο εξυπηρετεί και διαγνωστικούς σκοπούς για τις διάφορες συσκευές που είναι συνδεδεμένες σε αυτό. Οι διαγνωστικές διεργασίες που υποστηρίζονται είναι οι θεσμοθετημένες διαδικασίες διάγνωσης, οι καθιερωμένες διαδικασίες διάγνωσης που χρησιμοποιούνται από τη βιομηχανία καθώς και διαδικασίες διάγνωσης που αναπτύσσονται από τους κατασκευαστές.

Οι θεσμοθετημένες διαγνωστικές λειτουργίες και κάποια επίπεδα διαγνωστικών λειτουργιών που αναπτύσσονται από τη βιομηχανία και τα οποία χρησιμοποιούν τη συνιστώμενη πρακτική (recommended practice) θα πρέπει να δημιουργούν διαδικασίες και πλαίσια σύμφωνα με τη συνιστώμενη πρακτική (recommended practice).  Στα πρωτόκολλα SAE J1979 και SAE J2190 ορίζονται οι αναγνωριζόμενες λειτουργίες ελέγχου που είναι διαθέσιμες και έχουν δεσμευτεί για διαγνωστικούς σκοπούς. Ειδικές διαδικασίες ελέγχου που αναπτύσσουν οι κατασκευαστές και που χρησιμοποιούν το δίκτυο μπορούν να καθορίσουν διαδικασίες οι οποίες δε συμμορφώνονται με τις απαιτήσεις της συνιστώμενης πρακτικής.

- Παραμετρικά Διαγνωστικά Δεδομένα: Τα πρωτόκολλα SAE J1979 και SAE J2190 ορίζουν διαδικασίες ελέγχου καθώς επίσης και τη μορφοποίηση του πλαισίου για χρήση από εξοπλισμό ελέγχου, ο οποίος βρίσκεται εκτός του οχήματος, και μέσω του οποίου μπορούν να ληφθούν διαγνωστικά δεδομένα από αυτό.

- Κωδικοί Διάγνωσης Βλαβών: Στο πρωτόκολλο SAE J2012 ορίζονται οι κωδικοί σφάλματος (DTC – Diagnostics Trouble Codes) οι οποίοι αντιστοιχούν στις διάφορες δυσλειτουργίες των συστημάτων του οχήματος και επίσης ορίζεται ένα σύνολο κωδικών που μπορούν να χρησιμοποιηθούν από τους κατασκευαστές για δικούς τους σκοπούς. Στο πρωτόκολλο SAE J1979 και SAE J2190 περιλαμβάνονται μηνύματα τα οποία μπορούν να χρησιμοποιηθούν για την ανάκτηση των κωδικών αυτών από τα συστήματα των οχημάτων. Τέλος, η αντιστοίχιση των κωδικών σφάλματος σε δυσλειτουργίες του οχήματος μπορεί να γίνει σύμφωνα με τη δομή που περιγράφεται στο πρωτόκολλο SAE J2012.

Οι κυριότεροι κωδικοί διάγνωσης βλαβών παρατίθενται στο Παράρτημα στο τέλος της αναφοράς. Παρακάτω παρατίθεται τυποποιημένη δομή αυτών των κωδικών:

Κάθε κωδικός αποτελείται από ένα γράμμα ακολουθούμενο από 5 αριθμητικά ψηφία.

1) Το γράμμα στην αρχή του κωδικού μπορεί να είναι P, B, C ή U καθένα από τα οποία αναφέρεται σε διαφορετικό σύστημα όπου ανιχνεύεται η βλάβη. Συγκεκριμένα η αντιστοιχία έχει ως εξής:

- P = Powertrain (μηχανισμός μετάδοσης ισχύος)

- B = Body (αμάξωμα)

- C = Chassis (πλαίσιο)

- U = Undefined (απροσδιόριστο σύστημα)

2) Ο δεύτερος χαρακτήρας είναι ένα ψηφίο το οποίο υποδηλώνει αν ο κωδικός είναι ενσωματωμένος κωδικός του OBD II είτε εάν είναι συγκεκριμένος κωδικός κάποιου κατασκευαστή. Συγκεκριμένα αυτό το ψηφίο μπορεί να πάρει τις τιμές 0 και ένα ως εξής:

- 0: Το ‘0’ υποδηλώνει ότι ο κωδικός είναι ενσωματωμένος κωδικός του OBD II και κοινός για όλα τα οχήματα που είναι εφοδιασμένα με το συγκεκριμένο σύστημα.

- 1: Το ‘1’ υποδηλώνει ότι ο κωδικός είναι κάποιου συγκεκριμένου κατασκευαστή.

3) Ο τρίτος χαρακτήρας υποδηλώνει τον τύπο του υποσυστήματος στο οποίο αναφέρεται ο κωδικός. Συγκεκριμένα ισχύει η ακόλουθη αντιστοιχία:

- 1: Το ‘1’ αναφέρεται στο σύστημα εκπομπών.

- 2: Το ‘2’ αναφέρεται στο σύστημα ψεκασμού.

- 3: Το ‘3’ αναφέρεται στην ανάφλεξη ή την αφλογιστία.

- 4: Το ‘4’ αναφέρεται στον έλεγχο των εκπομπών.

- 5: Το ‘5’ αναφέρεται στην ταχύτητα του οχήματος

- 6: Το ‘6’ αναφέρεται στο κύκλωμα του κεντρικού επεξεργαστή

- 7:Το ‘7’ αναφέρεται στο σύστημα μετάδοσης κίνησης (transmission)

- 8: To ‘8’ αναφέρεται όπως και το ‘7’ στο σύστημα μετάδοσης κίνησης.

- 9: Δεσμευμένο ψηφίο για τον κατασκευαστή του οχήματος

- 0: Δεσμευμένο ψηφίο για τον κατασκευαστή του οχήματος

4) O τέταρτος και ο πέμπτος χαρακτήρας είναι μεταβλητοί και σχετίζονται με ένα συγκεκριμένο πρόβλημα.

Αναλυτικά οι βασικότεροι κωδικοί διάγνωσης βλαβών φαίνονται στο Παράρτημα. Να σημειώσουμε ότι αναφέρονται μόνο οι Ρ – κωδικοί καθώς η πλακέτα που σχεδιάστηκε και υλοποιήθηκε δεν υποστηρίζει εύρεση B, C και U κωδικών.

- Φιλτράρισμα Πλαισίων: Η συσκευή Διεπαφής Δικτύου (Network Interface Device) έχει τη δυνατότητα να φιλτράρει τα πλαίσια που κινούνται στο δίκτυο για να επιλέξει αυτά που αφορούν έναν συγκεκριμένο κόμβο. Επειδή στο πρωτόκολλο αυτό χρησιμοποιούνται διαφορετικές τεχνικές διευθυνσιοδότησης, τα κριτήρια φιλτραρίσματος μπορεί να περιλαμβάνουν τη σύγκριση των αρχικών bytes των διαφόρων πλαισίων. Άσχετα με το ποια μέθοδος χρησιμοποιείται, το φιλτράρισμα αποσκοπεί στην ελαχιστοποίηση του φόρτου του λογισμικού περιορίζοντας των αριθμό των λαμβανόμενων πλαισίων στον απολύτως απαραίτητο για κάθε κόμβο.

3.4.2 	Ενδιάμεσο Επίπεδο Μετατροπής – Data Link Layer

Η κυριότερη λειτουργία του Ενδιάμεσου Επιπέδου Μετατροπής είναι η μετατροπή των bits και των συμβόλων σε σωστά και έγκυρα πλαίσια. Άλλες λειτουργίες είναι η μετατροπή παράλληλων δεδομένων σε σειριακά, ο συγχρονισμός των bits και ο έλεγχος των σφαλμάτων. Αν κατά τον έλεγχο προκύψουν σφάλματα, το επίπεδο αυτό επιδιώκει να τα διορθώσει ή/και να ενημερώσει τα υψηλότερα επίπεδα.

Αναλυτικότερα το επίπεδο αυτό υποστηρίζει:

- Στρατηγική Διευθυνσιοδότησης: Δύο τύποι στρατηγικών διευθυνσιοδότησης ορίζονται και μπορούν να συνυπάρξουν σε αυτό το δίκτυο.  Οι δύο τύποι εξυπηρετούν διαφορετικά είδη εργασιών και αποτελεί μεγάλο πλεονέκτημα η δυνατότητα να χρησιμοποιούνται ταυτόχρονα οι δύο τύποι στο ίδιο δίκτυο.

- Φυσική Διευθυνσιοδότηση: Πλαίσια μπορούν να ανταλλαχθούν μεταξύ δύο συσκευών ανάλογα με τη φυσική τους διεύθυνση μέσα στο δίκτυο. Σε κάθε κόμβο αποδίδεται μία μοναδική φυσική διεύθυνση. Αυτός ο τύπος διευθυνσιοδότησης χρησιμοποιείται όταν η επικοινωνία πρέπει να γίνει μεταξύ συγκεκριμένων κόμβων, αποκλείοντας τους υπόλοιπους κόμβους που είναι συνδεδεμένοι στο δίκτυο. Ιδιαίτερα σε διαγνωστική λειτουργία είναι σημαντική η μοναδική διευθυνσιοδότηση και επομένως η ταυτοποίηση ενός συγκεκριμένου κόμβου.

- Λειτουργική Διευθυνσιοδότηση: Πλαίσια  μπορούν να μεταδοθούν μεταξύ πολλών συσκευών ανάλογα με τη λειτουργία του κάθε πλαισίου. Σε κάθε κόμβο (ο οποίος μπορεί να βρίσκεται οπουδήποτε στο δίκτυο) αποδίδεται ένα σύνολο λειτουργιών που το αφορούν, είτε ως αποστολέα είτε ως παραλήπτη. Αυτό το είδος διευθυνσιοδότησης χρησιμοποιείται όταν δεν είναι σημαντική η φυσική θέση του κόμβου αλλά η λειτουργία του μηνύματος που αποστέλλεται.

  - Πρόσβαση Δικτύου και Συγχρονισμός Δεδομένων: Η Διεπαφή του δικτύου μπορεί να υλοποιήσει ένα πρωτόκολλο πολλαπλής πρόσβασης με μηχανισμό διαιτησίας, χρησιμοποιώντας διαιτησία χωρίς απώλειες (nondestructive bit-by-bit arbitration) για να αποδίδει πρόσβαση στο δίαυλο. Εφόσον δεν υφίσταται σήμα χρονισμού στο δίκτυο, ο συγχρονισμός των κόμβων γίνεται βάσει των μεταβάσεων bits ή συμβόλων στο δίαυλο.

- Αποθήκευση Πλήρους Μηνύματος: Ένα ή περισσότερα μηνύματα μπορούν να αποθηκευθούν ολόκληρα στη συσκευή διεπαφής. Αυτή η προσέγγιση μειώνει το φορτίο του λογισμικού, αυξάνοντας παράλληλα το κόστος του υλικού. Η χρήση Φιλτραρίσματος Μηνυμάτων είναι επίσης δυνατή σε μία τέτοια συσκευή μειώνοντας το φορτίο του λογισμικού ακόμη περισσότερο.

- Αποθήκευση Byte (Byte Buffering)	: Κάθε byte ενός ληφθέντος μηνύματος αποθηκεύεται μόνο του στη συσκευή διεπαφής.  Η συσκευή ελέγχου είναι υπεύθυνη για το σωστό χρονισμό της συσκευής διεπαφής ώστε να ανταποκριθεί στην κίνηση των πλαισίων.

- Στοιχεία Δικτύου και Δομή

Η γενική μορφή έχει ως εξής:

idle, SOF, DATA_0, ..., DATA_N, CRC, EOD, NB, IFR_1, ..., IFR_N, EOF, IFS, idle

Τα παραπάνω ακρωνύμια ορίζονται στον Πίνακα 3.1:

| Ακρωνύμιο | Επεξήγηση – Λειτουργία |
| --- | --- |
| idle | Δίαυλος ελεύθερος (αδρανής), συμβαίνει πριν το SOF και μετά το IFS. Ορίζεται ως οποιαδήποτε περίοδος που ο δίαυλος βρίσκεται σε παθητική κατάσταση μετά από ένα IFS. Κατά τη διάρκεια του idle bus, οποιοσδήποτε κόμβος μπορεί να ξεκινήσει άμεσα μετάδοση. Παρόλα αυτά όταν δύο ή περισσότεροι κόμβοι ξεκινούν μετάδοση σχεδόν ταυτόχρονα, μπορεί να προκύψει διένεξη. Για το λόγο αυτό γίνεται επανασυγχρονισμός με την θετική ακμή του ρολογιού. |
| SOF | Έναρξη του Πλαισίου – Start Of Frame Χρησιμοποιείται για να ορίσει μονοσήμαντα την έναρξη του πλαισίου και δε συμμετέχει στον υπολογισμό του κώδικα ανίχνευσης σφαλμάτων CRC. |
| DATA | Bytes Δεδομένων (μήκους 8 bits το καθένα). Ο συγκεκριμένος αριθμός bytes δεδομένων, το καθένα μήκους 8 bits, που μπορεί να μεταδοθεί εναπόκειται στη βούληση του σχεδιαστή του συστήματος. Στην υλοποίηση Pulse Width Modulation (PWM) στα 41.6 Kbps το μέγιστο μήκος πλαισίου από το SOF ως το EOF (συμπεριλαμβανομένων των SOF και EOF) είναι 101 χρόνοι bit και ο μέγιστος αριθμός των bytes του μηνύματος (μη συμπεριλαμβάνοντας τους χαρακτήρες αρχής ή τέλους του πλαισίου SOF, EOD, EOF και IFS) είναι 12 bytes. |
| EOD | Τέλος Δεδομένων – End Of Data (μόνο όταν χρησιμο- ποιείται IFR) Χρησιμοποιείται για να σημάνει το τέλος της μετάδοσης από τον δημιουργό του πλαισίου. Αν υπάρχει απόκριση εντός πλαισίου (IFR) τότε αυτή ξεκινάει να μεταδίδεται μετά το EOD αλλά πριν το EOF. Αν δε χρησιμοποιείται IFR ο δίαυλος παραμένει σε παθητική κατάσταση έχοντας ως αποτέλεσμα τη δημιουργία End of Frame (EOF). Αν το πλαίσιο περιέχει ένα IFR, ο δημιουργός του πλαισίου περιμένει οι παραλήπτες να οδηγήσουν το δίκτυο με ένα ή περισσότερα IFR bytes αμέσως μετά το EOD. |
| CRC | CRC Byte (μπορεί να συμβεί και στο IFR) Για τη λειτουργία του CRC βλ. παράγραφο 3.4.2.1.1 |
| NB | Bit Κανονικοποίησης – Normalization Bit (μόνο στο VPW) Υπάρχει μόνο στο J1850 VPW και αναφέρεται εδώ για λόγους συμβατότητας. |
| IFR | Byte(s) Απόκρισης Εντός Πλαισίου – In-Frame Response Byte(s) Η λειτουργία του IFR παρατίθεται μετά το τέλος του Πίνακα (*). |
| EOF | Τέλος Πλαισίου – End of Frame Με την ολοκλήρωση του EOF ορίζεται το τέλος του πλαισίου. Μετά το τελευταίο προς μετάδοση byte (περι-λαμβανομένων και των IFR bytes, όπου χρησιμοποι-ούνται) ο δίαυλος τίθεται σε παθητική κατάσταση. Επομένως, όταν ολοκληρωθεί το EOF όλοι οι παραλήπτες θεωρούν ότι η μετάδοση έχει ολοκληρωθεί. |
| IFS | Διαχωριστικό Διάστημα μεταξύ των πλαισίων – Inter-Frame-Separation Διαχωριστικό Διάστημα μεταξύ των πλαισίων – Inter-Frame Separation (IFS) - Χρησιμοποιείται για να επιτρέπει το σωστό χρονισμό μεταξύ των κόμβων κατά τη μετάδοση διαδοχικών πλαισίων. Ένας αποστολέας δεν επιτρέπεται να ξεκινήσει τη μετάδοση στο δίαυλο πριν ολοκληρωθεί η ελάχιστη περίοδος του IFS. Αντίθετα οι παραλήπτες πρέπει να συγχρονίζονται με οποιοδήποτε SOF συμβεί πριν το τέλος του ελάχιστου χρόνου EOF, ώστε να υπάρχει ανοχή στις διακυμάνσεις του ρολογιού. Ένας αποστολέας που διεκδικεί την πρόσβαση στο δίαυλο πρέπει να περιμένει να εκπληρωθεί μία εκ των δύο ακόλουθων συνθηκών πριν ξεκινήσει τη μετάδοση του SOF: a. Να έχει περάσει το ελάχιστο διάστημα IFS. b. Να έχει περάσει το ελάχιστο διάστημα EOF και να έχει ανιχνευθεί μία νέα ακμή ρολογιού. |
| BRK | Break Mπορεί να συμβεί στο δίκτυο οποιαδήποτε στιγμή και χρησιμοποιείται για να διαχειριστεί τις καταστάσεις στις οποίες η επικοινωνία στο δίαυλο πρόκειται να τερματιστεί και όλοι οι κόμβοι τίθενται σε κατάσταση “έτοιμος για λήψη” (“ready to receive”) |

Πίνακας 3.1

*Bytes Απόκρισης Εντός Πλαισίου – In-Frame Response (IFR):	 Κατά την In-Frame Response, η μετάδοση των bytes απόκρισης ξεκινάει μετά το EOD. Αν το πρώτο bit του IFR-byte δε μεταδοθεί σε αυτό το σημείο και ο δίαυλος παραμείνει σε παθητική κατάσταση για όσο χρόνο έχει οριστεί το EOF, τότε ο δημιουργός του μηνύματος και όλοι οι παραλήπτες θεωρούν ότι το πλαίσιο έχει ολοκληρωθεί. Η μορφή των IFR-bytes μπορεί να είναι μία από τις ακόλουθες:

a.	Κανένα Byte Απόκρισης

b.	Ένα μόνο byte μεταδίδεται από έναν παραλήπτη (συνήθως πρόκειται για μοναδικό παραλήπτη (ID) ή για τη φυσική διεύθυνση του παραλήπτη).

c.	Πολλαπλά bytes, που το καθένα έχει αποσταλεί από έναν παραλήπτη. Τα bytes αυτά συνδέονται για να αποτελέσουν μια ακολουθία απόκρισης. Το byte απόκρισης κάθε παραλήπτη πρέπει να είναι μοναδικό και για το λόγο αυτό συνήθως αποτελείται από τη φυσική διεύθυνση. Αν κατά τη διαδικασία απόκρισης κάποιος κόμβος χάσει την πρόσβαση στο δίαυλο, τότε αναμεταδίδει το byte απόκρισης μέχρι ο παραλήπτης να εντοπίσει στην ακολουθία απόκρισης το byte απόκρισης του συγκεκριμένου κόμβου.  Αντίστοιχα, αν ένας παραλήπτης εντοπίσει το δικό του byte απόκρισης (δηλ. τη δική του φυσική διεύθυνση) μέσα στην ακολουθία απόκρισης τότε σταματάει τη μετάδοσή του, επιτρέποντας στους εναπομείναντες κόμβους να μεταδώσουν το δικό τους byte.

d.	Ένα ή περισσότερα bytes δεδομένων, όλα προερχόμενα από τον ίδιο κόμβο. Στα bytes αυτά μπορεί να προστεθεί και ένα CRC byte το οποίο υπολογίζεται μόνο για τα bytes απόκρισης.

Το όριο μήκους του πλαισίου εξακολουθεί να ισχύει όταν χρησιμοποιούνται IFR-bytes. Το άθροισμα των data bytes, CRC bytes, και IFR-bytes δεν πρέπει να υπερβαίνει το μήκος του πλαισίου.

Στην εικόνα 3.1 φαίνονται οι διαφορετικές μορφές των IFR Bytes.

1

Εικόνα 3.1: Είδη Απόκρισης Εντός Πλαισίου

- Στοιχεία Πλαισίου: Τα στοιχεία του πλαισίου πέρα από τα SOF, EOD, NB, EOF, IFS, και BRK είναι byte oriented και πρέπει να τελειώνουν με byte boundaries.

- Σειρά των bits: Το πρώτο bit που μεταδίδεται στο δίκτυο είναι το MSB (Σημαντικότερο bit – Most Significant bit).

- Μέγιστο Μήκος Πλαισίου: Ο μέγιστος αριθμός συνεχόμενων bits που μπορούν να αποσταλούν, δηλαδή ο μέγιστος χρόνος σε bit times που ένας κόμβος μπορεί να έχει πρόσβαση στο δίαυλο δεν μπορεί να υπερβαίνει ένα άνω όριο. Για τα συστήματα που χρησιμοποιούν Pulse Width Modulation (PWM) στα 41.6 Kbps	 το μέγιστο μήκος πλαισίου από το SOF ως το EOF (συμπεριλαμβανομένου του EOF) είναι 101 bits. Ο μέγιστος αριθμός bytes ενός μηνύματος εξαιρουμένων των SOF, EOD, EOF, και IFS) είναι 12 bytes.

3.4.2.1 	Ανίχνευση Σφάλματος

Παρακάτω περιγράφονται και κατηγοριοποιούνται οι καταστάσεις σφάλματος καθώς και οι μηχανισμοί ανίχνευσής τους. Η αντιμετώπιση του σφάλματος εναπόκειται στην κρίση του κατασκευαστή εκτός κι αν αναφέρεται διαφορετικά παρακάτω.

3.4.2.1.1	Cyclic Redundancy Check (CRC)

Χρησιμοποιείται τόσο στα συστήματα με κεφαλίδα ενός byte όσο και σε αυτά με κεφαλίδα 3 bytes. Ο μηχανισμός  του CRC αναλύεται ως εξής:

a.	Οι καταχωρητές (ή οι θέσεις μνήμης) όπου γίνεται ο υπολογισμός και ο έλεγχος του CRC βρίσκονται αντίστοιχα στον κόμβο του αποστολέα και του παραλήπτη και αρχικοποιούνται με λογικό 1 κατά τη διάρκεια του SOF.

b.	Όλα τα bits του πλαισίου που βρίσκονται μεταξύ του SOF και του πεδίου CRC χρησιμοποιούνται για την παραγωγή του Data Segment Polynomial που συμβολίζεται με D(X). Για κάθε δοσμένο frame, ο αριθμός αυτός μπορεί να μεταφραστεί ως μία δυαδική σταθερά μήκους n-bit όπου n το μήκος του frame σε bits.

c.	To CRC πολυώνυμο διαίρεσης είναι το X8 + X4 +  X3 + X2 + 1 και

συμβολίζεται με P(X).

d.	Από την ακόλουθη διαίρεση Modulo2 προκύπτει το Υπόλοιπο

Πολυώνυμο R(X):

1

Σημείωση: Q(X) είναι το πηλίκο της διαίρεσης.

e.	Το CRC byte τίθεται ίσο με το  ____R(X), όπου  ____R(X)  είναι το συμπλήρωμα

ως προς ένα του R(X).

f.	Το πολυώνυμο M(X)που μεταδίδεται με το πλαίσιο είναι το εξής:

2

g.	Στον παραλήπτη ολόκληρο το πλαίσιο που έχει ληφθεί, συμπεριλαμβανομένου του CRC byte που εστάλη περνάει μέσα από ένα κύκλωμα ελέγχου CRC. Ένα πλαίσιο χωρίς σφάλματα όταν περνάει από το κύκλωμα ελέγχου CRC θα έχει πάντα ως αποτέλεσμα το πολυώνυμο X7 + X6 + X2 (C4 hex) άσχετα με το περιεχόμενο του πλαισίου.

h.	 Παραδείγματα πλαισίων με τα αντίστοιχα CRC bytes υπάρχουν στον

Πίνακα 3.2.

| Bytes Δεδομένων (hex) | CRC (hex) |
| --- | --- |
| 00 00 00 00 | 59 |
| F2 01 83 | 37 |
| 0F AA 00 55 | 79 |
| 00 FF 55 11 | B8 |
| 33 22 55 AA BB CC DD EE FF | CB |
| 92 6B 55 | 8C |
| FF FF FF FF | 74 |

Πίνακας 3.2: Παραδείγματα Frames & Αντίστοιχων CRC Bytes

- Για να υποδηλωθεί ένα CRC σφάλμα χρησιμοποιείται μία σημαία

κατάστασης.

j.	Όταν χρησιμοποιείται CRC για την προστασία ενός IFR οι παραπάνω κανόνες εφαρμόζονται για τον καθορισμό του CRC, όμως οι κόμβοι αποστολέα και παραλήπτη λειτουργούν αντίστροφα. Ο υπολογισμός του CRC περιλαμβάνει μόνο τα IFR-bytes, ενώ τα SOF, EOD, EOF, και NB δε χρησιμοποιούνται στον υπολογισμό του CRC αλλά χρησιμεύουν ως χαρακτήρες αρχής ή τέλους δεδομένων (data delimiters).

Στην Εικόνα 3.2 απεικονίζεται μία τυπική γεννήτρια CRC, ενώ στην Εικόνα 3.3 απεικονίζεται ένα τυπικό κύκλωμα ελέγχου CRC.  Με κατάλληλη σύνδεση τα δύο κυκλώματα μπορούν να χρησιμοποιούν μόνο έναν καταχωρητή ολίσθησης και για τις δύο λειτουργίες τους.

2

Εικόνα 3.2: Γεννήτρια CRC

3

Εικόνα 3.3: Ελεγκτής CRC

3.4.2.1.2	Μήκος Πλαισίου/Μηνύματος

Ορίζεται ένα μέγιστο μήκος πλαισίου/μηνύματος που υποστηρίζεται από το πρωτόκολλο.  Ένα πλαίσιο που υπερβαίνει το καθορισμένο αυτό μήκος υποδηλώνει ενδεχόμενο σφάλμα.

3.4.2.1.3	Δεδομένα εκτός Ορίων – Out-of-Range	 – Data

Πρόκειται για δεδομένα που βρίσκονται εκτός των ορίων λειτουργίας του παραλήπτη και οφείλονται σε παρεμβολές στο δίκτυο επικοινωνιών του οχήματος. Η κατάσταση αυτή, στην οποία ο παραλήπτης δεν μπορεί να αποκωδικοποιήσει σωστά τα δεδομένα, μπορεί να εντοπιστεί από μία κατάλληλη συσκευή της οποίας η λειτουργία έγκειται στα παρακάτω:

- Ανάκτηση δεδομένων κρατώντας την έξοδο του παραλήπτη στην κατάσταση που βρισκόταν πριν την κατάσταση σφάλματος, για όσο διαρκούν οι παρεμβολές.

- Αν η διάρκεια των παρεμβολών είναι τέτοια ώστε να καταστραφεί κάποιο σύμβολο που βρίσκεται στο δίαυλο η ανάκτηση δεδομένων μπορεί να αποτύχει. Στην περίπτωση αυτή προκύπτει σφάλμα.

3.4.2.1.4	Ανίχνευση Μη Έγκυρων Bits

Σε κάποιες περιπτώσεις η ακεραιότητα των δεδομένων μπορεί να αυξηθεί αν ανιχνευθούν οι καταστάσεις στην ακολουθία δεδομένων όπου τα ληφθέντα bits δεν ικανοποιούν τις προδιαγραφές ούτε για λογικό 1 ούτε για λογικό 0.

3.4.2.1.5	Ανίχνευση Μη Έγκυρης Δομής Πλαισίου

Άσχετα με την κωδικοποίηση των δεδομένων η ακεραιότητά τους αυξάνεται αν ανιχνευθεί ένα EOD ή EOF σε κάποιο σημείο της ακολουθίας δεδομένων, που όμως δε συνάδει με το τέλος κάποιου byte, ή αν το μήκος του πλαισίου ξεπερνάει το αντίστοιχο όριο.

3.4.2.2 	Αντιμετώπιση Σφαλμάτων

- Μετάδοσης: 	Όταν ο δημιουργός ενός πλαισίου ανακαλύπτει μία κατάσταση σφάλματος στο δίκτυο πρέπει να σταματά τη μετάδοση του μηνύματος πριν την αρχή του επόμενου bit. Μπορεί να επανεκκινήσει τη μετάδοση του πλαισίου μετά από τον καθορισμένο χρόνο IFS ή αφού ανιχνεύσει μία ακμή μετά από κάποιο EOF.

- Λήψης:	 Αν ένα πλαίσιο που λαμβάνεται περιέχει κάποιο σφάλμα το πλαίσιο αυτό πρέπει να αγνοηθεί. Αν χρησιμοποιείται IFR ο παραλήπτης δεν πρέπει να αποκριθεί σε ένα πλαίσιο που περιέχει σφάλμα. Αυτή η έλλειψη απόκρισης λειτουργεί ως ένδειξη ανίχνευσης σφάλματος από τον παραλήπτη.

3.4.3 	Φυσικό Επίπεδο – Physical Layer

Το φυσικό επίπεδο και η καλωδίωσή του αποτελούν το μονοπάτι μέσω του οποίου γίνεται η ανταλλαγή δεδομένων μεταξύ των διαφόρων επιπέδων του Ενδιάμεσου Επιπέδου Μετατροπής. Τυπικά στοιχεία του Φυσικού Επιπέδου είναι το επίπεδο της τάσης και του ρεύματος, η σύνθετη αντίσταση του μέσου καθώς και ο ορισμός και χρονισμός του bit και των συμβόλων.

Στον τομέα αυτό ορίζονται οι βασικές παράμετροι που αναφέρονται στο υλικό, το φορτίο κάθε μονάδας, το μέγιστο επιτρεπόμενο μέγεθος δικτύου, τα ηλεκτρικά χαρακτηριστικά του δικτύου και των κόμβων και ορίζονται τα σφάλματα του φυσικού επιπέδου.

3.4.3.1	Υλικό Φυσικού Επιπέδου

Κάθε κόμβος του δικτύου πρέπει να έχει κατάλληλη τροφοδοσία και επίπεδο γης. Το δίκτυο μπορεί να υλοποιηθεί είτε με μονό είτε με διπλό καλώδιο. Στη δεύτερη περίπτωση μπορεί να χρησιμοποιηθούν είτε δύο καλώδια σε σταθερή απόσταση μεταξύ τους, είτε ένα τυλιγμένο ζεύγος καλωδίων.

3.4.3.2	Προδιαγραφές Μονάδας Φορτίου

Το ηλεκτρικό φορτίο που φέρει κάθε συσκευή που συνδέεται στο δίκτυο μετριέται σε μονάδες φορτίου (unit loads). Μία μονάδα φορτίου είναι μία ονομαστική τιμή, τέτοια ώστε αν όλοι οι κόμβοι αντιστοιχούν σε μία μονάδα φορτίου τότε μπορεί να συνδεθεί στο δίκτυο ο μέγιστος επιτρεπόμενος αριθμός κόμβων. Δεν υπάρχει περιορισμός για το μέγιστο φορτίο που μπορεί να επιφέρει κάθε κόμβος στο δίκτυο, δεν επιτρέπεται όμως το συνολικό φορτίο από όλους τους κόμβους να υπερβαίνει το όριο του εκάστοτε συστήματος.

3.4.3.3	Μέγιστος Αριθμός Κόμβων

Ο μέγιστος αριθμός κόμβων που μπορούν να συνδεθούν σε ένα σύστημα καθορίζεται θεωρώντας ότι καθένας επιφέρει φορτίο ίσο με μία μονάδα φορτίου.

3.4.3.4	Μέγιστο Μήκος Δικτύου

Η μέγιστη απόσταση μεταξύ δύο οποιονδήποτε κόμβων δεν πρέπει να υπερβαίνει μια δοσμένη απόσταση.

Οι γενικές απαιτήσεις του δικτύου συνοψίζονται στον Πίνακα 3.3.

| Περιγραφή Παραμέτρου | Τιμή Παραμέτρου |
| --- | --- |
| Μήκος Δικτύου Εντός Οχήματος – On-Vehicle Network Length | 35 μέτρα |
| Μήκος Δικτύου Εκτός Οχήματος – Off-Vehicle Network Length | 5 μέτρα |
| Συνολικό Μήκος Δικτύου – Total Vehicle Network Length | 40 μέτρα |
| Μέγιστος αριθμός μονάδων φορτίου - Maximum number of standard unit loads (συμπεριλαμβάνοντας τον εξοπλισμό εκτός οχήματος) | 32 κόμβοι |
| Αντίσταση φορτίου εκτός οχήματος – Off-vehicle load resistance | 10.6 ΚΩ min. |
| Χωρητικότητα Εκτός Οχήματος – Off-vehicle capacitance | 500 pF max. |
| Πίνακας 3.3: Γενικές Παράμετροι Δικτύου |  |

3.4.3.5	Bit Δεδομένων

Ο δίαυλος δεδομένων μπορεί να βρίσκεται σε μία εκ των δύο καταστάσεων, ενεργητική ή παθητική.  Η μετάβαση από παθητική σε ενεργητική γίνεται κατά την ακμή ανόδου του ρολογιού, ενώ η μετάβαση από ενεργητική σε παθητική γίνεται κατά την ακμή καθόδου.

Το πρωτόκολλο J1850 περιλαμβάνει δύο διαφορετικές μεθόδους κωδικοποίησης bit, τη Διαμόρφωση Πλάτους Παλμού (PWM) και τη Διαμόρφωση Μεταβλητού Πλάτους Παλμού – Variable Pulse Width (VPW). Στην παρούσα υλοποίηση χρησιμοποιήθηκε μόνο Pulse Width Modulation. Τα διαγράμματα χρονισμού που ακολουθούν αναπαριστούν τις απαιτήσεις για τη λογική κυματομορφή.  Την ευθύνη για τη μετάδοση έγκυρων bits φέρει ο αποστολέας και για το λόγο αυτό ο αποστολέας σε περίπτωση διένεξης πρέπει να επανασυγχρονίζεται, ώστε η κάθοδος του ρολογιού να πληροί τις προδιαγραφές που τέθηκαν. Αντίστοιχα, ο παραλήπτης πρέπει να περιλαμβάνει ένα ψηφιακό φίλτρο που να καθοδηγείται από ρολόι και έναν ψηφιακό ολοκληρωτή, ή ένα κύκλωμα majority vote sampling για την αποκωδικοποίηση των δεδομένων και το συγχρονισμό με το ρολόι.

Στο J1850 PWM ορίζονται τα παρακάτω bits και σύμβολα:

a.	Λογικό "1" bit

b.	Λογικό "0" bit

c.	Έναρξη Πλαισίου – Start of Frame (SOF)

d.	Τέλος Δεδομένων – End of Data (EOD)

e.	Τέλος Πλαισίου – End of Frame (EOF)

f.	Διαχωριστικό Διάστημα μεταξύ των πλαισίων – Inter-Frame Separation (IFS)

Τα παραπάνω bits και σύμβολα αναλύονται και εξηγούνται ακολούθως:

a) Λογικό "1": Ένα bit ορίζεται ως 1, όταν μία ακμή ανόδου ακολουθεί την προηγούμενη ακμή ανόδου μετά από χρόνο τουλάχιστον ίσο με Tp3 και όταν η ακμή καθόδου εμφανίζεται σε χρόνο Tp1 μετά την ακμή ανόδου. Επίσης δύο διαδοχικές ακμές ανόδου δεν πρέπει να απέχουν λιγότερο από Tp3. Η σχηματική απεικόνιση της παραπάνω περιγραφής παρουσιάζεται στην Εικόνα 3.4:

Εικόνα 3.4

b) Λογικό "0"	:  Αντίστοιχα με τον προηγούμενο ορισμό, ένα bit ορίζεται ως 0, από μια ακμή ανόδου που έπεται της προηγούμενης ακμής ανόδου τουλάχιστον κατά Tp3 και από την ακμή καθόδου που συμβαίνει μετά από Tp2 από την ακμή ανόδου. Τα παραπάνω φαίνονται σχηματικά στην Εικόνα 3.5:

Εικόνα 3.5

c) Η Έναρξη του Πλαισίου – Start of Frame (SOF) ορίζεται από μία ακμή ανόδου που χρησιμοποιείται ως σημείο αναφοράς και ακολουθεί την προηγούμενη ακμή ανόδου μετά από τουλάχιστον Tp5, από μία ακμή καθόδου που εμφανίζεται μετά από Tp7 από το σημείο αναφοράς, ενώ η ακμή ανόδου του πρώτου bit δεδομένων εμφανίζεται μετά από Tp4 από το σημείο αναφοράς όπως φαίνεται και από το σχηματικό διάγραμμα της Εικόνας 3.6:

Εικόνα 3.6

d) Το Τέλος των Δεδομένων – Εnd of Data (EOD)	 χρησιμοποιείται για να δηλώσει ο δημιουργός ενός πλαισίου το τέλος αυτού. Εάν το πλαίσιο περιέχει IFR κομμάτι, τότε αυτό ξεκινά αμέσως μετά το EOD bit (βλ. Εικόνα 3.7). Αν δεν υπάρχει IFR, τότε ο δίαυλος παραμένει σε παθητική κατάσταση για χρόνο ενός ακόμα bit, υποδηλώνοντας με αυτόν τον τρόπο το τέλος του πλαισίου (EOF).

Στο IFR, τα byte(s) απόκρισης ξεκινούν με την ακμή ανόδου του πρώτου bit απόκρισης, Tp4 μετά την ακμή ανόδου του τελευταίου bit που στέλνει ο δημιουργός του πλαισίου. Αν το πρώτο bit του byte απόκρισης δεν εμφανιστεί σε Tp4 και ο δίαυλος παραμείνει σε παθητική κατάσταση για διάρκεια ακόμα ενός bit, (δηλαδή για συνολικό χρόνο Tp5) τότε ο δημιουργός του πλαισίου και όλοι οι παραλήπτες του θεωρούν ότι το πλαίσιο έχει ολοκληρωθεί -στην πραγματικότητα το EOD έχει μετατραπεί σε EOF.

Εικόνα 3.7

e) Το  Τέλος Πλαισίου – End of Frame (EOF)	 ορίζει την ολοκλήρωση ενός πλαισίου. Ουσιαστικά, το πρώτο μέρος του EOF –όπως ειπώθηκε παραπάνω- αποτελεί το EOD. Μετά το τελευταίο byte (συμπεριλαμβανομένου του IFR τμήματος, αν υπάρχει) ο δίαυλος τίθεται σε παθητική κατάσταση. Το EOF έχει διάρκεια Tp5 από την ακμή ανόδου του τελευταίου bit και όταν αυτό ολοκληρωθεί, όλοι οι παραλήπτες θα θεωρήσουν ότι το πλαίσιο έχει τελειώσει.

f) Το Διαχωριστικό Διάστημα μεταξύ των πλαισίων – Inter-Frame Separation (IFS)	 χρησιμεύει στο συγχρονισμό των διαφόρων κόμβων κατά τη μετάδοση διαδοχικών μηνυμάτων. Ένας επίδοξος αποστολέας μηνύματος για να ξεκινήσει τη μετάδοση του SOF του πρέπει να περιμένει είτε να λήξει το IFS (το οποίο αποτελείται από Tp6 μετά την ακμή ανόδου του τελευταίου bit, είτε να διαπιστώσει ότι έχει παρέλθει το ελάχιστο διάστημα EOF και έχει επέλθει η επόμενη ακμή ανόδου.

Η Διακοπή – Break (BRK)	 χρησιμοποιείται για να διαχειριστεί τις καταστάσεις εκείνες όπου κάθε επικοινωνία πρέπει να διακοπεί και κάθε κόμβος πρέπει να τεθεί σε κατάσταση αναμονής για λήψη. Στη διαμόρφωση PWM το σύμβολο Break είναι ένα επιμηκυμένο σύμβολο SOF, το οποίο κάποιες συσκευές θα αναγνωρίσουν ως εσφαλμένο και για το λόγο αυτό θα αγνοήσουν το τρέχον πλαίσιο (αν υπάρχει). Μετά το Break ακολουθεί ένα IFS, (Tp9 μετά την ακμή ανόδου του Break) ώστε να συγχρονιστούν όλοι οι παραλήπτες. Αν ο κόμβος που προκαλεί τη διακοπή χρειάζεται εγγυημένη πρόσβαση στο δίαυλο, τότε πρέπει να στείλει ένα πλαίσιο μέγιστης προτεραιότητας, ώστε να είναι βέβαιο ότι δε θα χάσει την πρόσβαση στο δίαυλο λόγω ανταγωνισμού. H διαγραμματική απεικόνιση των παραπάνω φαίνεται στην Εικόνα 3.8:

Εικόνα 3.8

Κατά τη διάρκεια του Idle Bus (Idle) ο δίαυλος βρίσκεται σε παθητική κατάσταση και κάθε κόμβος μπορεί να ξεκινήσει μετάδοση άμεσα. Παρόλα αυτά μπορεί να προκύψει ανταγωνισμός, αν δύο ή περισσότεροι κόμβοι ξεκινήσουν μετάδοση σχεδόν ταυτόχρονα και για το λόγο αυτό γίνεται επανασυγχρονισμός με την ακμή ανόδου του ρολογιού (Εικόνα 3.9)	.

Εικόνα 3.9

3.4.3.6	Απαιτήσεις Χρονισμού των Συμβόλων PWM – PWM Symbol Timing Requirements

Ο χρονισμός των συμβόλων στην PWM υλοποίηση βασίζεται στη μετάβαση από την παθητική στην ενεργητική κατάσταση	. Το SOF και κάθε bit δεδομένων έχει μία ακμή οδήγησης (leading edge) με βάση την οποία καθορίζεται ο χρονισμός της επόμενης ακολουθίας. Η μετάβαση από ενεργητική σε παθητική κατάσταση (η οποία συμβαίνει μέσα στο SOF ή στα bits δεδομένων) δε χρησιμεύει ως σημείο αναφοράς για το χρονισμό, αλλά η ακμή οδήγησης αποτελεί το μοναδικό σημείο αναφοράς, γιατί η μετάβαση από παθητική σε ενεργητική κατάσταση στο δίαυλο είναι γρήγορη και  αποτελεί μία καθαρή ακμή, αντίθετα με τη μετάβαση από ενεργητική σε παθητική, η οποία είναι αργή και διφορούμενη εξαιτίας των μεταβολών της χωρητικότητας του δικτύου.

Στον Πίνακα 3.4 φαίνονται οι απαιτήσεις χρονισμού για την PWM υλοποίηση με διάρκεια bit 24 μs (41.6Kbps):

| Σύμβολο | Tx,min | Tx,nom | Tx, max | Rx,min | Rx,max |
| --- | --- | --- | --- | --- | --- |
| Tp1: Ενεργή Φάση "1" | 6 | 7 | 8 | 4 | 10 |
| Tp2: Ενεργή Φάση "0" | 14 | 15 | 16 | 12 | 18 |
| Tp3: Διάρκεια Bit | 23 | 24 | 25 | 21 | 27 |
| Tp4: Διάρκεια SOF / EOD | 47 | 48 | 49 | 42 | 54 |
| Tp5: Διάρκεια EOF | 70 | 72 | N/A 1 | 63 | N/A 1 |
| Tp6: Διάρκεια IFS | 93 | 96 | N/A 2 | 84 | N/A 2 |
| Tp7: Ενεργό SOF | 29 | 31 | 32 | 27 | 34 |
| Tp8: Ενεργό BRK | 37 | 39 | 41 | 35 | 43 |
| Tp9: Δίαρκεια Μετάβασης από BRK σε IFS | 116 | 120 | N/A 2 | 105 | N/A 2 |

Πίνακας 3.4:  Χρονικά πλάτη παλμών PWM (μsec)

Η ανοχή στη μετάδοση οφείλεται σε ταλαντώσεις, καθυστερήσεις στο φυσικό επίπεδο και διάφορες άλλες αιτίες, ενώ η ανοχή στη λήψη οφείλεται στη διαφορά τάσης μεταξύ εκπομπού και λήπτη καθώς και σε άλλα αίτια.

Ένα ληφθέν σύμβολο μπορεί να μην έχει σαφή τιμή, για παράδειγμα ένα σήμα το οποίο είναι ενεργό για 11μs, μπορεί να εκληφθεί ως ενεργό φάσης"1" (Tp1) ή ως ενεργό φάσης "0" (Tp2).   Αν ληφθεί λανθασμένη απόφαση τότε αυτή θα εντοπιστεί κατά τον υπολογισμό του CRC Byte. Στην Εικόνα 3.10  φαίνεται ένα παράδειγμα για το πώς πρέπει να αποκωδικοποιηθεί ένα σήμα ανάλογα με το πραγματικό πλάτος του παλμού.

4

Εικόνα 3.10: Παράδειγμα λήψης ενός διφορούμενου συμβόλου Tp1 ή Tp2

Σημειώνεται ότι ένας παλμός που έχει πλάτος μεταξύ Tp1(min) και Tp1(max) πρέπει υποχρεωτικά να αποκωδικοποιείται ως ενεργός φάσης "1", ενώ ένας παλμός πλάτους μεταξύ Tp2(min) και Tp2(max) πρέπει να αποκωδικοποιείται ως ενεργός φάσης "0". Ένας παλμός με πλάτος μεταξύ Tp1(max) και Tp2(min) δεν έχει σαφή τιμή για τα δεδομένα του συστήματος και μπορεί να αποκωδικοποιηθεί είτε ως "1" είτε ως "0".

Συνεχίζοντας στην ενότητα των bit δεδομένων θα πρέπει να αναφερθεί ο όρος Ανίχνευση Διένεξης (Contention Detection)	 με τον οποίο εννοείται η ανίχνευση συγκρουόμενων (=conflicting) συμβόλων ή bits. Διένεξη προκύπτει όταν περισσότεροι από ένας κόμβοι προσπαθούν να αποκτήσουν πρόσβαση στο δίαυλο σχεδόν ταυτόχρονα. Μέσω της διαδικασίας της διαιτησίας bit-by-bit ανιχνεύονται conflicting μεταδόσεις πλαισίων. Όταν ένας κόμβος ανιχνεύσει κάποια διαφορά μεταξύ του συμβόλου ή του bit που έλαβε σε σχέση με αυτό που μετέδωσε αντιλαμβάνεται διένεξη στη μετάδοση του πλαισίου του. Μόνο ο κόμβος του οποίου το πλαίσιο θα κερδίσει όλες τις διαμάχες με όλους τους υπόλοιπους κόμβους που ξεκίνησαν να μεταδίδουν πλαίσια ταυτόχρονα με αυτόν δε θα διαπιστώσει διένεξη.

Ο μηχανισμός της Διαιτησίας Bit-by-Bit 	διευθετεί τα conflicts που προκύπτουν λόγω διενέξεων. Ο μηχανισμός αυτός εφαρμόζεται σε κάθε σύμβολο ή bit του πλαισίου, ξεκινώντας από το SOF και συνεχίζοντας μέχρι το τέλος του πλαισίου. H διαιτησία Bit-by-bit βασίζεται σε φυσικό επίπεδο στην υπεροχή της ενεργητικής έναντι της παθητικής κατάστασης. Συγκεκριμένα όλα τα σήματα που μεταδίδονται στο δίαυλο αποτελούνται από εναλλασσόμενες ενεργητικές και παθητικές καταστάσεις. Κατά την ταυτόχρονη μετάδοση πολλών μηνυμάτων ο δίαυλος βρίσκεται διαρκώς σε ενεργητική  κατάσταση, επομένως αν ο κόμβος που αποστέλλει ένα μήνυμα διαπιστώσει κατά τη διάρκεια αποστολής της κεφαλίδας ότι στο δίαυλο υπάρχει διαφορετικό λογικό επίπεδο τότε σταματά τη μετάδοση του μηνύματός του πριν το επόμενο bit. Αν ο αποστολέας αντιληφθεί διαφορετικό επίπεδο στο δίαυλο από το αναμενόμενο κατά τη διάρκεια του κομματιού δεδομένων του πλαισίου μπορεί να αντιδράσει με έναν από τους εξής τρόπους:

α.	Μπορεί να διακόψει τη μετάδοση πριν την αρχή του επόμενου bit (όπως και στην περίπτωση του σφάλματος στην κεφαλίδα). Αν η μετάδοση διακοπεί πρόωρα λόγω διένεξης στο τέλος κάποιου byte υπάρχει μία περίπτωση στις 28 το τελευταίο byte που θα ληφθεί να είναι το CRC byte των προηγούμενων bytes και άρα το frame να εκληφθεί ως έγκυρο. Για να αντιμετωπιστεί αυτό το ενδεχόμενο όλοι οι κόμβοι πρέπει να επιβεβαιώνουν ότι τα μηνύματα που έλαβαν έχουν το σωστό μήκος.

β.  	Μπορεί να μεταδώσει επιπλέον bits (λιγότερα από 8) όταν ανιχνεύσει διένεξη στο τέλος ενός byte, ώστε όλοι οι άλλοι παραλήπτες να λάβουν ένα σφάλμα πλαισίου σε περίπτωση που προκύψει διένεξη λόγω θορύβου.

Η πρόσβαση στο δίαυλο παραχωρείται με προτεραιότητα στους κόμβους που αποστέλλουν ένα ενεργητικό σήμα σε σχέση με αυτούς που αποστέλλουν παθητικό σήμα. Στην Εικόνα 3.11 φαίνεται η διαδικασία αυτή σε φυσικό επίπεδο.

Εικόνα 3.11:  Διαιτησία

Όπως έχει ήδη ειπωθεί, η διαδικασία της διαιτησίας εφαρμόζεται σε ολόκληρο το πλαίσιο. Καθώς μεταδίδεται κάθε σύμβολο ή bit κάθε ταυτόχρονα μεταδιδόμενου πλαισίου όλοι οι κόμβοι ανιχνεύουν ενδεχόμενη διένεξη. Οι κόμβοι που ανιχνεύουν διένεξη χάνουν την προτεραιότητά τους και σταματούν τη μετάδοση ενώ οι εναπομείναντες κόμβοι συνεχίζουν κανονικά τη μετάδοση. Επομένως ο κόμβος που τελικώς επικρατεί στο δίαυλο είναι αυτός που δε χρειάστηκε να ανιχνεύσει διένεξη.

Η Διαιτησία όπως περιγράφηκε παραπάνω εφαρμόζεται σε όλα τα σύμβολα ή bits αρχίζοντας από το SOF και φτάνοντας μέχρι και το EOF, καθώς και στα σύμβολα του IFR –αν αυτό υπάρχει.

Εκτός από όσα έχουν ήδη αναφερθεί, μέσω της διαδικασίας της διαιτησίας υλοποιείται ένας μηχανισμός απόδοσης προτεραιότητας στα πλαίσια. Αν η διαδικασία της διαιτησίας ολοκληρωθεί πριν την εκκίνηση της μετάδοσης του τμήματος δεδομένων του πλαισίου, αν δηλαδή απορριφθούν όλα τα πλαίσια πλην ενός, τότε ουσιαστικά έχει λάβει χώρα μια διαδικασία απόδοσης προτεραιότητας.

Αν δύο ή περισσότεροι κόμβοι προσπαθήσουν να αποκτήσουν πρόσβαση στο δίαυλο κατά τη διάρκεια του ίδιου παραθύρου συγχρονισμού των πλαισίων τότε η διαιτησία θα γίνει με βάση την τιμή του bit κάθε πλαισίου που μεταδίδεται εκείνη τη χρονική στιγμή. Με δεδομένο ότι η ενεργητική κατάσταση υπερισχύει της παθητικής το πλαίσιο που μετέδωσε το ενεργητικό σήμα θα επιβεβαιώσει το μήνυμα που έστειλε και άρα θα κερδίσει τη διαιτησία. Ως αποτέλεσμα, τη διαδικασία της διαιτησίας κερδίζουν (ή θα έπρεπε να κερδίζουν) τελικά τα πλαίσια μεγαλύτερης προτεραιότητας, γεγονός που αιτιολογεί γιατί η διαιτησία χαρακτηρίστηκε ως μηχανισμός απόδοσης προτεραιότητας.

Αν λάβουμε επιπλέον υπ’ όψιν ότι η ενεργητική κατάσταση αντιστοιχεί στο λογικό μηδέν, γίνεται άμεσα αντιληπτό ότι τα πλαίσια των οποίων τα πρώτα bytes μετά το SOF έχουν την μικρότερη τιμή έχουν μεγαλύτερη προτεραιότητα έναντι των υπολοίπων, με το μηδέν να αποτελεί προφανώς την τιμή με την απόλυτη προτεραιότητα. Ο μηχανισμός αυτός λειτουργεί με τον ίδιο ακριβώς τρόπο άσχετα με τον αριθμό των bytes που θα χρησιμοποιηθούν για την διαιτησία.

3.4.3.7 Αφύπνιση Κόμβων μέσω του Φυσικού Επιπέδου - Node Wake-Up Via Physical Layer

Η μετάβαση από μία ανενεργή κατάσταση ή κατάσταση αναμονής σε λειτουργική κατάσταση είναι μια σημαντική ανάγκη στα δίκτυα των οχημάτων. Η μετάβαση από τη μία κατάσταση στην άλλη καθορίζεται από το Επίπεδο Τομέα – Session Layer, το οποίο ορίζεται από δύο διαφορετικές σκοπιές, απαραίτητες και οι δύο για να οριστεί πλήρως η διαδικασία Αφύπνισης Κόμβων. Αφενός (α) από τη σκοπιά του Υλικού Δικτύου, χωρίς να λαμβάνονται υπ’ όψιν οι κόμβοι, αλλά και (β) από την σκοπιά του κάθε κόμβου ξεχωριστά όπως αναλύεται στη συνέχεια:

α) Υλικό Δικτύου

Ένα δίκτυο μπορεί να χαρακτηριστεί ως Unbiased	 ή Biased (μη πολωμένο ή πολωμένο), ανάλογα με το αν βρίσκεται σε λειτουργική κατάσταση ή όχι.

Σε ένα unbiased δίκτυο όλοι οι κόμβοι βρίσκονται σε επίπεδο γης και η σύνθετη αντίσταση των κόμβων δεν ελέγχεται. Σε ένα τέτοιο δίκτυο δεν μπορεί να υπάρξει κανενός είδους επικοινωνία, αλλά πρέπει το δίκτυο να έρθει πρώτα σε biased κατάσταση. Η μετάβαση από τη μία κατάσταση στην άλλη μπορεί να λειτουργήσει ως σήμα αφύπνισης για συγκεκριμένες εφαρμογές, χωρίς αυτό όμως να είναι απόλυτο. Για παράδειγμα είναι δυνατό σε ένα biased δίκτυο να υπάρχουν κόμβοι σε κατάσταση ύπνου ή σε ένα unbiased δίκτυο να υπάρχουν ενεργοί σταθμοί.

Σε ένα Biased δίκτυο όλοι οι κόμβοι βρίσκονται σε παθητικό επίπεδο (όταν δεν υπάρχει κάποια ανταλλαγή μηνυμάτων), ενώ οι σύνθετες αντιστάσεις είναι κατάλληλες ώστε να μπορεί να λάβει χώρα επικοινωνία. Οι μεμονωμένοι κόμβοι μπορεί να βρίσκονται σε κάποια από τις τρείς καταστάσεις που περιγράφονται παρακάτω. Σημειώνεται ότι οι κόμβοι οι οποίοι βρίσκονται στην (προαιρετική) κατάσταση ύπνου πρέπει να “ξυπνήσουν” μέσα στον κατάλληλο “χρόνο αφύπνισης”. Με άλλα λόγια, όλοι οι κόμβοι που μπορούν να ξυπνήσουν με κατάλληλα σήματα από το δίκτυο, υποχρεώνονται να το κάνουν. Σε αντίθετη περίπτωση οι κόμβοι αυτοί θεωρείται ότι δεν τροφοδοτούνται (unpowered). Με τον περιορισμό αυτό, καθορίζεται ένα μέγιστο (και όχι άπειρο) χρονικό διάστημα καθυστέρησης που είναι αποδεκτό μέχρις ότου ξυπνήσουν όλοι οι κόμβοι.

Ο χρόνος που χρειάζεται η μετάβαση από την unbiased στην biased κατάσταση δεν καθορίζεται σαφώς, αλλά αφήνεται στην ευχέρεια του σχεδιαστή, καθώς η παράμετρος αυτή εξαρτάται από τις ανάγκες της κάθε εφαρμογής, ενώ μπορεί να μην υφίσταται καν, αν για παράδειγμα το δίκτυο είναι μόνιμα biased. Ομοίως δεν ορίζεται και ο χρόνος μετάβασης μεταξύ των υπολοίπων καταστάσεων.

β) Μεμονωμένοι Κόμβοι - Individual Nodes

Οι μεμονωμένοι κόμβοι μπορεί να βρίσκονται σε μία από τις ακόλουθες καταστάσεις:

- Μη Τροφοδοσίας - Unpowered Node:  Ένας κόμβος που βρίσκεται σε αυτήν την κατάσταση δεν μπορεί ούτε να επικοινωνήσει με το δίκτυο, αλλά ούτε και να αντιδράσει σε ενδεχόμενα σήματα αφύπνισης.

- Ύπνου – Sleeping Node:	 Ένας κόμβος που βρίσκεται σε αυτή την κατάσταση μπορεί να έχει κάποια (προαιρετική) κατάσταση χαμηλής ενέργειας (low power standby mode), από την οποία να μπορεί να ξυπνήσει αν ανιχνεύσει κάποιο σήμα αφύπνισης. Κάθε συσκευή του δικτύου που βρίσκεται σε κατάσταση ύπνου μπορεί να αφυπνιστεί είτε από κάποια άλλη συσκευή μέσω του δικτύου είτε από το host μέσω της διεπαφής. Δεν υπάρχει καμία συγκεκριμένη απαίτηση για τη μετάβαση από την ενεργή κατάσταση σε κατάσταση ύπνου.

- Ενεργός/ Λειτουργικός – Μη Ενεργός/Λειτουργικός – Awake / Operational	 - An awake / operational: Πρόκειται για κόμβους που μπορούν να λαμβάνουν και να αποστέλλουν πλαίσια στο δίκτυο.  Ο χρόνος μετάβασης από την αποστολή του σήματος αφύπνισης μέχρι την πλήρη ενεργοποίηση του κόμβου ώστε να μπορεί να επικοινωνεί στο δίκτυο εξαρτάται από την εφαρμογή.  Η μετάβαση του δίαυλου από παθητική σε ενεργητική κατάσταση μπορεί να εκληφθεί ως σήμα αφύπνισης από έναν μέχρι εκείνη τη στιγμή κοιμώμενο κόμβο.

3.4.3.8 Σφάλματα του Φυσικού Επιπέδου – Physical Layer Fault Considerations

Σε περίπτωση που υπάρξει κάποια αστοχία το δίκτυο ανταποκρίνεται σε αυτήν υποχρεωτικά με συγκεκριμένο τρόπο (Required Fault Tolerant Modes ) όπως αναλύεται στη συνέχεια	:

- Απώλεια Τροφοδοσία Κόμβου: Όλοι οι κόμβοι συνεχίζουν να ανταποκρίνονται στις ηλεκτρικές απαιτήσεις του δικτύου σε περίπτωση που υπάρξει πτώση τάσης ή διακοπή της τροφοδοσίας.

- Δίαυλος Συνδεδεμένος στη γη: Όταν ο δίαυλος γειώνεται η επικοινωνία στο δίκτυο διακόπτεται, αλλά δεν προκαλείται βλάβη σε κανέναν κόμβο.

- Δίαυλος Συνδεδεμένος στην μπαταρία: Όταν ο δίαυλος βρίσκεται στην τάση της μπαταρίας η επικοινωνία στο δίκτυο διακόπτεται, αλλά δεν προκαλείται βλάβη σε κανέναν κόμβο.

- Απώλεια γείωσης ενός κόμβου: Όταν ένας κόμβος χάσει το επίπεδο γης του, τότε η επικοινωνία μεταξύ των υπόλοιπων κόμβων συνεχίζονται κανονικά.

Προαιρετικά, μπορεί να γίνει πρόβλεψη ώστε το δίκτυο να αντιδρά κατά προκαθορισμένο τρόπο και σε άλλες περιπτώσεις αστοχίας (Optional Fault Tolerant Modes). Συγκεκριμένα όταν 	αλλοιωθεί ο τερματισμός ή το biasing σε κάποιο σημείο του δικτύου, καθώς και όταν ένας κόμβος αποσυνδέεται από το δίκτυο δεν αλλοιώνεται η λειτουργικότητα του δικτύου και οι υπόλοιποι κόμβοι εξακολουθούν να ανταλλάζουν μηνύματα. Επίσης σε συστήματα με δίαυλο διπλού καλωδίου, οι κόμβοι εξακολουθούν να έχουν πλήρη επικοινωνία και με τον προκαθορισμένο ρυθμό bit στις ακόλουθες περιπτώσεις:

(1)	Ανοιχτοκύκλωμα του "+" ή του "-" του δίαυλου

(2)	Γείωση του "+" ή του "-" του δίαυλου

(3)	Σύνδεση του "+" ή του "-" του δίαυλου με την τάση της μπαταρίας.

(4)	Σύνδεση του "+" ή του "-" του δίαυλου με οποιαδήποτε άλλη τάση

μεταξύ της τάσης της γης και της τάσης της μπαταρίας.

Για τη χρονική περίοδο που υφίσταται η βλάβη στο δίκτυο, η ανοχή στο θόρυβο καθώς και οι παρεμβολές μπορεί να αλλάξουν, όμως μετά την αποκατάσταση της βλάβης επανέρχονται σε κανονικά επίπεδα.

Όλοι οι κόμβοι προστατεύονται από τις προαναφερθείσες βλάβες, ώστε να μπορούν να επανέλθουν αυτόματα στην αρχική λειτουργική τους κατάσταση όταν διορθωθεί η βλάβη.

Τέλος σε περίπτωση που το "+" και το "-" του δίαυλου βραχυκυκλωθούν, η επικοινωνία διακόπτεται και δεν προβλέπεται κάποια αντίδραση από το δίκτυο.

Στον παρακάτω πίνακα (Πίνακας 3.5) παρουσιάζονται όλες οι DC παράμετροι της PWM:

| Παράμετρος | Σύμβολο | Min | Typ | Max | Μονάδες |
| --- | --- | --- | --- | --- | --- |
| Input High Voltage | Vih | 2.80 | --- | 6.25 | volts |
| Input Low Voltage | Vil | -1.00 | --- | 2.20 | volts |
| Output High Voltage | Voh | 3.80 | --- | 5.25 | volts |
| Output Low Voltage | Vol | 0.00 | --- | 1.20 | volts |
| Absolute Ground Offset Voltage | Vgo | 0.00 | --- | 1.00 | volts |
| Bus (+) Driver & Bus (-) Termination Supply Voltage | Vsup | 4.75 | 5.00 | 5.25 | volts |
| Receiver Input Common Mode Operating Range | Vcm | 1.80 | --- | 2.75 | volts |
| Receiver Hysteresis & Overdrive | Vhys | --- | --- | 180 | mvolts |
| Network Resistance (each wire) | Rload | 85 | --- | 378 | ohms |
| Network Capacitance (each wire) | Cload | 500 | --- | 15,000 | pF |
| Network Time Constant 2 | Tload | --- | --- | 1.35 | μsec |
| Signal Transition Time | Tt | --- | --- | 1.75 | μsec |
| Node Resistance (unit load, each wire) | Rul | --- | 2,880 | --- | ohms |
| Node Capacitance (unit load, each wire to ground) | Culg | --- | 250 | --- | pF |
| Node Capacitance (unit load, wire-to-wire) | Culw | --- | 10 | --- | pF |
| Node Leakage Current (each wire - active state) | IleakA | --- | --- | 100 | μAmp |
| Node Leakage Current (each wire - passive state, unpowered node) | IleakPU | --- | --- | 100 | μAmp |
| Node Leakage Current (each wire - passive state, powered node) | IleakPP | --- | --- | 20 | μAmp |

Πίνακας 3.5: DC παράμετροι PWM

3.5 	Υλοποίηση Δικτύου – Network Implementation

Τα μηνύματα ξεκινούν με ένα ή τρία bytes που αποτελούν την κεφαλίδα (Header). Τα bytes της κεφαλίδας ορίζουν πλήρως τις απαιτήσεις της υλοποίησης του δικτύου. Στα παρακάτω σχήματα (Σχήμα 3.1, 3.2, 3.3) φαίνονται η γενική μορφή της κεφαλίδας ενός και τριών bytes.

| Single Byte Header: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | 6 | 5 |  | 4 | 3 | 2 |  | 1 | 0 |
| ID Μηνύματος (256) |  |  |  |  |  |  |  |  |  |
| One Byte Form of Consolidated Header: |  |  |  |  |  |  |  |  |  |
| Bit 7 | 6 | 5 |  | 4 | 3 | 2 |  | 1 | 0 |
| X | x | x |  | H=1 | X | X |  | x | x |
| ID Μηνύματος (128) |  |  |  |  |  |  |  |  |  |
| Σχήμα 3.1: Single Byte Header & One Byte Form of Consolidated Header Three Byte Form of Consolidated Header |  |  |  |  |  |  |  |  |  |
| Byte 1 |  |  | Byte 2 |  |  |  | Byte 3 |  |  |  |
| Βλ. Σχήμα 3.3 |  |  | Διεύθυνση Παραλήπτη |  |  |  | Διεύθυνση Αποστολέα |  |  |  |

Σχήμα 3.2: Three Byte Form of Consolidated Header

Byte 1 of Three Byte Form of Consolidated Header:

| Bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P | P | P | 0 | K | Y | Z | Z |
| Προτεραιότητα (0 έως 7) |  |  | H=0 | Τύπος Μηνύματος (βλ. παρακάτω) |  |  |  |

| Bit | Σημασία | Τιμή | Σημασία |
| --- | --- | --- | --- |
| K | In-Frame Response (IFR) | 0 1 | Απαιτείται IFR Δεν απαιτείται IFR |
| Y | Λειτουργία Διευθυνσιοδότηση | 0 1 | Λειτουργική Διευθυνσιοδότηση Φυσική Διευθυνσιοδότηση |
| ZZ | Μήνυμα Ειδικού Τύπου | 0 0 0 1 1 0 1 1 | Η σημασία αυτών των τιμών εξαρτάται από τα Κ και Υ παραπάνω και αναλύονται στο πρωτόκολλο J2178/1. |

Σχήμα 3.3: First Byte of Three Byte Form of Consolidated Header

Εφόσον ολοκληρώθηκε η παρουσίαση και η επεξήγηση του πρωτοκόλλου στο οποίο βασίστηκε η εφαρμογή, ακολουθεί η αναλυτική περιγραφή της διαδικασίας που ακολουθήθηκε για να σχεδιαστεί και να υλοποιηθεί τόσο το ηλεκτρονικό κύκλωμα όσο και το γραφικό περιβάλλον (GUI). Παράλληλα θα αναλυθεί η λειτουργία τους και ο τρόπος που επικοινωνούν προκειμένου να ληφθεί τελικά το επιθυμητό αποτέλεσμα.

# ΠΕΡΙΓΡΑΦΗ ΤΗΣ ΔΙΑΔΙΚΑΣΙΑΣ

# 4.1 Σχεδιασμός και ανάλυση του ηλεκτρονικού κυκλώματος

# Στην παράγραφο που ακολουθεί θα περιγραφεί το ηλεκτρονικό κύκλωμα και η λειτουργία του. Το κύκλωμα αποτελείται από τα εξής στάδια:

    - Μικροελεγκτής

# Χρησιμοποιήθηκε ένα ολοκληρωμένο ELM327, το οποίο είναι ένας μικροελεγκτής Motorola (PIC18F2480) με ενσωματωμένες τις απαραίτητες ρουτίνες για υποστήριξη πρωτοκόλλων OBD.

Εικόνα 4.1: Μικροελεγκτής

# Το κυρίως κύκλωμα του μικροελεγκτή περιλαμβάνει έναν κρύσταλλο των 4MHz για χρονισμό, καθώς και 4 LEDs ενδεικτικά λειτουργίας. Ο μικροελεγκτής τροφοδοτείται μέσω μίας διόδου και ενός ρυθμιστή τάσης 78L05 ο οποίος είναι απαραίτητος για να υποβαθμίσει την τάση του αυτοκινήτου από τα 12V στα 5V που είναι και η τάση λειτουργίας του ολοκληρωμένου. Στο σημείο αυτό θα πρέπει να παρατηρήσουμε ότι χρησιμοποιήθηκε ρυθμιστής μέγιστου ρεύματος 1A, ενώ το κύκλωμα θα μπορούσε οριακά να τροφοδοτηθεί και μέσω ενός ρυθμιστή των 100mA.

# Μέσω του διαιρέτη τάσης (R34, R35) το κύκλωμα δίνει την τάση της μπαταρίας του οχήματος. Ο διαιρέτης χρησιμοποιείται για να περιορίσει την τάση σε ασφαλή επίπεδα για το ολοκληρωμένο (0-5V). (Σημειώνεται ότι οι τιμές των αντιστάσεων αυτών βρέθηκαν μετά από βαθμονόμηση –καλιμπράρισμα- με συγκεκριμένη μπαταρία και σε κάθε νέα εφαρμογή θα πρέπει να υπολογίζονται εκ νέου). Τέλος σε κάθε στάδιο (τροφοδοσία, υπολογισμός τάσης μπαταρίας, χρονισμός) χρησιμοποιήθηκαν πυκνωτές οι οποίοι γειώνουν τον υψίσυχνο θόρυβο.

# Εικόνα 4.2: Κύκλωμα μικροελεγκτή με τα κυκλώματα τροφοδοσίας, χρονισμού και υπολογισμού τάσης μπαταρίας

    - Διασύνδεση J1850 PWM

# Το πρωτόκολλο J1850 PWM χρειάζεται μια θετική τάση τροφοδοσίας 5V. Το κανάλι J1850+ οδηγείται από ένα NPN (Q1) και ένα PNP (Q2) τρανζίστορ, ενώ το κανάλι J1850- από ένα NPN (Q3). Επειδή το ολοκληρωμένο διαθέτει μία είσοδο πρέπει να μετατραπεί η διαφορική έξοδος του οχήματος σε μία μονή είσοδο για χρήση από το μικροελεγκτή. Για το λόγο αυτό χρησιμοποιείται το PNP τρανζίστορ Q4, το οποίο ενέχει ρόλο διαφορικού ενισχυτή. Το Q4 σε σειρά με την δίοδο D3 θέτουν ένα κατώφλι τάσης περίπου 1V, για ανοχή στο θόρυβο. Η αντίσταση R11 περιορίζει το ρεύμα, ενώ η R12 αναγκάζει το Q4 να μην άγει όταν η είσοδος είναι ανοιχτοκυκλωμένη. Επιπλέον η αντίσταση R36 αναγκάζει το Q4 να “κλείσει” πιο γρήγορα.

# Εικόνα 4.3: Κύκλωμα εισόδου J1850 PWM

    - Διασύνδεση RS232

# Επειδή στα σύγχρονα οχήματα δεν υπάρχει αρνητική τάση τροφοδοσίας, η διασύνδεση RS232 έχει σχεδιαστεί ώστε να λειτουργεί χωρίς αυτή. Ως αποτέλεσμα το κύκλωμα τραβάει ρεύμα από τον υπολογιστή για να λειτουργήσει. Στο κύκλωμα έχει υλοποιηθεί μία τυπική διασύνδεση RS232 με διακριτά στοιχεία, με χρήση 9pin βύσματος. Για μεγαλύτερη απλότητα θα μπορούσε να έχει χρησιμοποιηθεί κάποιο ολοκληρωμένο κύκλωμα, καθώς υπάρχει πλήρης συμβατότητα των περισσότερων ολοκληρωμένων της αγοράς με τις εξόδους του μικροελεγκτή.

# Επιπλέον στην πλακέτα υλοποιείται διασύνδεση για σύνδεση με το πρωτόκολλο ISO–9141 και CAN, τα οποία όμως δεν υποστηρίζονται από το γραφικό περιβάλλον.

# Εικόνα 4.4: Κύκλωμα εξόδου RS232

    - Διασύνδεση CAN και ISO-9141

Στη διασύνδεση CAN έχει χρησιμοποιηθεί το MCP2551, το οποίο είναι ένας CAN Transceiver, αντικαθιστώντας την υλοποίηση με διακριτά στοιχεία η οποία θα αύξανε την πολυπλοκότητα. Στη διασύνδεση ISO-9141, χρησιμοποιούνται δύο NPN τρανζίστορ, τα Q6 και Q7, τα οποία έχουν συνδεδεμένες στο συλλέκτη τους αντιστάσεις ονομαστικής τιμής 510Ω και ισχύος ½W, αφού για τάση 13.8V έχουμε ισχύ πάνω στις αντιστάσεις της τάξης των 0.4W. Τα δεδομένα εισέρχονται από την Κ-είσοδο (εδώ θα πρέπει να σημειωθεί ότι κάποια οχήματα δε χρησιμοποιούν L-γραμμή) και οι αντιστάσεις R20 και R21 υποβαθμίζουν την τάση του σήματος, και θέτουν το κατώφλι στα 9.1V (rising) και 4.7V (falling), προσφέροντας μεγάλη ανοχή στο θόρυβο και προστατεύοντας το ολοκληρωμένο.

    - Διασύνδεση με το όχημα

Τέλος πρέπει να αναφερθεί ότι όλες οι είσοδοι των υποστηριζόμενων πρωτοκόλλων συνδέονται με το όχημα μέσω του ειδικού αντάπτορα (J1962), οι υποδοχές του οποίου φαίνονται στο παρακάτω σχήμα (Εικόνα 4.5). Προφανώς κάθε όχημα έχει ενεργές μόνο τις υποδοχές εκείνες που αντιστοιχούν στο πρωτόκολλο που υποστηρίζεται από το όχημα.

# Εικόνα 4.5: Υποδοχή J1962

# Στον παρακάτω πίνακα φαίνεται η ‘λειτουργία’ κάθε pin του J1962:

| # Pin | # Λειτουργία | # Pin | # Λειτουργία |
| --- | --- | --- | --- |
| # 1 | # (SW CAN) | # 9 |  |
| # 2 | # J1850+ | # 10 | # J1850- |
| # 3 | # MS CAN H | # 11 | # MS CAN L |
| # 4 | # Chassis Ground | # 12 |  |
| # 5 | # Signal Ground | # 13 |  |
| # 6 | # CAN H | # 14 | # CAN L |
| # 7 | # ISO-9141 K-Line | # 15 | # ISO-9141 L-Line |
| # 8 |  | # 16 | # Vehicle Battery |

# Για τους ακροδέκτες που έχουν κενό δεν καθορίζεται λειτουργία από το SAE-J1979, αλλά επιλέγεται από τον κατασκευαστή.

Διασύνδεση CAN

Διασύνδεση με το όχημα

Διασύνδεση ISO-9141

# Εικόνα 4.6: Κύκλωμα εισόδου CAN, ISO-9141 και διασύνδεση με το όχημα.

    - Συνολικό Κύκλωμα

# Όλα τα προαναφερθέντα στάδια φαίνονται στο ολοκληρωμένο σχηματικό διάγραμμα που ακολουθεί:

Εικόνα 4.7: Συνολικό Κύκλωμα

# Από το παραπάνω σχηματικό και με χρήση εργαλείου σχεδίασης προέκυψαν οι ακόλουθες μάσκες για το πάνω και το κάτω (top και bottom) layer αντίστοιχα:

Εικόνα 4.8: Top Layer

Εικόνα 4.9: Bottom Layer

Με βάση τις παραπάνω μάσκες τυπώθηκε το κύκλωμα στην πλακέτα:

Φωτογραφία 4.1: Επάνω Όψη

Φωτογραφία 4.2: Κάτω Όψη

4.2	Υλοποίηση του γραφικού περιβάλλοντος (GUI)

Το γραφικό περιβάλλον που έχει υλοποιηθεί δίνει τη δυνατότητα στο χρήστη να ανακτήσει τους αποθηκευμένους κωδικούς σφάλματος από το όχημα και να διαγράψει τους κωδικούς αυτούς (σε όσα οχήματα το επιτρέπουν) αφού η βλάβη έχει επιδιορθωθεί, ενώ μπορεί να αναπαραστήσει γραφικά σε πραγματικό χρόνο την ταχύτητα του οχήματος, την ταχύτητα περιστροφής του κινητήρα, καθώς και την τάση της μπαταρίας.

Η εικόνα που παίρνουμε ενεργοποιώντας τα ηλεκτρικά του κινητήρα, χωρίς όμως να τον θέσουμε σε λειτουργία είναι:

Εικόνα 4.10: Αρχικό Παράθυρο Γραφικού Περιβάλλοντος

Επιλέγοντας το “Battery Voltage” το πρόγραμμα στέλνει μια εντολή ΑΤ στο όχημα (ATRV) και λαμβάνει ως απάντηση την τάση της μπαταρίας την οποία και αναγράφει στην οθόνη. Αν επιλέξουμε “Get DTCs”, τότε το πρόγραμμα στέλνει στο όχημα το δεκαεξαδικό κωδικό 010C και λαμβάνει ως απάντηση τους κωδικούς σφάλματος που έχουν αποθηκευθεί στη μνήμη του οχήματος. Κατόπιν το πρόγραμμα προσπαθεί να ταυτοποιήσει κάθε κωδικό που έλαβε με το αντίστοιχο σφάλμα, ανοίγοντας μια βάση δεδομένων και εκτελώντας σειριακή αναζήτηση. Όταν καταφέρει να ταυτοποιήσει τον κωδικό, τον τυπώνει στο αντίστοιχο πεδίο μαζί με την επεξήγηση της βλάβης. Αν δε βρει κάποια αντιστοιχία για τον κωδικό, εμφανίζει στο ίδιο πεδίο το μήνυμα “Unknown Trouble Code”. Στην παρακάτω εικόνα παρατίθενται τα αποτελέσματα που λάβαμε, έχοντας αποσυνδέσει την ανάφλεξη σε έναν κύλινδρο:

Εικόνα 4.11: Αντιστοίχηση κωδικών σε αντίστοιχες βλάβες κατά την πειραματική διαδικασία.

Επιλέγοντας “Clear DTCs”, το πρόγραμμα στέλνει στο όχημα το δεκαεξαδικό κωδικό 04, με αποτέλεσμα τη διαγραφή των κωδικών σφάλματος και την απενεργοποίηση της ένδειξης MIL.

Θα πρέπει να τονιστεί ότι η λειτουργία “ClearDTCs” θα πρέπει να επιλέγεται μόνο όταν όλες οι βλάβες του οχήματος έχουν επισκευαστεί, διότι διαφορετικά υπάρχει κίνδυνος αφενός να προκληθεί σοβαρότερη βλάβη στο όχημα και αφετέρου να αυξηθούν οι εκπομπές καυσαερίων. Για το λόγο αυτό το πρόγραμμα εμφανίζει ένα μήνυμα επιβεβαίωσης διαγραφής όπως φαίνεται και στην Εικόνα 4.11 (με προεπιλογή την ακύρωση διαγραφής), ενώ και αρκετοί κατασκευαστές έχουν προνοήσει, ώστε η ενδεικτική λυχνία να απενεργοποιείται (και οι κωδικοί σφάλματος να διαγράφονται) όταν η βλάβη δεν επανεμφανιστεί για 3 διαδοχικές επανεκκινήσεις του κινητήρα.

Εικόνα 4.12: Μήνυμα επιβεβαίωσης μετά από αίτημα διαγραφής των κωδικών από το χρήστη

Αν θέσουμε σε λειτουργία τον κινητήρα, και επιλέξουμε “RPM” και “Speed” (ή ένα από τα δύο), στο στροφόμετρο απεικονίζονται οι στροφές λειτουργίας του κινητήρα και στο ταχύμετρο η ταχύτητα του οχήματος σε πραγματικό χρόνο. Η ανανέωση γίνεται κάθε 300msec για κάθε γράφημα ξεχωριστά, καθώς αυτό είναι ένα χρονικό παράθυρο μέσα στο οποίο είναι πολύ πιθανό να προλάβει το όχημα να λάβει το αίτημα και να αποκριθεί. Σε ακραίες περιπτώσεις αυτό το χρονικό παράθυρο ενδέχεται να μην επαρκεί, αν στο όχημα συμβαίνουν άλλα υψηλότερης προτεραιότητας συμβάντα, όπως πχ η λειτουργία ενός ηλεκτρονικού συστήματος ασφαλείας. Στις περιπτώσεις αυτές η εικόνα φαίνεται να παγώνει στιγμιαία, πράγμα όμως φυσιολογικό, που οφείλεται στην απόδοση προτεραιότητας που εφαρμόζει το πρωτόκολλο. Η ίδια συμπεριφορά μπορεί να προκύψει –σκόπιμα– αν λόγω υψηλού θορύβου αποτύχει η λήψη έγκυρου μηνύματος, οπότε το στιγμιότυπο θα παραμείνει στιγμιαία παγωμένο, αποφεύγοντας το σύστημα να αναπαραστήσει κάποια τυχαία τιμή αντί της πραγματικής.

Εικόνα 4.13: Παράδειγμα παγωμένου στιγμιότυπου

Στην παραπάνω εικόνα ο κινητήρας λειτουργεί στο ρελαντί και το όχημα είναι ακίνητο, ενώ στην παρακάτω το όχημα κινείται με σχετικά υψηλή ταχύτητα.

Εικόνα 4.14: Πραγματική απεικόνιση της ταχύτητας και των στοφών του οχήματος

Ο κώδικας που γράφτηκε και υλοποίησε το συγκεκριμένο γραφικό περιβάλλον παρατίθεται στο Παράρτημα Α που ακολουθεί.

Στη συνέχεια ακολουθεί αξιολόγηση της εφαρμογής που αναπτύχθηκε και περιγράφηκε παραπάνω ενώ προτείνονται και τρόποι για μελλοντική βελτίωση.

ΑΠΟΤΕΛΕΣΜΑΤΑ

ΣΥΜΠΕΡΑΣΜΑΤΑ

ΚΑΙ ΜΕΛΛΟΝΤΙΚΕΣ

ΕΞΕΛΙΞΕΙΣ

5.1 Δοκιμή και Αποτελέσματα

Για να πιστοποιηθεί η αξιοπιστία και η αποτελεσματικότητα τόσο της πλακέτας που σχεδιάστηκε και τυπώθηκε όσο και του γραφικού περιβάλλοντος πραγματοποιήθηκε δοκιμή του συστήματος σε πραγματικό χρόνο ως εξής:

Σε όχημα Ford Mondeo 1.8 (2004) με κινητήρα βενζίνης, συνδέθηκε η πλακέτα με serial adaptor J1962 – OBD II connector και μέσω σειριακής θύρας συνδέθηκε σε φορητό υπολογιστή (lap top). Στη συνέχεια προκλήθηκε τεχνητά βλάβη με σκοπό να πιστοποιηθεί η ανίχνευσή της από το σύστημα. Η βλάβη αυτή συνίστατο στη αποσύνδεση του μπουζί του τρίτου κυλίνδρου. Ακολούθως τέθηκε σε λειτουργία ο κινητήρας και μετά την εκκίνηση λάβαμε κάποιους ακόλουθους κωδικούς σφάλματος (βλ. Εικόνα 4.10 όπου έχει ληφθεί στιγμιότυπο με τους τυπωμένους κωδικούς σφάλματος).

Παρόλο που η βλάβη ήταν μοναδική το σύστημα ανέφερε περισσότερα σφάλματα. Αυτό είναι αναμενόμενο καθώς η συγκεκριμένη ενέργεια είχε ως αποτέλεσμα τη διακοπή της τροφοδοσίας και σε άλλα τμήματα του οχήματος καθένα από τα οποία ανιχνεύθηκε ως ξεχωριστή βλάβη.

Ο παραπάνω έλεγχος απέδειξε την αποτελεσματικότητα και τη μεγάλη ακρίβεια του συστήματος που σχεδιάστηκε και υλοποιήθηκε.

Πέρα απ’ όσα έχουν περιγραφεί ως τώρα υπάρχουν και πολλές ακόμα ενέργειες που μπορούν να γίνουν μελλοντικά για ακόμα μεγαλύτερη ευκολία και πιο βέλτιστα αποτελέσματα. Κάποιες από αυτές προτείνονται στην παράγραφο 5.2 που ακολουθεί:

5.2 Προτάσεις για μελλοντική εξέλιξη

Προκειμένου η εφαρμογή που δημιουργήθηκε να επεκταθεί προτείνονται τα ακόλουθα:

- Το γραφικό περιβάλλον GUI μπορεί να επεκταθεί και αν εμπλουτιστεί έτσι ώστε να εμφανίζει περισσότερες λειτουργίες. Θα μπορούσε δηλαδή με κατάλληλες προσθήκες να εμφανίζει ταχύτητα οχήματος, γωνία γκαζιού, θερμοκρασία κινητήρα, ενδείξεις αισθητήρα οξυγόνου και άλλες.

- Είναι επίσης δυνατόν να υπάρξει επεμβατική λειτουργία της πλακέτας. Με τον όρο επεμβατική λειτουργία εννοούμε να μπορεί ο χρήστης να κάνει ενέργειες στην πλακέτα των οποίων το αποτέλεσμα να φαίνεται σε πραγματικό χρόνο στο όχημα (για παράδειγμα να ‘πατιέται το γκάζι’ μέσω του γραφικού περιβάλλοντος και αυτό να συμβαίνει την ίδια στιγμή στο όχημα). Αξίζει να τονιστεί σε αυτό το σημείο ότι η επεμβατική λειτουργία μπορεί να εφαρμοστεί μόνο στα συστήματα που είναι by-wire (πχ γκάζι).

- Η συγκεκριμένη πλακέτα που υλοποιήθηκε μπορεί να υποστηρίξει και άλλα πρωτόκολλα πέρα από το SAE J1850 PWM όπως το CAN και το ISO αλλά για να γίνει κάτι τέτοιο χρειάζεται επέκταση του γραφικού περιβάλλοντος.

- Το κύκλωμα που υλοποιήθηκε υποστηρίζει μόνο P-Codes και θα μπορούσε με τις κατάλληλες προσθήκες να υποστηρίζει και εύρεση B, C και U Codes οι οποίοι αναφέρονται σε Body, Chassis και Network προβλήματα αντίστοιχα.

- Ένα πολύ σημαντικό βήμα είναι η δημιουργία και χρησιμοποίηση νέων ντετερμινιστικών πρωτοκόλλων (όπως π.χ. το FlexRay), προκειμένου να αντικατασταθούν διάφορα από τα συστήματα των οχημάτων (όπως τιμόνι, φρένα κλπ), με άλλα αντίστοιχα by-wire συστήματα, δηλαδή συστήματα όπου δεν υπάρχει φυσική, αλλά μόνο ηλεκτρονική/ηλεκτρική σύνδεση μεταξύ των χειριστηρίων τους και των φυσικών μερών τους. Τα τελευταία, έχουν πολύ σημαντικά πλεονεκτήματα μερικά από τα οποία είναι το μικρότερο βάρος του οχήματος και κατά συνέπεια η μικρότερη κατανάλωση, το μειωμένο κόστος και για κάποια από αυτά (πχ. Τιμόνι) η καλύτερη παθητική ασφάλεια.

ΠΑΡΑΡΤΗΜΑ Α

Στο Παράρτημα Α, παρατίθεται ο πλήρης κώδικας του γραφικού περιβάλλοντος που αναπτύχθηκε. Ο κώδικας αυτός συνίσταται σε 6 κλάσεις:

A.1	Main.java

package jserial;

import javax.comm.*;

/**

* @author George

*/

public class Main {

/**

* @param args the command line arguments

*/

SimpleRead serialReader;

SimpleWrite serialWriter;

final SerialGUI serialwin;

SerialPort serialPortCOM3;

public Main() {

serialReader = new SimpleRead();

serialPortCOM3 = serialReader.serialPort;

serialWriter = new SimpleWrite(serialPortCOM3);

serialwin = new SerialGUI();

Gauge gauge1 = serialwin.getCanvas1();

gauge1.setMaximum(7);

gauge1.setCurrent(3);

gauge1.setLegend("RPM*1000");

gauge1.setUnits("");

Gauge gauge2 = serialwin.getCanvas2();

gauge2.setMaximum(240);

gauge2.setCurrent(70);

gauge2.setLegend("KPH");

gauge2.setUnits("");

java.awt.EventQueue.invokeLater(new Runnable() {

public void run() {

serialwin.setVisible(true);

}

});

serialReader.main = this;

serialWriter.main = this;

serialwin.main = this;

}

public static void main(String[] args) {

Main main = new Main();

}

}

A.2	SerialGUI.java

/*

* SerialGUI.java

* Created on January 9, 2008, 6:29 PM

*/

package jserial;

import javax.swing.JOptionPane;

import javax.swing.JTextArea;

import javax.swing.JTextField;

/**

*

* @author George

*/

public class SerialGUI extends javax.swing.JFrame {

Main main;

/** Creates new form SerialGUI */

public SerialGUI() {

setTitle("OBD II ScanTool");

initComponents();

}

/** This method is called from within the constructor to

* initialize the form.

*/

// <editor-fold defaultstate="collapsed" desc="Generated Code">//GEN-BEGIN:initComponents

private void initComponents() {

popupMenu1 = new java.awt.PopupMenu();

jPanel1 = new javax.swing.JPanel();

jButton1 = new javax.swing.JButton();

jScrollPane2 = new javax.swing.JScrollPane();

jTextArea2 = new javax.swing.JTextArea();

jButton2 = new javax.swing.JButton();

jButton3 = new javax.swing.JButton();

jTextField1 = new javax.swing.JTextField();

jButton4 = new javax.swing.JButton();

jButton5 = new javax.swing.JButton();

canvas1 = new Gauge(0,7);

canvas2 = new Gauge(0,240);

popupMenu1.setLabel("PopupMenu");

setDefaultCloseOperation(javax.swing.WindowConstants.EXIT_ON_CLOSE);

jButton1.setText("Speed");

jButton1.addMouseListener(new java.awt.event.MouseAdapter() {

public void mouseClicked(java.awt.event.MouseEvent evt) {

jButton1MouseClicked(evt);

}

});

jTextArea2.setColumns(20);

jTextArea2.setFont(new java.awt.Font("Tahoma", 0, 12));

jTextArea2.setRows(5);

jTextArea2.setText("These Trouble Codes were retreived from the vehicle: ");

jScrollPane2.setViewportView(jTextArea2);

jButton2.setText("Get DTCs");

jButton2.addMouseListener(new java.awt.event.MouseAdapter() {

public void mouseClicked(java.awt.event.MouseEvent evt) {

jButton2MouseClicked(evt);

}

});

jButton2.addActionListener(new java.awt.event.ActionListener() {

public void actionPerformed(java.awt.event.ActionEvent evt) {

jButton2ActionPerformed(evt);

}

});

jButton3.setText("Battery Voltage");

jButton3.addMouseListener(new java.awt.event.MouseAdapter() {

public void mouseClicked(java.awt.event.MouseEvent evt) {

jButton3MouseClicked(evt);

}

});

jTextField1.setText("0.0V");

jButton4.setText("Clear DTCs");

jButton4.addMouseListener(new java.awt.event.MouseAdapter() {

public void mouseClicked(java.awt.event.MouseEvent evt) {

jButton4MouseClicked(evt);

}

});

jButton5.setText("RPM");

jButton5.addMouseListener(new java.awt.event.MouseAdapter() {

public void mouseClicked(java.awt.event.MouseEvent evt) {

jButton5MouseClicked(evt);

}

});

javax.swing.GroupLayout jPanel1Layout = new javax.swing.GroupLayout(jPanel1);

jPanel1.setLayout(jPanel1Layout);

jPanel1Layout.setHorizontalGroup(

jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(javax.swing.GroupLayout.Alignment.TRAILING, jPanel1Layout.createSequentialGroup()

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(jPanel1Layout.createSequentialGroup()

.addContainerGap()

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.TRAILING)

.addGroup(javax.swing.GroupLayout.Alignment.LEADING, jPanel1Layout.createSequentialGroup()

.addComponent(canvas2, javax.swing.GroupLayout.PREFERRED_SIZE, 306, javax.swing.GroupLayout.PREFERRED_SIZE)

.addPreferredGap(javax.swing.LayoutStyle.ComponentPlacement.RELATED, javax.swing.GroupLayout.DEFAULT_SIZE, Short.MAX_VALUE)

.addComponent(canvas1, javax.swing.GroupLayout.PREFERRED_SIZE, 295, javax.swing.GroupLayout.PREFERRED_SIZE))

.addGroup(javax.swing.GroupLayout.Alignment.LEADING, jPanel1Layout.createSequentialGroup()

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addComponent(jButton4, javax.swing.GroupLayout.DEFAULT_SIZE, 139, Short.MAX_VALUE)

.addComponent(jButton2, javax.swing.GroupLayout.DEFAULT_SIZE, 139, Short.MAX_VALUE)

.addComponent(jButton3, javax.swing.GroupLayout.Alignment.TRAILING, javax.swing.GroupLayout.DEFAULT_SIZE, 139, Short.MAX_VALUE))

.addPreferredGap(javax.swing.LayoutStyle.ComponentPlacement.RELATED)

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addComponent(jTextField1, javax.swing.GroupLayout.PREFERRED_SIZE, 77, javax.swing.GroupLayout.PREFERRED_SIZE)

.addComponent(jScrollPane2, javax.swing.GroupLayout.PREFERRED_SIZE, 403, javax.swing.GroupLayout.PREFERRED_SIZE))

.addGap(63, 63, 63)))

.addGap(93, 93, 93))

.addGroup(javax.swing.GroupLayout.Alignment.TRAILING, jPanel1Layout.createSequentialGroup()

.addGap(119, 119, 119)

.addComponent(jButton1)

.addPreferredGap(javax.swing.LayoutStyle.ComponentPlacement.RELATED, 256, Short.MAX_VALUE)

.addComponent(jButton5)

.addGap(223, 223, 223)))

.addContainerGap())

);

jPanel1Layout.setVerticalGroup(

jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(jPanel1Layout.createSequentialGroup()

.addContainerGap()

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addComponent(canvas1, 0, 0, Short.MAX_VALUE)

.addComponent(canvas2, javax.swing.GroupLayout.PREFERRED_SIZE, 275, javax.swing.GroupLayout.PREFERRED_SIZE))

.addPreferredGap(javax.swing.LayoutStyle.ComponentPlacement.RELATED)

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.TRAILING)

.addComponent(jButton5)

.addComponent(jButton1))

.addGap(33, 33, 33)

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(jPanel1Layout.createSequentialGroup()

.addComponent(jButton2, javax.swing.GroupLayout.PREFERRED_SIZE, 32, javax.swing.GroupLayout.PREFERRED_SIZE)

.addGap(18, 18, 18)

.addComponent(jButton4, javax.swing.GroupLayout.PREFERRED_SIZE, 34, javax.swing.GroupLayout.PREFERRED_SIZE))

.addComponent(jScrollPane2, javax.swing.GroupLayout.PREFERRED_SIZE, 118, javax.swing.GroupLayout.PREFERRED_SIZE))

.addGap(41, 41, 41)

.addGroup(jPanel1Layout.createParallelGroup(javax.swing.GroupLayout.Alignment.BASELINE)

.addComponent(jButton3, javax.swing.GroupLayout.DEFAULT_SIZE, javax.swing.GroupLayout.DEFAULT_SIZE, Short.MAX_VALUE)

.addComponent(jTextField1, javax.swing.GroupLayout.PREFERRED_SIZE, javax.swing.GroupLayout.DEFAULT_SIZE, javax.swing.GroupLayout.PREFERRED_SIZE))

.addGap(62, 62, 62))

);

javax.swing.GroupLayout layout = new javax.swing.GroupLayout(getContentPane());

getContentPane().setLayout(layout);

layout.setHorizontalGroup(

layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(javax.swing.GroupLayout.Alignment.TRAILING, layout.createSequentialGroup()

.addContainerGap(javax.swing.GroupLayout.DEFAULT_SIZE, Short.MAX_VALUE)

.addComponent(jPanel1, javax.swing.GroupLayout.PREFERRED_SIZE, javax.swing.GroupLayout.DEFAULT_SIZE, javax.swing.GroupLayout.PREFERRED_SIZE)

.addGap(73, 73, 73))

);

layout.setVerticalGroup(

layout.createParallelGroup(javax.swing.GroupLayout.Alignment.LEADING)

.addGroup(layout.createSequentialGroup()

.addContainerGap(37, Short.MAX_VALUE)

.addComponent(jPanel1, javax.swing.GroupLayout.PREFERRED_SIZE, javax.swing.GroupLayout.DEFAULT_SIZE, javax.swing.GroupLayout.PREFERRED_SIZE))

);

pack();

}// </editor-fold>//GEN-END:initComponents

private void jButton1MouseClicked(java.awt.event.MouseEvent evt) {//GEN-FIRST:event_jButton1MouseClicked

main.serialReader.kph = true;

main.serialReader.ctc = false;

main.serialReader.rpm = false;

main.serialReader.atrv = false;

main.serialWriter.write("010C");

}//GEN-LAST:event_jButton1MouseClicked

private void jButton2MouseClicked(java.awt.event.MouseEvent evt) {//GEN-FIRST:event_jButton2MouseClicked

main.serialReader.kph = false;

main.serialReader.ctc = true;

main.serialReader.rpm = false;

main.serialReader.atrv = false;

jTextArea2.setText("");

main.serialWriter.write("03");

}//GEN-LAST:event_jButton2MouseClicked

private void jButton3MouseClicked(java.awt.event.MouseEvent evt) {//GEN-FIRST:event_jButton3MouseClicked

main.serialReader.kph = false;

main.serialReader.ctc = false;

main.serialReader.rpm = false;

main.serialReader.atrv = true;

jTextField1.setText("");

main.serialWriter.write("ATRV");

}//GEN-LAST:event_jButton3MouseClicked

private void jButton4MouseClicked(java.awt.event.MouseEvent evt) {//GEN-FIRST:event_jButton4MouseClicked

Object[] options = {"Yes", "No"};

int n = JOptionPane.showOptionDialog(jPanel1,

"Are you sure?",

"Reset Confirmation",

JOptionPane.YES_NO_CANCEL_OPTION,

JOptionPane.QUESTION_MESSAGE,

null,

options,

options[1]);

System.out.println(n);

if (n == 0) {

main.serialWriter.write("04");

}

JOptionPane.showMessageDialog(jPanel1, "DTC successfully cleared");

}//GEN-LAST:event_jButton4MouseClicked

private void jButton5MouseClicked(java.awt.event.MouseEvent evt) {//GEN-FIRST:event_jButton5MouseClicked

main.serialReader.kph = false;

main.serialReader.ctc = false;

main.serialReader.rpm = true;

main.serialReader.atrv = false;

jTextField1.setText("");

main.serialWriter.write("010C");

}//GEN-LAST:event_jButton5MouseClicked

private void jButton2ActionPerformed(java.awt.event.ActionEvent evt) {//GEN-FIRST:event_jButton2ActionPerformed

// TODO add your handling code here:

}//GEN-LAST:event_jButton2ActionPerformed

public JTextField getJTextField1() {

return jTextField1;

}

public void setJTextField1(String str) {

jTextField1.setText(str);

}

public JTextArea getJTextArea2() {

return jTextArea2;

}

public Gauge getCanvas1() {

return (Gauge) canvas1;

}

public Gauge getCanvas2() {

return (Gauge) canvas2;

}

/**

* @param args the command line arguments

*/

public static void main(String args[]) {

}

// Variables declaration //GEN-BEGIN:variables

private java.awt.Canvas canvas1;

private java.awt.Canvas canvas2;

private javax.swing.JButton jButton1;

private javax.swing.JButton jButton2;

private javax.swing.JButton jButton3;

private javax.swing.JButton jButton4;

private javax.swing.JButton jButton5;

private javax.swing.JPanel jPanel1;

private javax.swing.JScrollPane jScrollPane2;

private javax.swing.JTextArea jTextArea2;

private javax.swing.JTextField jTextField1;

private java.awt.PopupMenu popupMenu1;

// End of variables declaration//GEN-END:variables

}

A.3	SimpleWrite.java

package jserial;

/**

/*

* @(#)SimpleWrite.java	1.12 98/06/25 SMI

*

* Copyright 2003 Sun Microsystems, Inc. All rights reserved.

* SUN PROPRIETARY/CONFIDENTIAL. Use is subject to license terms.

*/

import java.io.*;

import java.util.*;

import javax.comm.*;

/**

* Class declaration

* @author

* @version 1.10, 08/04/00

*/

public class SimpleWrite {

Enumeration portList;

CommPortIdentifier portId;

String messageString = "Testing…";

SerialPort serialPort;

OutputStream outputStream;

boolean outputBufferEmptyFlag = false;

Main main;

String defaultPort = "COM1";

/**

* Method declaration

*/

public void write(String command) {

try {

outputStream.write(command.getBytes());

} catch (IOException e) {

}

try {

Thread.sleep(300);

} catch (Exception e) {

}

}

public SimpleWrite(SerialPort serialPort) {

this.serialPort = serialPort;

boolean portFound = false;

portList = CommPortIdentifier.getPortIdentifiers();

while (portList.hasMoreElements()) {

portId = (CommPortIdentifier) portList.nextElement();

if (portId.getPortType() == CommPortIdentifier.PORT_SERIAL) {

if (portId.getName().equals(defaultPort)) {

System.out.println("Found port " + defaultPort);

portFound = true;

try {

serialPort =

(SerialPort) portId.open("SimpleWrite", 2000);

} catch (PortInUseException e) {

System.out.println("Port in use.");

}

try {

//outputStream = serialPort.getOutputStream();

outputStream = serialPort.getOutputStream();

} catch (IOException e) {

}

try {

serialPort.setSerialPortParams(9600,

SerialPort.DATABITS_8,

SerialPort.STOPBITS_1,

SerialPort.PARITY_NONE);

} catch (UnsupportedCommOperationException e) {

}

try {

serialPort.notifyOnOutputEmpty(true);

} catch (Exception e) {

System.out.println("Error setting event notification");

System.out.println(e.toString());

System.exit(-1);

}

}

}

if (!portFound) {

System.out.println("port " + defaultPort + " not found.");

}

}

}

/*

public static void main(String[] args) {

SimpleWrite swrite = new SimpleWrite();

boolean portFound = false;

System.out.println(

"Writing \"" + swrite.messageString + "\" to " + swrite.serialPort.getName());

try {

swrite.outputStream.write(swrite.messageString.getBytes());

} catch (IOException e) {

}

try {

Thread.sleep(2000);

} catch (Exception e) {

}

swrite.serialPort.close();

System.exit(1);

} */

}

A.4	SimpleRead.java

/*

* @(#)SimpleRead.java	1.12 98/06/25 SMI

*

* Copyright 2003 Sun Microsystems, Inc. All rights reserved.

* SUN PROPRIETARY/CONFIDENTIAL. Use is subject to license terms.

*/

package jserial;

import java.io.*;

import java.util.*;

import javax.comm.*;

import javax.swing.JComponent;

/**

* Class declaration

*

*

* @author

* @version 1.8, 08/03/00

*/

public class SimpleRead implements Runnable, SerialPortEventListener {

InputStream		      inputStream;

SerialPort		      serialPort;

Thread		      readThread;

/**

* Method declaration

*/

/**

* Constructor declaration

*/

private javax.swing.JComponent output;

Main main;

Hashtable troubles;

boolean ctc = false;

boolean kph = false;

boolean atrv = false;

boolean rpm = false;

public SimpleRead() {

troubles = CSVReader.parseCSV();

String defaultPort = "COM1";

CommPortIdentifier portId = null;

Enumeration portList = CommPortIdentifier.getPortIdentifiers();

while (portList.hasMoreElements()) {

portId = (CommPortIdentifier) portList.nextElement();

if (portId.getPortType() == CommPortIdentifier.PORT_SERIAL) {

if (portId.getName().equals(defaultPort)) {

System.out.println("Found port: "+defaultPort);

break;

}

}

}

try {

serialPort = (SerialPort) portId.open("SimpleReadApp", 2000);

} catch (PortInUseException e) {}

try {

inputStream = serialPort.getInputStream();

} catch (IOException e) {}

try {

serialPort.addEventListener(this);

} catch (TooManyListenersException e) {}

serialPort.notifyOnDataAvailable(true);

try {

serialPort.setSerialPortParams(9600, SerialPort.DATABITS_8,

SerialPort.STOPBITS_1,

SerialPort.PARITY_NONE);

} catch (UnsupportedCommOperationException e) {}

readThread = new Thread(this);

readThread.start();

}

/**

* Method declaration

*/

public void run() {

try {

Thread.sleep(20000);

} catch (InterruptedException e) {}

}

/**

* Method declaration

*/

public static String codeToString(byte[] code) {

Byte fstByte = new Byte(code[0]);

Byte sndByte = new Byte(code[1]);

int hex1 = fstByte/16;

int hex2 = fstByte%16;

int hex3 = sndByte/16;

int hex4 = sndByte%16;

return hexToCodes(hex1) + hex2 + hex3 + hex4;

}

public static String hexToCodes (int hex) {

int mod = hex % 4;

if (hex / 4 == 0) {

return "P" + mod;

} else if (hex / 4 == 1) {

return "C" + mod;

} else if (hex / 4 == 2) {

return "B" + mod;

} else

return "U" + mod;

}

public void serialEvent(SerialPortEvent event) {

switch (event.getEventType()) {

case SerialPortEvent.BI:

case SerialPortEvent.OE:

case SerialPortEvent.FE:

case SerialPortEvent.PE:

case SerialPortEvent.CD:

case SerialPortEvent.CTS:

case SerialPortEvent.DSR:

case SerialPortEvent.RI:

case SerialPortEvent.OUTPUT_BUFFER_EMPTY:

break;

case SerialPortEvent.DATA_AVAILABLE:

byte[] readBuffer = new byte[20];

try {

int numBytes = 0;

while (inputStream.available() > 0) {

numBytes = inputStream.read(readBuffer);

System.out.println("read " + numBytes + " bytes");

String textRead = new String(readBuffer,0,numBytes);

System.out.println(textRead);

if (kph) {

int kph_value = (int) readBuffer[0];

main.serialwin.getCanvas2().setCurrent(kph_value);

main.serialWriter.write("010D");

} else if (ctc) {

while (numBytes > 0) {

byte[] temp = new byte[2];

temp[0] = readBuffer[6-numBytes+1];

temp[1] = readBuffer[6-numBytes+2];

String res = codeToString(temp);

main.serialwin.getJTextArea2().append(res + " : " + troubles.get(res) + "\n");

numBytes -= 2;

}

} else if (rpm) {

byte[] temp = new byte[2];

temp[0] = readBuffer[1];

temp[1] = readBuffer[2];

int A = (int) temp[0];

int B = (int) temp[1];

int rpm_value = ((A*256)+B)/4;

main.serialwin.getCanvas1().setCurrent(rpm_value);

main.serialWriter.write("010C");

}

else if (atrv) {

main.serialwin.setJTextField1(textRead);

}

}

} catch (IOException e) {}

break;

}

}

}

A.5 	CSVReader.java

package jserial;

import java.io.BufferedReader;

import java.io.FileNotFoundException;

import java.io.FileReader;

import java.io.IOException;

import java.util.Hashtable;

import java.util.StringTokenizer;

import java.util.logging.Level;

import java.util.logging.Logger;

/**

* @author George

*/

public class CSVReader {

static String file = "book1.csv";

public static Hashtable parseCSV() {

BufferedReader bfrd = null;

Hashtable result = new Hashtable();

try {

String line;

bfrd = new BufferedReader(new FileReader(file));

while ((line = bfrd.readLine()) != null) {

StringTokenizer st = new StringTokenizer(line,";");

result.put(st.nextToken(), st.nextToken());

}

} catch (IOException ex) {

Logger.getLogger(CSVReader.class.getName()).log(Level.SEVERE, null, ex);

} finally {

try {

bfrd.close();

} catch (IOException ex) {

Logger.getLogger(CSVReader.class.getName()).log(Level.SEVERE, null, ex);

}

}

return result;

}

public static void main(String[] args) {

System.out.println(parseCSV());

}

}

A.6 	Gauge.java

package jserial;

/*

* Copyright (c) 1995 Widget Workshop, Inc. All Rights Reserved.

*/

import java.awt.*;

import java.awt.image.*;

import java.awt.Color;

import java.awt.Canvas;

/**

* <P>

* Accessible Attributes:

* <PRE>

* MinimumValue      Low end of the scale

* MaximumValue      High end of the scale

* CurrentValue      Current setting (where the Pointer points)

* CriticalThreshold Percentage of MaxValue at which we start drawing using the Critical color

* WarningThreshold  Percentage of MaxValue at which we start drawing using the Warning color

* Legend            What we're measuring (e.g., Speed)

* Units             What units we're using (e.g., MPH)

* </PRE>

* <P>

* Attributes not yet publically accessible (next version, perhaps):

* <PRE>

* MinimumAngle  Angle at which Minimum Value will be drawn

* MaximumAngle  Angle at which Maximum Value will be drawn

* Margin        Distance between the Scale and the Component's bounds() rect

* Separation    Distance between Gauge subobjects (e.g, between Pointer and Scale)

* CriticalColor Color used to draw the Critical Section of the Scale(e.g., Color.red)

* WarningColor  Color used to draw the Warning Section of the Scale

* NormalColor   Color used to draw the Normal Section of the Scale

* InternalColor Color used to draw the interior of the Scale

* PenRadius     Used to determine thickness of Pointer and Scale

* Draw3D        Should the background be drawn as a raised rectangle?

* TickSize      How long (in pixels) is a BigTick?

* BigTick       Interval starting at Minimum at which BigTicks will be drawn

* LittleTick    Interval starting at Minimum at which LittleTicks will be drawn

* </PRE>

* <P>

* @author	Grant R. Gainey

* @version	1.0 03-JAN-1996

*

*/

public class Gauge extends Canvas {

/*

* Base attributes

*/

// Publically settable:

private int 		minVal, maxVal, currVal;

private double 		warnPercent, critPercent;

private String 		legend;

private String 		units;

// Still internal-only:

private Color 		warnColor, critColor, internalColor, normalColor;

private int		separation = 10;

private int 		margin = separation;

private double 		minAngleRads = Math.PI;	// 180 degrees

private double 		maxAngleRads = 0.0;	// zero degrees

private int		penRadius = 1;

private boolean		draw3D = true;

private int		tickSize = 8;

private int		bigTick = 10;

private int		littleTick = 5;

/*

* Computed attributes (derived from Base or elsewhere)

*/

private int		warnVal, critVal;

private String 		maxStr, minStr;

private Dimension 	minSize, maxSize, legendSize, unitsSize;

private Point 		pivotLoc;

private int   		pointerLen;

private int 		minAngleDegs, maxAngleDegs;

private Rectangle 	scaleRect;

private FontMetrics 	myMetrics = null;

private int		halfTick = (int)Math.round(tickSize/2.0);

/**

* Constructs a Gauge object. Uses all default values.

* @see Gauge#init

*/

public Gauge() {

init();

};

/**

* Constructs a Gauge object with specified Min and Max.

* @param min Minimum Gauge value

* @param max Maximum Gauge Value

* @see Gauge#init

*/

public Gauge(int min, int max) {

init();

setMinimum(min);

setMaximum(max);

setCurrent(min);

};

/**

* Constructs a Gauge with the specified Min, Max, Legend, and Units values.

* @param min		Minimum Value

* @param max		Maximum Value

* @param warn		Percentage of Max of start of Warning area

* @param crit		Percentage of Max of start of Critical area

* @param newLegend		Gauge Legend

* @param newUnits		Gauge Units

*/

public Gauge(int min, int max, float warn, float crit, String newLegend, String newUnits) {

init();

setMinimum(min);

setMaximum(max);

setCurrent(min);

warnPercent = warn;

critPercent = crit;

warnVal = (int)Math.round(maxVal*warnPercent);

critVal = (int)Math.round(maxVal*critPercent);

setLegend(newLegend);

setUnits(newUnits);

};

/*

* Utility Routines

*/

// Degrees-to-Radians

private double d2r (int degs) {

return Math.round((degs/180)*Math.PI);

};

// Radians-to-Degrees

private int r2d (double rads) {

return (int)Math.round((rads/Math.PI)*180);

};

// Based on current font, determine dimensions of all strings.

// Reset PointerLen here.

private void setStringMetrics() {

if (myMetrics==null) return;

minSize = new Dimension(myMetrics.stringWidth(minStr),

myMetrics.getHeight());

maxSize = new Dimension(myMetrics.stringWidth(maxStr),

myMetrics.getHeight());

legendSize = new Dimension(myMetrics.stringWidth(legend),

myMetrics.getHeight());

unitsSize = new Dimension(myMetrics.stringWidth(units),

myMetrics.getHeight());

margin += myMetrics.getHeight();

pointerLen = (int)Math.round(getSize().width/2.0) - margin - separation;

};

// Return rect into which the scale-arc will be drawn

private Rectangle findScaleRect() {

return new Rectangle(margin, pivotLoc.y - pointerLen, 2*pointerLen, pointerLen);

};

// Determine locations of Pivot and Scale, and len of Pointer

// Called after any size-change

private void partition() {

pivotLoc = new Point( getSize().width/2, getSize().height/2 );

pointerLen = (int)Math.round(getSize().width/2.0) - margin - separation;

scaleRect = findScaleRect();

};

/**

* init() sets Guage's attributes to a "sane" set of initial values.

* <P>

* Defaults are:

* <PRE>

*  Minimum         = 0

*  Maximum         = 200

*  Current         = Minimum

*  Legend          = "Velocity"

*  Units           = "KPH"

*  WarningThresh   = 0.7

*  CriticalThresh  = 0.9

*  CriticalColor   = Color.red

*  WarningColor    = Color.yellow

*  InternalColor   = Color.black

*  NormalColor     = getForeground()

*  ForegroundColor = Color.green

*  BackgroundColor = Color.gray

*  Font            = "Courier", BOLD, 18

*  </PRE>

*/

protected void init() {

minVal = 0;

maxVal = 8;

currVal = minVal;

warnPercent = 0.7;

critPercent = 0.9;

warnVal = (int)Math.round(maxVal*warnPercent);

critVal = (int)Math.round(maxVal*critPercent);

units = new String("KPH");

minStr = String.valueOf(minVal);

maxStr = String.valueOf(maxVal);

setStringMetrics();

warnColor = Color.orange;

critColor = Color.red;

internalColor = Color.BLUE;

partition();

minAngleDegs = r2d(minAngleRads);

maxAngleDegs = r2d(maxAngleRads);

setForeground(Color.green);

normalColor = getForeground();

setBackground(Color.DARK_GRAY);

setFont(new Font("Courier", Font.ITALIC, 18));

};

/**

* Change the Gauge's Legend (what this Gauge is measuring; e.g., Speed or Swap Rate)

* @param newLegend	Legend's new value

*/

public void setLegend(String newLegend) {

legend = newLegend;

if (myMetrics!=null) {

legendSize = new Dimension(myMetrics.stringWidth(legend),

myMetrics.getHeight());

}

repaint();

};

/**

* Change the Gauge's Units (what Value is measured in; e.g., KPH or PPM)

* @param newUnits	Units's new value

*/

public void setUnits(String newUnits) {

units = newUnits;

if (myMetrics!=null) {

unitsSize = new Dimension(myMetrics.stringWidth(units),

myMetrics.getHeight());

}

repaint();

};

/**

* Change the Gauge's Minimum Value

* @param min New Minimum

*/

public void setMinimum(int min) {

minVal = min;

minStr = String.valueOf(minVal);

if (myMetrics!=null) {

minSize = new Dimension(myMetrics.stringWidth(minStr),

myMetrics.getHeight());

}

repaint();

};

public int getMinimum() {

return minVal;

};

/**

* Change the Gauge's Maximum Value

* @param max New Maximum

*/

public void setMaximum(int max) {

maxVal = max;

maxStr = String.valueOf(maxVal);

if (myMetrics!=null) {

maxSize = new Dimension(myMetrics.stringWidth(maxStr),

myMetrics.getHeight());

}

warnVal = (int)Math.round(maxVal*warnPercent);

critVal = (int)Math.round(maxVal*critPercent);

repaint();

};

public int getMaximum() {

return maxVal;

};

/**

* Change the Gauge's Current Value (the value the pointer points at)

* @param newCurr New Current

*/

public void setCurrent(int newCurr) {

currVal = newCurr;

repaint();

};

public int getCurrent() {

return currVal;

};

/**

* Change the Gauge's Warning Threshold.  This is the percentage of the Maximum Value

* at which the Warning Region begins.

* @param newWarnPercnt	New Warning Threshold

*/

public void setWarningThreshold(float newWarnPcnt) {

warnPercent = newWarnPcnt;

warnVal = (int)Math.round(maxVal*warnPercent);

repaint();

};

public double getWarningThreshold() {

return warnPercent;

};

/**

* Change the Gauge's Critical Threshold.  This is the percentage of the Maximum Value

* at which the Critical Region begins.

* @param newCritPercnt	New Critical Threshold

*/

public void setCriticalThreshold(float newCritPcnt) {

critPercent = newCritPcnt;

critVal = (int)Math.round(maxVal*critPercent);

repaint();

};

public double getCriticalThreshold() {

return critPercent;

};

/**

* At update(); draw into an off-screen image and put up the whole

* image.  This clears up any flickering.

*/

public void update(Graphics g) {

Image osImg = createImage(getSize().width, getSize().height);

osImg.getGraphics().fillRect(0,0,getSize().width, getSize().height);

paint(osImg.getGraphics());

g.drawImage(osImg,0,0,null);

};

/**

* If Draw3d is set, draw the background as a filled 3D rect in the

* current Background Color.  Otherwise, draw a flat rect.

*/

public void drawBackground(Graphics g) {

g.setColor(getBackground());

if (draw3D) {

for (int i=0;i<=3;i++) {

g.fill3DRect(i,i,getSize().width-(2*i), getSize().height-(2*i),true);

}

} else {

g.fillRect(0,0, getSize().width, getSize().height);

}

};

/**

* drawTick draws one tick-mark from center of scale-arc _INWARDS_

* @param where		Value at which to draw the tick-mark

* @param offset		How long to make the tick-mark

*/

public void drawTick(Graphics g, int where, int offset) {

int arcCenter = pointerLen+separation;

Point startPt = mapValToPoint(where, arcCenter-offset);

Point endPt   = mapValToPoint(where, arcCenter);

g.setColor(getValColor(where));

g.drawLine(startPt.x, startPt.y, endPt.x, endPt.y);

};

/**

* drawScale draws the ScaleArc itself, and the tick-marks

*/

public void drawScale(Graphics g) {

// Draw the meter-interior.  Then draw the Arc itself.  Then, draw tickmarks.

g.setColor(internalColor);

g.fillArc(margin, margin,

getSize().width - 2*margin, getSize().height - 2*margin,

maxAngleDegs, (minAngleDegs-maxAngleDegs));

for (int i=-penRadius;i<=penRadius;i++) {

// Draw the critical-color segment

g.setColor(critColor);

g.drawArc(margin+i, margin+i,

getSize().width - 2*margin - 2*i, getSize().height - 2*margin - 2*i,

maxAngleDegs,

(int)Math.round(minAngleDegs*(1.0-critPercent)));

// Draw the warning-color segment

g.setColor(warnColor);

g.drawArc(margin+i, margin+i,

getSize().width - 2*margin - 2*i, getSize().height - 2*margin - 2*i,

(int)Math.round(minAngleDegs*(1.0-critPercent)),

(int)Math.round(minAngleDegs*(critPercent-warnPercent)));

// Draw the normal-color segment

g.setColor(normalColor);

g.drawArc(margin+i, margin+i,

getSize().width - 2*margin - 2*i, getSize().height - 2*margin - 2*i,

minAngleDegs,

(int)Math.round(minAngleDegs*(-warnPercent)));

}

for (int i=minVal;i<=maxVal;i+=littleTick) {drawTick(g,i,halfTick);}

for (int i=minVal;i<=maxVal;i+=bigTick)    {drawTick(g,i,tickSize);}

};

/**

* drawStrings draws the Legend, Units, Min, and Max.

*/

public void drawStrings(Graphics g) {

g.setColor(getForeground());

g.setFont(getFont());

g.drawString(minStr, separation, pivotLoc.y + minSize.height);

g.drawString(maxStr, getLocation().x + getSize().width - 2*separation - maxSize.width -7 ,

pivotLoc.y + minSize.height);

g.drawString(legend,

pivotLoc.x - (int)Math.round(legendSize.width/2),

pivotLoc.y + legendSize.height + margin -15);

g.drawString(units,

pivotLoc.x - (int)Math.round(unitsSize.width/2),

pivotLoc.y + legendSize.height+margin + unitsSize.height+separation);

repaint();

};

/**

* getValColor returns the color associated with the region into which the

* value falls.

*	@param val		value of interest

*/

public Color getValColor(int val) {

if (val < warnVal) {

return normalColor;

} else if (val < critVal) {

return warnColor;

} else {

return critColor;

}

};

/**

* mapValToPoint returns a Point on the Scale for the specified value

*/

public Point mapValToPoint(int val) {

double diffRads = minAngleRads - maxAngleRads;

double valPcnt  = (double)val/(double)maxVal;

double valRads  = minAngleRads - diffRads*valPcnt;

return new Point(pivotLoc.x + (int)Math.round(Math.cos(valRads)*pointerLen),

pivotLoc.y - (int)Math.round(Math.sin(valRads)*pointerLen));

};

/**

* mapValToPoint returns a Point "radius" units from the Pivit for the specified value

*/

public Point mapValToPoint(int val, int radius) {

double diffRads = minAngleRads - maxAngleRads;

double valPcnt  = (double)val/(double)maxVal;

double valRads  = minAngleRads - diffRads*valPcnt;

return new Point(pivotLoc.x + (int)Math.round(Math.cos(valRads)*radius),

pivotLoc.y - (int)Math.round(Math.sin(valRads)*radius));

};

/**

* drawPointer draws the Pivot and the Pointer

*/

public void drawPointer(Graphics g) {

g.setColor(getValColor(currVal));

g.fillOval(pivotLoc.x-5, pivotLoc.y-5, 11, 11);

Point scalePt = mapValToPoint(currVal);

for (int i=-penRadius;i<=penRadius;i++) {

g.drawLine(pivotLoc.x, pivotLoc.y+i, scalePt.x, scalePt.y+i);

}

};

/**

* When resized, recalculate where everything lives.

* <P>

* Rules for resizing a Gauge:

* <P>

*  <UL>

*  <LI>Gauge MUST be square; take the smaller of w/h as new side-length

*  <LI>Gauge MUST be at least its min-size

*  </UL>

*/

public void setSize(int w, int h) {

int side = Math.max(Math.min(w,h), getMinimumSize().width);

super.setSize(side, side);

partition();

repaint();

};

/**

* @see Gauge#resize(int,int)

*/

public void setSize(Dimension dim) {

int side = Math.max(Math.min(dim.width,dim.height), getMinimumSize().width);

dim.width = side; dim.height = side;

super.setSize(dim);

partition();

repaint();

};

/**

* @see Gauge#resize(int,int)

*/

public void setBounds(int x, int y, int w, int h) {

int side = Math.max(Math.min(w,h), getMinimumSize().width);

super.setBounds(x,y,side,side);

partition();

repaint();

};

/**

* A Gauge wants to be no smaller than (100,100)

*/

public Dimension getMinimumSize() {

return new Dimension(100,100);

};

/**

* Gauges prefer their minSize.

* @see Gauge#getMinimumSize

*/

public Dimension getPreferredSize() {

return getMinimumSize();

};

/**

* paint() paints in this order:

* <OL>

* <LI>Background (includes Internal area)

* <LI>Scale (includes TickMarks)

* <LI>Strings (Legend, Units, Min, Max)

* <LI>Pointer (inlcudes Pivot)

* </OL>

*/

public void paint(Graphics g) {

if (myMetrics==null) {

myMetrics = getGraphics().getFontMetrics();

setStringMetrics();

}

//Image osImg = createImage(size().width, size().height);

//Graphics osGfx = osImg.getGraphics();

drawBackground(g);

drawScale(g);

drawStrings(g);

drawPointer(g);

//g.drawImage(osImg, 0, 0, this);

//System.out.println(this.toString());

};

public void repaint() {

super.repaint();

};

public void repaint(long when) {

super.repaint(when);

};

public void repaint(int x, int y, int w, int h) {

super.repaint(x,y,w,h);

};

public void repaint(long when, int x, int y, int w, int h) {

super.repaint(when,x,y,w,h);

};

public void processMouseEvent(java.awt.event.MouseEvent evt) {//, int x, int y) {

super.processMouseEvent((java.awt.event.MouseEvent) evt);//,x,y);

};

public void processMouseMotionEvent(java.awt.event.MouseEvent evt) {//, int x, int y) {

super.processMouseMotionEvent((java.awt.event.MouseEvent) evt);//,x,y);

};

public String toString() {

String strBuf = getClass().getName();

strBuf += "Rect: " +getBounds().toString()+ '\n';

strBuf += "Min : " +minVal+ " Max: " +maxVal+ " Curr: " +currVal+ '\n';

strBuf += "wP  : " +warnPercent+ " cP:" + critPercent+ '\n';

strBuf += "wV  : " +warnVal+ " cV:" + critVal+ '\n';

strBuf += "Lgnd: " +legend+'\n';

strBuf += "Unit: " +units+'\n';

strBuf += "MnAR: " +minAngleRads+ " MnAD: " +minAngleDegs+ '\n';

strBuf += "MxAR: " +maxAngleRads+ " MxAD: " +maxAngleDegs+ '\n';

strBuf += "PivotLoc: " +pivotLoc.toString()+ '\n';

strBuf += "PtrLen: " +pointerLen+ '\n';

strBuf += "SclRect : " +scaleRect.toString()+ '\n';

return strBuf;

}

}

ΠΑΡΑΡΤΗΜΑ Β

Στο παράρτημα που ακολουθεί αναφέρονται οι βασικότεροι κωδικοί σφαλμάτων με τις επεξηγήσεις τους, όπως ενδέχεται να εμφανιστούν στο πεδίο Get DTCs του γραφικού περιβάλλοντος:

  - Βασικότεροι Ρ – Codes:

| P0100 | Mass or Volume Air Flow Circuit Malfunction |
| --- | --- |
| P0101 | Mass or Volume Air Flow Circuit Range/Performance Problem |
| P0102 | Mass or Volume Air Flow Circuit Low Input |
| P0103 | Mass or Volume Air Flow Circuit High Input |
| P0104 | Mass or Volume Air Flow Circuit Intermittent |
| P0105 | Manifold Absolute Pressure/Barometric Pressure Circuit Malfunction |
| P0106 | Manifold Absolute Pressure/Barometric Pressure Circuit Range/Performance Problem |
| P0107 | Manifold Absolute Pressure/Barometric Pressure Circuit Low Input |
| P0108 | Manifold Absolute Pressure/Barometric Pressure Circuit High Input |
| P0109 | Manifold Absolute Pressure/Barometric Pressure Circuit Intermittent |
| P0109 | Intake Air Temperature Circuit Malfunction |
| P0110 | IAT Circuit Malfunction |
| P0111 | Intake Air Temperature Circuit Range/Performance Problem |
| P0112 | Intake Air Temperature Circuit Low Input |
| P0113 | Intake Air Temperature Circuit High Input |
| P0114 | Intake Air Temperature Circuit Intermittent |
| P0115 | Engine Coolant Temperature Circuit Malfunction |
| P0116 | Engine Coolant Temperature Circuit Range/Performance Problem |
| P0117 | Engine Coolant Temperature Circuit Low Input |
| P0118 | Engine Coolant Temperature Circuit High Input |
| P0119 | Engine Coolant Temperature Circuit Intermittent |
| P0120 | Throttle Position Sensor/Switch A Circuit Malfunction |
| P0121 | Throttle Position Sensor/Switch A Circuit Range/Performance Problem |
| P0122 | Throttle Position Sensor/Switch A Circuit Low Input |
| P0123 | Throttle Position Sensor/Switch A Circuit High Input |
| P0124 | Throttle Position Sensor/Switch A Circuit Intermittent |
| P0125 | Insufficient Coolant Temperature for Closed Loop Fuel Control; ECT Excessive Time to Closed Loop Fuel Control |
| P0126 | Insufficient Coolant Temperature for Stable Operation |
| P0128 | Coolant Thermostat Malfunction |
| P0130 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 1) |
| P0131 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 1) |
| P0132 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 1) |
| P0133 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 1) |
| P0134 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 1) |
| P0135 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 1) |
| P0136 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 2) |
| P0137 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 2) |
| P0138 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 2) |
| P0139 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 2) |
| P0140 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 2) |
| P0141 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 2) |
| P0142 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 3) |
| P0143 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 3) |
| P0144 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 3) |
| P0145 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 3) |
| P0146 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 3) |
| P0147 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 3) |
| P0150 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 1) |
| P0151 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 1) |
| P0154 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 1) |
| P0155 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0156 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 2) |
| P0157 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 2) |
| P0158 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 2) |
| P0159 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 2) |
| P0160 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 2) |
| P0161 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 2) |
| P0162 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 3) |
| P0163 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 3) |
| P0164 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 3) |
| P0165 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 3) |
| P0166 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 3) |
| P0167 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 3) |
| P0170 | Fuel Trim Malfunction (Bank 1) |
| P0171 | Fuel Trim System Lean (Bank 1) |
| P0172 | Fuel Trim too Rich (Bank 1) |
| P0173 | Fuel Trim Malfunction (Bank 2) |
| P0174 | Fuel Trim too Lean (Bank 2) |
| P0175 | Fuel Trim too Rich (Bank 2) |
| P0176 | Fuel Composition Sensor Circuit Malfunction |
| P0177 | Fuel Composition Sensor Circuit Range/Performance |
| P0178 | Fuel Composition Sensor Circuit Low Input |
| P0179 | Fuel Composition Sensor Circuit High Input |
| P0180 | Fuel Temperature Sensor A Circuit Malfunction |
| P0181 | Fuel Temperature Sensor A Circuit Range/Performance |
| P0182 | Fuel Temperature Sensor A Circuit Low Input |
| P0183 | Fuel Temperature Sensor A Circuit High Input |
| P0184 | Fuel Temperature Sensor A Circuit Intermittent |
| P0185 | Fuel Temperature Sensor B Circuit Malfunction |
| P0186 | Fuel Temperature Sensor B Circuit Range/Performance |
| P0187 | Fuel Temperature Sensor B Circuit Low Input |
| P0188 | Fuel Temperature Sensor B Circuit High Input |
| P0189 | Fuel Temperature Sensor B Circuit Intermittent |
| P0190 | Fuel Rail Pressure Sensor Circuit Malfunction |
| P0191 | Fuel Rail Pressure Sensor Circuit Range/Performance |
| P0192 | Fuel Rail Pressure Sensor Circuit Low Input |
| P0193 | Fuel Rail Pressure Sensor Circuit High Input |
| P0194 | Fuel Rail Pressure Sensor Circuit Intermittent |
| P0195 | Engine Oil Temperature Sensor Malfunction |
| P0196 | Engine Oil Temperature Sensor Range/Performance |
| P0197 | Engine Oil Temperature Sensor Low |
| P0198 | Engine Oil Temperature Sensor High |
| P0199 | Engine Oil Temperature Sensor Intermittent |
| P0200 | Injector Circuit Malfunction |
| P0201 | Injector Circuit Malfunction - Cylinder 1 |
| P0202 | Injector Circuit Malfunction - Cylinder 2 |
| P0203 | Injector Circuit Malfunction - Cylinder 3 |
| P0204 | Injector Circuit Malfunction - Cylinder 4 |
| P0205 | Injector Circuit Malfunction - Cylinder 5 |
| P0206 | Injector Circuit Malfunction - Cylinder 6 |
| P0207 | Injector Circuit Malfunction - Cylinder 7 |
| P0208 | Injector Circuit Malfunction - Cylinder 8 |
| P0209 | Injector Circuit Malfunction - Cylinder 9 |
| P0210 | Injector Circuit Malfunction - Cylinder 10 |
| P0211 | Injector Circuit Malfunction - Cylinder 11 |
| P0212 | Injector Circuit Malfunction - Cylinder 12 |
| P0213 | Cold Start Injector 1 Malfunction |
| P0214 | Cold Start Injector 2 Malfunction |
| P0215 | Engine Shutoff Solenoid Malfunction |
| P0216 | Injection Timing Control Circuit Malfunction |
| P0217 | Engine Overtemp Condition |
| P0218 | Transmission Over Temperature Condition |
| P0219 | Engine Overspeed Condition |
| P0220 | Throttle/Petal Position Sensor/Switch B Circuit Malfunction |
| P0221 | Throttle/Petal Position Sensor/Switch B Circuit Range/Performance Problem |
| P0222 | Throttle/Petal Position Sensor/Switch B Circuit Low Input |
| P0223 | Throttle/Petal Position Sensor/Switch B Circuit High Input |
| P0224 | Throttle/Petal Position Sensor/Switch B Circuit Intermittent |
| P0225 | Throttle/Petal Position Sensor/Switch C Circuit Malfunction |
| P0226 | Throttle/Petal Position Sensor/Switch C Circuit Range/Performance Problem |
| P0227 | Throttle/Petal Position Sensor/Switch C Circuit Low Input |
| P0228 | Throttle/Petal Position Sensor/Switch C Circuit High Input |
| P0229 | Throttle/Petal Position Sensor/Switch C Circuit Intermittent |
| P0230 | Fuel Pump Primary Circuit Malfunction |
| P0231 | Fuel Pump Secondary Circuit Low |
| P0232 | Fuel Pump Secondary Circuit High |
| P0233 | Fuel Pump Secondary Circuit Intermittent |
| P0234 | Engine Overboost Condition |
| P0235 | Turbocharger Boost Sensor A Circuit Malfunction |
| P0236 | Turbocharger Boost Sensor A Circuit Range/Performance |
| P0237 | Turbocharger Boost Sensor A Circuit Low |
| P0238 | Turbocharger Boost Sensor A Circuit High |
| P0239 | Turbocharger Boost Sensor B Malfunction |
| P0240 | Turbocharger Boost Sensor B Circuit Range/Performance |
| P0241 | Turbocharger Boost Sensor B Circuit Low |
| P0242 | Turbocharger Boost Sensor B Circuit High |
| P0243 | Turbocharger Wastegate Solenoid A Malfunction |
| P0244 | Turbocharger Wastegate Solenoid A Range/Performance |
| P0245 | Turbocharger Wastegate Solenoid A Low |
| P0246 | Turbocharger Wastegate Solenoid A High |
| P0247 | Turbocharger Wastegate Solenoid B Malfunction |
| P0248 | Turbocharger Wastegate Solenoid B Range/Performance |
| P0249 | Turbocharger Wastegate Solenoid B Low |
| P0250 | Turbocharger Wastegate Solenoid B High |
| P0251 | Injection Pump Fuel Metering Control "A" Malfunction (Cam/Rotor/Injector) |
| P0252 | Injection Pump Fuel Metering Control "A" Range/Performance (Cam/Rotor/Injector) |
| P0253 | Injection Pump Fuel Metering Control "A" Low (Cam/Rotor/Injector) |
| P0254 | Injection Pump Fuel Metering Control "A" High (Cam/Rotor/Injector) |
| P0255 | Injection Pump Fuel Metering Control "A" Intermittent (Cam/Rotor/Injector) |
| P0256 | Injection Pump Fuel Metering Control "B" Malfunction (Cam/Rotor/Injector) |
| P0257 | Injection Pump Fuel Metering Control "B" Range/Performance (Cam/Rotor/Injector) |
| P0258 | Injection Pump Fuel Metering Control "B" Low (Cam/Rotor/Injector) |
| P0259 | Injection Pump Fuel Metering Control "B" High (Cam/Rotor/Injector) |
| P0260 | Injection Pump Fuel Metering Control "B" Intermittent (Cam/Rotor/Injector) |
| P0261 | Cylinder 1 Injector Circuit Low |
| P0262 | Cylinder 1 Injector Circuit High |
| P0263 | Cylinder 1 Contribution/Balance Fault |
| P0264 | Cylinder 2 Injector Circuit Low |
| P0265 | Cylinder 2 Injector Circuit High |
| P0266 | Cylinder 2 Contribution/Balance Fault |
| P0267 | Cylinder 3 Injector Circuit Low |
| P0268 | Cylinder 3 Injector Circuit High |
| P0269 | Cylinder 3 Contribution/Balance Fault |
| P0270 | Cylinder 4 Injector Circuit Low |
| P0271 | Cylinder 4 Injector Circuit High |
| P0272 | Cylinder 4 Contribution/Balance Fault |
| P0273 | Cylinder 5 Injector Circuit Low |
| P0274 | Cylinder 5 Injector Circuit High |
| P0275 | Cylinder 5 Contribution/Balance Fault |
| P0276 | Cylinder 6 Injector Circuit Low |
| P0277 | Cylinder 6 Injector Circuit High |
| P0278 | Cylinder 6 Contribution/Balance Fault |
| P0279 | Cylinder 7 Injector Circuit Low |
| P0280 | Cylinder 7 Injector Circuit High |
| P0281 | Cylinder 7 Contribution/Balance Fault |
| P0282 | Cylinder 8 Injector Circuit Low |
| P0283 | Cylinder 8 Injector Circuit High |
| P0284 | Cylinder 8 Contribution/Balance Fault |
| P0285 | Cylinder 9 Injector Circuit Low |
| P0286 | Cylinder 9 Injector Circuit High |
| P0287 | Cylinder 9 Contribution/Balance Fault |
| P0288 | Cylinder 10 Injector Circuit Low |
| P0289 | Cylinder 10 Injector Circuit High |
| P0290 | Cylinder 10 Contribution/Balance Fault |
| P0291 | Cylinder 11 Injector Circuit Low |
| P0292 | Cylinder 11 Injector Circuit High |
| P0293 | Cylinder 11 Contribution/Balance Fault |
| P0294 | Cylinder 12 Injector Circuit Low |
| P0295 | Cylinder 12 Injector Circuit High |
| P0296 | Cylinder 12 Contribution/Range Fault |
| P0300 | Random/Multiple Cylinder Misfire Detected |
| P0301 | Cylinder 1 Misfire Detected |
| P0302 | Cylinder 2 Misfire Detected |
| P0303 | Cylinder 3 Misfire Detected |
| P0304 | Cylinder 4 Misfire Detected |
| P0305 | Cylinder 5 Misfire Detected |
| P0306 | Cylinder 6 Misfire Detected |
| P0307 | Cylinder 7 Misfire Detected |
| P0308 | Cylinder 8 Misfire Detected |
| P0309 | Cylinder 9 Misfire Detected |
| P0310 | Cylinder 10 Misfire Detected |
| P0311 | Cylinder 11 Misfire Detected |
| P0312 | Cylinder 12 Misfire Detected |
| P0320 | Ignition/Distributor Engine Speed Input Circuit Malfunction |
| P0321 | Ignition/Distributor Engine Speed Input Circuit Range/Performance |
| P0322 | Ignition/Distributor Engine Speed Input Circuit No Signal |
| P0323 | Ignition/Distributor Engine Speed Input Circuit Intermittent |
| P0325 | Knock Sensor 1 Circuit Malfunction (Bank 1 or Single Sensor) |
| P0326 | Knock Sensor 1 Circuit Range/Performance (Bank 1 or Single Sensor) |
| P0327 | Knock Sensor 1 Circuit Low Input (Bank 1 or Single Sensor) |
| P0328 | Knock Sensor 1 Circuit High Input (Bank 1 or Single Sensor) |
| P0329 | Knock Sensor 1 Circuit Intermittent (Bank 1 or Single Sensor) |
| P0330 | Knock Sensor 2 Circuit Malfunction (Bank 2) |
| P0331 | Knock Sensor 2 Circuit Range/Performance (Bank 2) |
| P0332 | Knock Sensor 2 Circuit Low Input (Bank 2) |
| P0333 | Knock Sensor 2 Circuit High Input (Bank 2) |
| P0334 | Knock Sensor 2 Circuit Intermittent (Bank 2) |
| P0335 | Crankshaft Position Sensor A Circuit Malfunction |
| P0336 | Crankshaft Position Sensor A Circuit Range/Performance |
| P0337 | Crankshaft Position Sensor A Circuit Low Input |
| P0338 | Crankshaft Position Sensor A Circuit High Input |
| P0339 | Crankshaft Position Sensor A Circuit Intermittent |
| P0340 | Camshaft Position Sensor Circuit Malfunction |
| P0341 | Camshaft Position Sensor Circuit Range/Performance |
| P0342 | Camshaft Position Sensor Circuit Low Input |
| P0343 | Camshaft Position Sensor Circuit High Input |
| P0344 | Camshaft Position Sensor Circuit Intermittent |
| P0350 | Ignition Coil Primary/Secondary Circuit Malfunction |
| P0351 | Ignition Coil A Primary/Secondary Circuit Malfunction |
| P0352 | Ignition Coil B Primary/Secondary Circuit Malfunction |
| P0353 | Ignition Coil C Primary/Secondary Circuit Malfunction |
| P0354 | Ignition Coil D Primary/Secondary Circuit Malfunction |
| P0355 | Ignition Coil E Primary/Secondary Circuit Malfunction |
| P0356 | Ignition Coil F Primary/Secondary Circuit Malfunction |
| P0357 | Ignition Coil G Primary/Secondary Circuit Malfunction |
| P0358 | Ignition Coil H Primary/Secondary Circuit Malfunction |
| P0359 | Ignition Coil I Primary/Secondary Circuit Malfunction |
| P0360 | Ignition Coil J Primary/Secondary Circuit Malfunction |
| P0361 | Ignition Coil K Primary/Secondary Circuit Malfunction |
| P0362 | Ignition Coil L Primary/Secondary Circuit Malfunction |
| P0370 | Timing Reference High Resolution Signal A Malfunction |
| P0371 | Timing Reference High Resolution Signal A Too Many Pulses |
| P0372 | Timing Reference High Resolution Signal A Too Few Pulses |
| P0373 | Timing Reference High Resolution Signal A Intermittent/Erratic Pulses |
| P0374 | Timing Reference High Resolution Signal A No Pulses |
| P0375 | Timing Reference High Resolution Signal B Malfunction |
| P0376 | Timing Reference High Resolution Signal B Too Many Pulses |
| P0377 | Timing Reference High Resolution Signal B Too Few Pulses |
| P0378 | Timing Reference High Resolution Signal B Intermittent/Erratic Pulses |
| P0379 | Timing Reference High Resolution Signal B No Pulses |
| P0380 | Glow Plug/Heater Circuit "A" Malfunction |
| P0381 | Glow Plug/Heater Indicator Circuit Malfunction |
| P0382 | Exhaust Gas Recirculation Flow Malfunction |
| P0385 | Crankshaft Position Sensor B Circuit Malfunction |
| P0386 | Crankshaft Position Sensor B Circuit Range/Performance |
| P0387 | Crankshaft Position Sensor B Circuit Low Input |
| P0388 | Crankshaft Position Sensor B Circuit High Input |
| P0389 | Crankshaft Position Sensor B Circuit Intermittent |
| P0400 | Exhaust Gas Recirculation Flow Malfunction |
| P0401 | Exhaust Gas Recirculation Flow Insufficient Detected |
| P0402 | Exhaust Gas Recirculation Flow Excessive Detected |
| P0403 | Exhaust Gas Recirculation Circuit Malfunction |
| P0404 | Exhaust Gas Recirculation Circuit Range/Performance |
| P0405 | Exhaust Gas Recirculation Sensor A Circuit Low |
| P0406 | Exhaust Gas Recirculation Sensor A Circuit High |
| P0407 | Exhaust Gas Recirculation Sensor B Circuit Low |
| P0408 | Exhaust Gas Recirculation Sensor B Circuit High |
| P0410 | Secondary Air Injection System Malfunction |
| P0411 | Secondary Air Injection System Incorrect Flow Detected |
| P0412 | Secondary Air Injection System Switching Valve A Circuit Malfunction |
| P0413 | Secondary Air Injection System Switching Valve A Circuit Open |
| P0414 | Secondary Air Injection System Switching Valve A Circuit Shorted |
| P0415 | Secondary Air Injection System Switching Valve B Circuit Malfunction |
| P0416 | Secondary Air Injection System Switching Valve B Circuit Open |
| P0417 | Secondary Air Injection System Switching Valve B Circuit Shorted |
| P0418 | Secondary Air Injection System Relay "A" Circuit Malfunction |
| P0419 | Secondary Air Injection System Relay "B" Circuit Malfunction |
| P0420 | Catalyst System Efficiency Below Threshold (Bank 1) |
| P0421 | Warm Up Catalyst Efficiency Below Threshold (Bank 1) |
| P0422 | Main Catalyst Efficiency Below Threshold (Bank 1) |
| P0423 | Heated Catalyst Efficiency Below Threshold (Bank 1) |
| P0424 | Heated Catalyst Temperature Below Threshold (Bank 1) |
| P0426 | Catalyst Temperature Sensor Range/Performance (Bank 1) |
| P0427 | Catalyst Temperature Sensor Low Input (Bank 1) |
| P0428 | Catalyst Temperature Sensor High Input (Bank 1) |
| P0430 | Catalyst System Efficiency Below Threshold (Bank 2) |
| P0431 | Warm Up Catalyst Efficiency Below Threshold (Bank 2) |
| P0432 | Main Catalyst Efficiency Below Threshold (Bank 2) |
| P0433 | Heated Catalyst Efficiency Below Threshold (Bank 2) |
| P0434 | Heated Catalyst Temperature Below Threshold (Bank 2) |
| P0436 | Catalyst Temperature Sensor Range/Performance (Bank 2) |
| P0437 | Catalyst Temperature Sensor Low Input (Bank 2) |
| P0438 | Catalyst Temperature Sensor High Input (Bank 2) |
| P0440 | Evaporative Emission Control System Malfunction |
| P0441 | Evaporative Emission Control System Incorrect Purge Flow |
| P0442 | Evaporative Emission Control System Leak Detected (small leak) |
| P0443 | Evaporative Emission Control System Purge Control Valve Circuit Malfunction |
| P0444 | Evaporative Emission Control System Purge Control Valve Circuit Open |
| P0445 | Evaporative Emission Control System Purge Control Valve Circuit Shorted |
| P0446 | Evaporative Emission Control System Vent Control Circuit Malfunction |
| P0447 | Evaporative Emission Control System Vent Control Circuit Open |
| P0448 | Evaporative Emission Control System Vent Control Circuit Shorted |
| P0449 | Evaporative Emission Control System Vent Valve/Solenoid Circuit Malfunction |
| P0450 | Evaporative Emission Control System Pressure Sensor Malfunction |
| P0451 | Evaporative Emission Control System Pressure Sensor Range/Performance |
| P0452 | Evaporative Emission Control System Pressure Sensor Low Input |
| P0453 | Evaporative Emission Control System Pressure Sensor High Input |
| P0454 | Evaporative Emission Control System Pressure Sensor Intermittent |
| P0455 | Evaporative Emission Control System Leak Detected (gross leak) |
| P0460 | Fuel Level Sensor Circuit Malfunction |
| P0461 | Fuel Level Sensor Circuit Range/Performance |
| P0462 | Fuel Level Sensor Circuit Low Input |
| P0463 | Fuel Level Sensor Circuit High Input |
| P0464 | Fuel Level Sensor Circuit Intermittent |
| P0465 | Purge Flow Sensor Circuit Malfunction |
| P0466 | Purge Flow Sensor Circuit Range/Performance |
| P0467 | Purge Flow Sensor Circuit Low Input |
| P0468 | Purge Flow Sensor Circuit High Input |
| P0469 | Purge Flow Sensor Circuit Intermittent |
| P0470 | Exhaust Pressure Sensor Malfunction |
| P0471 | Exhaust Pressure Sensor Range/Performance |
| P0472 | Exhaust Pressure Sensor Low |
| P0473 | Exhaust Pressure Sensor High |
| P0474 | Exhaust Pressure Sensor Intermittent |
| P0475 | Exhaust Pressure Control Valve Malfunction |
| P0476 | Exhaust Pressure Control Valve Range/Performance |
| P0477 | Exhaust Pressure Control Valve Low |
| P0478 | Exhaust Pressure Control Valve High |
| P0479 | Exhaust Pressure Control Valve Intermittent |
| P0480 | Cooling Fan 1 Control Circuit Malfunction |
| P0481 | Cooling Fan 2 Control Circuit Malfunction |
| P0482 | Cooling Fan 3 Control Circuit Malfunction |
| P0483 | Cooling Fan Rationality Check Malfunction |
| P0484 | Cooling Fan Circuit Over Current |
| P0485 | Cooling Fan Power/Ground Circuit Malfunction |
| P0500 | Vehicle Speed Sensor Malfunction |
| P0501 | Vehicle Speed Sensor Range/Performance |
| P0502 | Vehicle Speed Sensor Low Input |
| P0503 | Vehicle Speed Sensor Intermittent/Erratic/High |
| P0505 | Idle Control System Malfunction |
| P0506 | Idle Control System RPM Lower Than Expected |
| P0507 | Idle Control System RPM Higher Than Expected |
| P0510 | Closed Throttle Position Switch Malfunction |
| P0520 | Engine Oil Pressure Sensor/Switch Circuit Malfunction |
| P0521 | Engine Oil Pressure Sensor/Switch Circuit Range/Performance |
| P0522 | Engine Oil Pressure Sensor/Switch Circuit Low Voltage |
| P0523 | Engine Oil Pressure Sensor/Switch Circuit High Voltage |
| P0530 | A/C Refrigerant Pressure Sensor Circuit Malfunction |
| P0531 | A/C Refrigerant Pressure Sensor Circuit Range/Performance |
| P0532 | A/C Refrigerant Pressure Sensor Circuit Low Input |
| P0533 | A/C Refrigerant Pressure Sensor Circuit High Input |
| P0534 | Air Conditioner Refrigerant Charge Loss |
| P0550 | Power Steering Pressure Sensor Circuit Malfunction |
| P0551 | Power Steering Pressure Sensor Circuit Range/Performance |
| P0552 | Power Steering Pressure Sensor Circuit Low Input |
| P0553 | Power Steering Pressure Sensor Circuit High Input |
| P0554 | Power Steering Pressure Sensor Circuit Intermittent |
| P0560 | System Voltage Malfunction |
| P0561 | System Voltage Unstable |
| P0562 | System Voltage Low |
| P0562 | A/C pressure sensor low voltage(Chrysler) |
| P0563 | System Voltage High |
| P0565 | Cruise Control On Signal Malfunction |
| P0566 | Cruise Control Off Signal Malfunction |
| P0567 | Cruise Control Resume Signal Malfunction |
| P0568 | Cruise Control Set Signal Malfunction |
| P0569 | Cruise Control Coast Signal Malfunction |
| P0570 | Cruise Control Accel Signal Malfunction |
| P0571 | Cruise Control/Brake Switch A Circuit Malfunction |
| P0572 | Cruise Control/Brake Switch A Circuit Low |
| P0573 | Cruise Control/Brake Switch A Circuit High |
| P0574 | Cruise Control Related Malfunction |
| P0575 | Cruise Control Related Malfunction |
| P0576 | Cruise Control Related Malfunction |
| P0576 | Cruise Control Related Malfunction |
| P0578 | Cruise Control Related Malfunction |
| P0579 | Cruise Control Related Malfunction |
| P0580 | Cruise Control Related Malfunction |
| P0600 | Serial Communication Link Malfunction |
| P0601 | Control Module Read Only Memory(ROM) |
| P0602 | Control Module Programming Error |
| P0603 | Internal Control Module Keep Alive Memory (KAM) Error |
| P0604 | Internal Control Module Random Access Memory (RAM) Error |
| P0605 | Internal Control Module Read Only Memory (ROM) Error |
| P0606 | PCM Processor Fault |
| P0608 | Control Module VSS Output "A" Malfunction |
| P0609 | Control Module VSS Output "B" Malfunction |
| P0620 | Generator Control Circuit Malfunction |
| P0621 | Generator Lamp "L" Control Circuit Malfunction |
| P0622 | Generator Field "F" Control Circuit Malfunction |
| P0650 | Malfunction Indicator Lamp (MIL) Control Circuit Malfunction |
| P0654 | Engine RPM Output Circuit Malfunction |
| P0655 | Engine Hot Lamp Output Control Circuit Malfucntion |
| P0656 | Fuel Level Output Circuit Malfunction |
| P0700 | Transmission Control System Malfunction |
| P0701 | Transmission Control System Range/Performance |
| P0702 | Transmission Control System Electrical |
| P0703 | Torque Converter/Brake Switch B Circuit Malfunction |
| P0704 | Clutch Switch Input Circuit Malfunction |
| P0705 | Transmission Range Sensor Circuit malfunction (PRNDL Input) |
| P0706 | Transmission Range Sensor Circuit Range/Performance |
| P0707 | Transmission Range Sensor Circuit Low Input |
| P0708 | Transmission Range Sensor Circuit High Input |
| P0709 | Transmission Range Sensor Circuit Intermittent |
| P0710 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P0711 | Transmission Fluid Temperature Sensor Circuit Range/Performance |
| P0712 | Transmission Fluid Temperature Sensor Circuit Low Input |
| P0713 | Transmission Fluid Temperature Sensor Circuit High Input |
| P0714 | Transmission Fluid Temperature Sensor Circuit Intermittent |
| P0715 | Input/Turbine Speed Sensor Circuit Malfunction |
| P0716 | Input/Turbine Speed Sensor Circuit Range/Performance |
| P0717 | Input/Turbine Speed Sensor Circuit No Signal |
| P0718 | Input/Turbine Speed Sensor Circuit Intermittent |
| P0719 | Torque Converter/Brake Switch B Circuit Low |
| P0720 | Output Speed Sensor Circuit Malfunction |
| P0721 | Output Speed Sensor Range/Performance |
| P0722 | Output Speed Sensor No Signal |
| P0723 | Output Speed Sensor Intermittent |
| P0724 | Torque Converter/Brake Switch B Circuit High |
| P0725 | Engine Speed input Circuit Malfunction |
| P0726 | Engine Speed Input Circuit Range/Performance |
| P0727 | Engine Speed Input Circuit No Signal |
| P0728 | Engine Speed Input Circuit Intermittent |
| P0730 | Incorrect Gear Ratio |
| P0731 | Gear 1 Incorrect ratio |
| P0732 | Gear 2 Incorrect ratio |
| P0733 | Gear 3 Incorrect ratio |
| P0734 | Gear 4 Incorrect ratio |
| P0735 | Gear 5 Incorrect ratio |
| P0736 | Reverse incorrect gear ratio |
| P0740 | Torque Converter Clutch Circuit Malfuction |
| P0741 | Torque Converter Clutch Circuit Performance or Stuck Off |
| P0742 | Torque Converter Clutch Circuit Stuck On |
| P0743 | Torque Converter Clutch Circuit Electrical |
| P0744 | Torque Converter Clutch Circuit Intermittent |
| P0745 | Pressure Control Solenoid Malfunction |
| P0746 | Pressure Control Solenoid Performance or Stuck Off |
| P0747 | Pressure Control Solenoid Stuck On |
| P0748 | Pressure Control Solenoid Electrical |
| P0749 | Pressure Control Solenoid Intermittent |
| P0750 | Shift Solenoid A Malfunction |
| P0751 | Shift Solenoid A Performance or Stuck Off/ 1-2 Shift Solenoid Valve Performance |
| P0752 | Shift Solenoid A Stuck On |
| P0753 | Shift Solenoid A Electrical/ 1-2 Shift Solenoid Circuit Electrical |
| P0754 | Shift Solenoid A Intermittent |
| P0755 | Shift Solenoid B Malfunction |
| P0756 | Shift Solenoid B Performance or Stuck Off/ 2-3 Shift Solenoid Valve Performance |
| P0757 | Shift Solenoid B Stuck On |
| P0758 | Shift Solenoid B Electrical/ 2-3 Shift Solenoid Circuit Electrical |
| P0759 | Shift Solenoid B Intermittent |
| P0760 | Shift Solenoid C Malfunction |
| P0761 | Shift Solenoid C Performance or Stuck Off |
| P0762 | Shift Solenoid C Stuck On |
| P0763 | Shift Solenoid C Electrical |
| P0764 | Shift Solenoid C Intermittent |
| P0765 | Shift Solenoid D Malfunction |
| P0766 | Shift Solenoid D Performance or Stuck Off |
| P0767 | Shift Solenoid D Stuck On |
| P0768 | Shift Solenoid D Electrical |
| P0769 | Shift Solenoid D Intermittent |
| P0770 | Shift Solenoid E Malfunction |
| P0771 | Shift Solenoid E Performance or Stuck Off |
| P0772 | Shift Solenoid E Stuck On |
| P0773 | Shift Solenoid E Electrical |
| P0774 | Shift Solenoid E Intermittent |
| P0775 | Pressure Control Solenoid B Malfunction |
| P0776 | Pressure Control Solenoid B Performance |
| P0777 | Pressure Control Solenoid B Stuck On |
| P0778 | Pressure Control Solenoid B Electrical |
| P0779 | Pressure Control Solenoid B Intermittent |
| P0780 | Shift Malfunction |
| P0781 | 1-2 Shift Malfunction |
| P0782 | 2-3 Shift Malfunction |
| P0783 | 3-4 Shift Malfunction |
| P0784 | 4-5 Shift Malfunction |
| P0785 | Shift/Timing Solenoid Malfunction/ 3-2 Shift Solenoid Circuit Electrical |
| P0786 | Shift/Timing Solenoid Range/Performance |
| P0787 | Shift/Timing Solenoid Low |
| P0788 | Shift/Timing Solenoid High |
| P0789 | Shift/Timing Solenoid Intermittent |
| P0790 | Normal/Performance Switch Circuit Malfunction |
| P0801 | Reverse Inhibit Control Circuit Malfunction |
| P0803 | 1-4 Upshift (Skip Shift) Solenoid Control Circuit Malfunction |
| P0804 | 1-4 Upshift (Skip Shift) Lamp Control Circuit Malfunction |
| P1100 | MAF Sensor Intermittent/ Check of all OBDII Systems Not Complete |
| P1101 | MAF Sensor Out Of Self Test Range./ KOER Not Able To Complete KOER Aborted |
| P1102 | MAF Sensor In Range But Lower Than Expected |
| P1103 | MAF Sensor In Range But Higher Than Expected |
| P1104 | MAF Ground Malfunction |
| P1105 | Dual Alternator Upper Fault |
| P1106 | Dual Alternator Lower Fault/ Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent High Voltage |
| P1107 | Dual Alternator Lower Circuit Malfunction/ Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent Low Voltage |
| P1108 | Dual Alternator Battery Lamp Circuit Malfunction |
| P1109 | IAT - B Sensor Intermittent |
| P1110 | IAT Sensor (D/C) Open/Short |
| P1111 | Intake Air Temperature (IAT) Sensor Circuit Intermittent High Voltage |
| P1112 | Intake Air Temperature (IAT) Sensor Circuit Intermittent Low Voltage |
| P1113 | IAT Sensor Open/Short |
| P1114 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent Low Voltage/IAT - B Circuit Low Input |
| P1115 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent High Voltage/IAT - B Circuit High Input |
| P1116 | Engine Coolant sensor out of range/ECT Sensor Out Of Self Test Range |
| P1117 | Engine Coolant Sensor intermittent/ECT Sensor Intermittent |
| P1118 | Manifold Absolute Temperature Circuit Low Input |
| P1119 | Manifold Absolute Temperature Circuit High Input |
| P1120 | Throttle position sensor out of range |
| P1121 | Throttle Position (TP) Sensor Circuit Intermittent High Voltage |
| P1122 | Throttle Position (TP) Sensor Circuit Intermittent Low Voltage |
| P1123 | Throttle Position Sensor In Range But Higher Than Expected |
| P1124 | Throttle Position Sensor Out Of Self Test Range |
| P1125 | Throttle position sensor intermittent |
| P1126 | Throttle Position (Narrow Range) Sensor Circuit Malfunction |
| P1127 | Exhaust Not Warm, Downstream O2 Sensor |
| P1128 | Upstream Heated O2 Sensors Swapped |
| P1129 | Downstream Heated O2 Sensors Swapped |
| P1130 | Lack Of HO2S Switch - Adaptive Fuel At Limit |
| P1131 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1132 | Lack Of HO2S Switch - Sensor Indicates Rich |
| P1133 | HO2S Insufficient Switching Sensor 1 |
| P1134 | HO2S Transition Time Ratio Sensor 1 |
| P1135 | Pedal Position Sensor A Circuit Intermittent |
| P1136 | Fan Control Circuit Malfunction |
| P1137 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1138 | Lack Of HO2S12 Switch - Sensor Indicates Rich |
| P1139 | Water In Fuel Indicator Circuit Malfunction |
| P1140 | Water In Fuel Condition |
| P1141 | Fuel Restriction Indicator Circuit Malfunction |
| P1142 | Fuel Restriction Condition |
| P1143 | Air Assist Control Valve Range/Performance |
| P1144 | Air Assist Control Valve Circuit Malfunction |
| P1150 | Lack Of HO2S21 Switch - Adaptive Fuel At Limit |
| P1151 | Lack Of HO2S21 Switch - Sensor Indicates Lean |
| P1152 | Lack Of HO2S21 Switch - Sensor Indicates Rich |
| P1153 | Bank 2 Fuel Control Shifted Lean |
| P1154 | Bank 2 Fuel Control Shifted Rich |
| P1155 | Alternative Fuel Controller |
| P1156 | Fuel Select Switch Malfunction |
| P1157 | Lack Of HO2S22 Switch - Sensor Indicates Lean |
| P1158 | Lack Of HO2S22 Switch - Sensor Indicates Rich |
| P1159 | Fuel Stepper Motor Malfunction |
| P1167 | Invalid Test,throttle not depressed |
| P1168 | Fuel Rail Sensor In-Range Low Failure |
| P1169 | Fuel Rail Sensor In-Range High Failure |
| P1170 | ESO - Engine Shut Off Solenoid Fault |
| P1171 | Rotor Sensor Fault |
| P1172 | Rotor Control Fault |
| P1173 | Rotor Calibration Fault |
| P1174 | Cam Sensor Fault |
| P1175 | Cam Control Fault |
| P1176 | Cam Calibration Fault |
| P1177 | Synchronization Fault |
| P1178 | ( open ) |
| P1180 | Fuel Delivery System Malfunction - Low |
| P1181 | Fuel Delivery System Malfunction - High |
| P1182 | Fuel Shut Off Solenoid Malfunction |
| P1183 | Engine Oil Temperature Circuit Malfunction |
| P1184 | Engine Oil Temperature Out Of Self Test Range |
| P1185 | FTS High - Fuel Pump Temperature Sensor High |
| P1186 | FTS Low - Fuel Pump Temperature Sensor Low |
| P1187 | Variant Selection |
| P1188 | Calibration Memory Fault |
| P1189 | Pump Speed Signal Fault |
| P1190 | Calibration Resistor Out Of Range |
| P1191 | Key Line Voltage |
| P1192 | Voltage External |
| P1193 | EGR Drive Overcurrent |
| P1194 | ECU A/D Converter |
| P1195 | SCP HBCC Failed To Initialize |
| P1196 | Key Off Voltage High |
| P1197 | Key Off Voltage Low |
| P1198 | Pump Rotor Control Underfueling |
| P1199 | Fuel Level Input Circuit Low |
| P1200 | Injector Control Circuit |
| P1201 | Injector Circuit Open / Shorted - Cylinder #1 |
| P1202 | Injector Circuit Open / Shorted - Cylinder #2 |
| P1203 | Injector Circuit Open / Shorted - Cylinder #3 |
| P1204 | Injector Circuit Open / Shorted - Cylinder #4 |
| P1205 | Injector Circuit Open / Shorted - Cylinder #5 |
| P1206 | Injector Circuit Open / Shorted - Cylinder #6 |
| P1209 | Injector Control Pressure System Fault |
| P1210 | Injector Control Pressure Above Expected Level |
| P1211 | Injector Control Pressure Sensor Above / Below Desired |
| P1212 | Injector Control Pressure Not Detected During Crank |
| P1213 | Start Injector Circuit Malfunction |
| P1214 | Pedal Position Sensor B Circuit Intermittent |
| P1215 | Pedal Position Sensor C Circuit Low Input |
| P1216 | Pedal Position Sensor C Circuit High Input |
| P1217 | Pedal Position Sensor C Circuit Intermittent |
| P1218 | CID High |
| P1219 | CID Low |
| P1220 | Series Throttle Control System Malfunction |
| P1221 | Traction Control System Malfunction |
| P1222 | Traction Control Output Circuit Malfunction |
| P1223 | Pedal Demand Sensor B Circuit High Input |
| P1224 | Throttle Position Sensor B Out Of Self Test Range |
| P1225 | Needle Lift Sensor Malfunction |
| P1226 | Control Sleeve Sensor Malfunction |
| P1227 | Wastegate Failed Closed (Over Pressure) |
| P1228 | Wastegate Failed Open (Under Pressure) |
| P1229 | Intercooler Pump Driver Fault |
| P1230 | Fuel Pump Low Speed Malfunction |
| P1231 | Fuel Pump Secondary Circuit Low, High Speed |
| P1232 | Fuel Pump Speed Primary Circuit Malfunction |
| P1233 | Fuel Pump Driver Module Off Line |
| P1234 | Fuel Pump Driver Module Off Line |
| P1235 | Fuel Pump Control Out Of Range |
| P1236 | Fuel Pump Control Out Of Range |
| P1237 | Fuel Pump Secondary Circuit Malfunction |
| P1238 | Fuel Pump Secondary Circuit Malfunction |
| P1239 | Speed Fuel Pump Positive Feed Fault |
| P1240 | Sensor Power Supply Malfunction |
| P1241 | Sensor Power Supply Low Input |
| P1242 | Sensor Power Supply High Input |
| P1243 | Second Fuel Pump Faulty or Ground Fault |
| P1244 | Alternator Load Input Failed High |
| P1245 | Alternator Load Input Failed Low |
| P1246 | Alternator Load Input Failed |
| P1247 | Turbo Boost Pressure Low |
| P1248 | Turbo Boost Pressure Not Detected |
| P1249 | Wastegate Control Valve Performance |
| P1250 | PRC Solenoid Circuit Malfunction |
| P1251 | Air Mixture Solenoid Circuit Malfunction |
| P1252 | Pedal Correlation PDS1 and LPDS High |
| P1253 | Pedal Correlation PDS1 and LPDS Low |
| P1254 | Pedal Correlation PDS2 and LPDS High |
| P1255 | Pedal Correlation PDS2 and LPDS Low |
| P1256 | Pedal Correlation PDS1 and HPDS |
| P1257 | Pedal Correlation PDS2 and HPDS |
| P1258 | Pedal Correlation PDS1 and PDS2 |
| P1259 | Immobilizer to PCM Signal Error |
| P1260 | THEFT Detected, Vehicle Immobilzed |
| P1261 | Cylinder #1 High To Low Side Short |
| P1262 | Cylinder #2 High To Low Side Short |
| P1263 | Cylinder #3 High To Low Side Short |
| P1264 | Cylinder #4 High To Low Side Short |
| P1265 | Cylinder #5 High To Low Side Short |
| P1266 | Cylinder #6 High To Low Side Short |
| P1267 | Cylinder #7 High To Low Side Short |
| P1268 | Cylinder #8 High To Low Side Short |
| P1269 | Immobilizer Code Not Programmed |
| P1270 | Engine RPM Or Speed Limiter Reached |
| P1271 | Cylinder #1 High To Low Side Open |
| P1272 | Cylinder #2 High To Low Side Open |
| P1273 | Cylinder #3 High To Low Side Open |
| P1274 | Cylinder #4 High To Low Side Open |
| P1275 | Cylinder #5 High To Low Side Open |
| P1276 | Cylinder #6 High To Low Side Open |
| P1277 | Cylinder #7 High To Low Side Open |
| P1278 | Cylinder #8 High To Low Side Open |
| P1280 | Injection Control Pressure Out Of Range Low |
| P1281 | Injection Control Pressure Out Of Range High |
| P1282 | Excessive Injection Control Pressure |
| P1283 | IPR Circuit Failure |
| P1284 | Aborted KOER - ICP Failure |
| P1285 | Cylinder head over temp sensed |
| P1286 | Fuel Pulse In Range But Lower Than Expected |
| P1287 | Fuel Pulse In Range But Higher Than Expected |
| P1288 | Cylinder Head Temp Sensor Out Of Self Test Range |
| P1289 | Cylinder Head Temp Sensor High Input |
| P1290 | Cylinder Head Temp Sensor Low Input |
| P1291 | Injector High Side Short To GND Or VBATT - Bank 1 |
| P1292 | Injector High Side Short To GND Or VBATT - Bank 2 |
| P1293 | Injector High Side Open - Bank 1 |
| P1294 | Injector High Side Open - Bank 2/Target idle not reached |
| P1295 | Multi-faults - Bank 1 - With Low Side Shorts |
| P1296 | Multi-faults - Bank 2 - With Low Side Shorts |
| P1297 | Injector High Sides Shorted Together |
| P1298 | IDM Failure |
| P1299 | Cylinder Head Overtemperature Protection Active |
| P1300 | Boost Calibration Fault |
| P1301 | Boost Calibration High |
| P1302 | Boost Calibration Low |
| P1303 | EGR Calibration Fault |
| P1304 | EGR Calibration High |
| P1305 | EGR Calibration Low |
| P1306 | Kickdown Relay Pull - In Circuit Fault |
| P1307 | Kickdown Relay Hold Circuit Fault |
| P1308 | A/C Clutch Circuit Fault |
| P1309 | Misfire Monitor AICE Chip Fault |
| P1313 | Misfire Rate Catalyst Damage Fault - Bank 1 |
| P1314 | Misfire Rate Catalyst Damage Fault - Bank 2 |
| P1315 | Persistent Misfire |
| P1316 | Injector Circuit / IDM Codes Detected |
| P1317 | Injector Circuit / IDM Codes Not Updated |
| P1336 | Crank / Cam Sensor Range / Performance |
| P1340 | Camshaft Position Sensor B Circuit Malfunction |
| P1341 | Camshaft Position Sensor B Range / Performance |
| P1345 | SGC (Cam Position) Sensor Circuit Malfunction/ Crankshaft Position - Camshaft Position Correlation |
| P1346 | Fuel Level Sensor B Circuit Malfunction |
| P1347 | Fuel Level Sensor B Range / Performance |
| P1348 | Fuel Level Sensor B Circuit Low |
| P1349 | Fuel Level Sensor B Circuit High |
| P1350 | Fuel Level Sensor B Intermittent/Bypass Line Monitor |
| P1351 | IDM Input Circuit Malfunction/ Ignition Coil Control Circuit High Voltage |
| P1352 | Ignition Coil A Primary Circuit Malfunction |
| P1353 | Ignition Coil B Primary Circuit Malfunction |
| P1354 | Ignition Coil C Primary Circuit Malfunction |
| P1355 | Ignition Coil D Primary Circuit Malfunction |
| P1360 | Ignition Coil A Secondary Circuit Malfunction |
| P1361 | Ignition Control (IC) Circuit Low Voltage |
| P1362 | Ignition Coil C Secondary Circuit Malfunction |
| P1363 | Ignition Coil D Secondary Circuit Malfunction |
| P1364 | Ignition Coil Primary Circuit Failure |
| P1365 | Ignition Coil Secondary Circuit Failure |
| P1366 | Ignition Spare |
| P1367 | Ignition Spare |
| P1368 | Ignition Spare |
| P1369 | Engine Temperature Light Monitor Failure |
| P1370 | Insufficient RMP Increase During Spark Test |
| P1371 | Ignition Coil - Cylinder 1 - Early Activation Fault |
| P1372 | Ignition Coil - Cylinder 2 - Early Activation Fault |
| P1373 | Ignition Coil - Cylinder 3 - Early Activation Fault |
| P1374 | Crankshaft Position (CKP)/Ignition Coil - Cylinder 4 - Early Activation Fault |
| P1375 | Ignition Coil - Cylinder 5 - Early Activation Fault |
| P1376 | Ignition Coil - Cylinder 6 - Early Activation Fault |
| P1380 | Misfire Detected - Rough Road Data Not Available |
| P1381 | Variable Cam Timing Overadvanced (Bank #1)/ Misfire Detected - No Communication with BCM |
| P1382 | Variable Cam Timing Solenoid #1 Circuit Malfunction |
| P1383 | Variable Cam Timing Overretarded (Bank #1) |
| P1384 | VVT Solenoid A Malfunction |
| P1385 | Variable Cam Timing Solenoid B Malfunction |
| P1386 | Variable Cam Timing Overadvanced (Bank #2) |
| P1387 | Variable Cam Timing Solenoid #2 Circuit Malfunction |
| P1388 | Variable Cam Timing Overretarded (Bank #2) |
| P1389 | Glow Plug Circuit High Side Low Input |
| P1390 | Octane Adjust Pin Out Of Self Test Range |
| P1391 | Glow Plug Circuit Low Input (Bank #1) |
| P1392 | Glow Plug Circuit High Input (Bank #1) |
| P1393 | Glow Plug Circuit Low Input (Bank #2) |
| P1394 | Glow Plug Circuit High Input (Bank #2) |
| P1395 | Glow Plug Monitor Fault (Bank #1) |
| P1396 | Glow Plug Monitor Fault (Bank #2) |
| P1397 | System Voltage Out Of Self Test Range |
| P1398 | VVT Solenoid B Circuit High Input |
| P1399 | Glow Plug Circuit High Side, High Input |
| P1400 | DPFE Circuit Low Input |
| P1401 | DPFE Circuit High Input |
| P1402 | EGR Metering Orifice Restricted |
| P1403 | DPFE Sensor Hoses Reversed |
| P1404 | IAT - B Circuit Malfunction/ Exhaust Gas Recirculation Closed Position Performance |
| P1405 | DPFE Sensor Upstream Hose Off Or Plugged |
| P1406 | Exhaust Gas Recirculation (EGR) Position Sensor Performance |
| P1407 | EGR No Flow Detected |
| P1408 | EGR Flow Out Of Self Test Range |
| P1409 | EVR Control Circuit Malfunction |
| P1411 | SAI System Incorrect Downstream Flow Detected |
| P1413 | SAI System Monitor Circuit Low Input |
| P1414 | SAI System Monitor Circuit High Input |
| P1415 | Air Pump Circuit Malfunction/ (AIR) System Bank 1 |
| P1416 | Port Air Circuit Malfunction/ (AIR) System Bank 2 |
| P1417 | Port Air Relief Circuit Malfunction |
| P1418 | Split Air #1 Circuit Malfunction |
| P1419 | Split Air #2 Circuit Malfunction |
| P1420 | Catalyst Temperature Sensor Failure |
| P1421 | Catalyst Damage |
| P1422 | EGI Temperature Sensor Failure |
| P1423 | EGI Functionality Test Failed |
| P1424 | EGI Glow Plug Primary Failure |
| P1425 | EGI Glow Plug Secondary Failure |
| P1426 | EGI Mini - MAF Failed Out Of Range |
| P1427 | EGI Mini - MAF Failed Short Circuit |
| P1428 | EGI Mini - MAF Failed Open Circuit |
| P1429 | Electric Air Pump Primary Failure |
| P1430 | Electric Air Pump Secondary Failure |
| P1433 | A/C Refrigerant Temperature Circuit Low |
| P1434 | A/C Refrigerant Temperature Circuit High |
| P1435 | A/C Refrigerant Temperature Circuit Range/Performance |
| P1436 | A/C Evaporator Air Temperature Circuit Low |
| P1437 | A/C Evaporator Air Temperature Circuit High |
| P1438 | A/C Evaporator Air Temperature Circuit Range/Performance |
| P1439 | Floor Temperature Switch Circuit Malfunction |
| P1440 | Purge Valve Stuck Open |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Chevrolet Only |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Oldsmobile Only |
| P1442 | Evaporative Emission Control System Leak Detected |
| P1443 | Evaporative Emission Control System Control Valve |
| P1444 | Purge Flow Sensor Circuit Low Input |
| P1445 | Purge Flow Sensor Circuit High Input |
| P1446 | Evaporative Vac Solenoid Circuit Malfunction |
| P1447 | ELC System Closure Valve Flow Fault |
| P1448 | ELC System 2 Fault |
| P1449 | Evaporative Check Solenoid Circuit Malfunction |
| P1450 | Unable To Bleed Up Fuel Tank Vacuum |
| P1451 | Evap Emission Control Sys Vent Control Valve Circuit |
| P1452 | Unable To Bleed - Up Vacuum in Tank |
| P1453 | Fuel Tank Pressure Relief Valve Malfunction |
| P1454 | Evaporative System Vacuum Test Malfunction |
| P1455 | Evap Emission Control Sys Leak Detected (Gross Leak/No Flow) |
| P1456 | Fuel Tank Temperature Sensor Circuit Malfunction |
| P1457 | Unable To Pull Vacuum In Tank |
| P1460 | Wide open throttle A/C cutoff relay circuit |
| P1461 | A/C pressure sensor circuit voltage low |
| P1462 | A/C pressure sensor circuit voltage high |
| P1463 | A/C Pressure Sensor Insufficient Pressure Change |
| P1464 | A/C Demand Out of Self Test Range |
| P1465 | A/C Relay Circuit Malfunction |
| P1466 | A/C Refrigerant Temperature Sensor/Circuit Malfunction |
| P1467 | A/C Compressor Temperature Sensor Malfunction |
| P1468 | SSPOD Open Circuit or Closed Circuit Fault |
| P1469 | Low A/C Cycling Period |
| P1470 | A/C Cycling Period Too Short |
| P1471 | Electrodrive Fan 1 Operational Failure (Driver Side) |
| P1472 | Electrodrive Fan 2 Operational Failure (Passenger Side) |
| P1473 | Fan Secondary High With Fan(s) Off |
| P1474 | Low Fan Control Primary Circuit Malfunction |
| P1475 | Fan Relay (Low) Circuit Malfunction |
| P1476 | Fan Relay (High) Circuit Malfunction |
| P1477 | Additional Fan Relay Circuit Malfunction |
| P1478 | Cooling Fan Driver Fault |
| P1479 | High Fan Control Primary Circuit Malfunction |
| P1480 | Fan Secondary Low with Low Fan On |
| P1481 | Fan Secondary Low With High Fan On |
| P1482 | SCP |
| P1483 | Power To Fan Circuit Overcurrent |
| P1484 | Open Power To Ground VCRM |
| P1485 | EGRV Circuit Malfunction |
| P1486 | EGRA Circuit Malfunction |
| P1487 | EGRCHK Solenoid Circuit Malfunction |
| P1490 | Secondary Air Relief Solenoid Circuit Malfunction |
| P1491 | Secondary Switch Solenoid Circuit Malfunction |
| P1492 | APLSOL Solenoid Circuit Malfunction |
| P1493 | RCNT Solenoid Circuit Malfunction |
| P1494 | SPCUT Solenoid Circuit Malfunction |
| P1495 | TCSPL Solenoid Circuit Malfunction |
| P1500 | Vehicle Speed Sensor Intermittent |
| P1501 | Vehicle Speed Sensor Out Of Self Test Range |
| P1502 | Vehicle Speed Sensor Intermittent Malfunction |
| P1503 | Auxillary Speed Sensor Fault |
| P1504 | Idle Air Control Circuit Malfunction |
| P1505 | Idle Air Control System At Adaptive Clip |
| P1506 | Idle Air Control Overspeed Error |
| P1507 | Idle Air Control Underspeed Error |
| P1508 | Idle Control System Circuit Open |
| P1509 | Idle Control System Circuit Shorted |
| P1510 | Idle Signal Circuit Malfunction |
| P1511 | Idle Switch (Electric Control Throttle) Circuit Malfunction |
| P1512 | Intake Manifold Runner Control (Bank 1) Stuck Closed |
| P1513 | Intake Manifold Runner Control (Bank 2) Stuck Closed |
| P1514 | High Load Neutral/Drive Fault |
| P1515 | Electric Current Circuit Malfunction |
| P1516 | IMRC Input Error (Bank 1) |
| P1517 | IMRC Input Error (Bank 2) |
| P1518 | Intake Manifold Runner Control (Stuck Open) |
| P1519 | Intake Manifold Runner Control (Stuck Closed) |
| P1520 | Intake Manifold Runner Control Circuit Malfunction |
| P1521 | Variable Intake Solenoid #1 Circuit Malfunction |
| P1522 | Variable Intake Solenoid #2 Circuit Malfunction |
| P1523 | IVC Solenoid Circuit Malfunction |
| P1524 | Variable Intake Solenoid System |
| P1525 | Air Bypass Valve System |
| P1526 | Air Bypass System |
| P1527 | Accelerate Warmup Solenoid Circuit Malfunction |
| P1528 | Subsidiary Throttle Valve Solenoid Circuit Malfunction |
| P1529 | SCAIR Solenoid Circuit Malfunction |
| P1530 | A/C Clutch Circuit Malfunction |
| P1531 | Invalid Test - Accelerator Pedal Movement |
| P1532 | IMCC Circuit Malfunction, Bank B |
| P1533 | AAI Circuit Malfunction |
| P1534 | Inertia Switch Activated |
| P1535 | Blower Fan Speed Circuit Range/Performance |
| P1536 | Parking Brake Switch Circuit Failure |
| P1537 | Intake Manifold Runner Control (Bank 1) Stuck Open |
| P1538 | Intake Manifold Runner Control (Bank 2) Stuck Open |
| P1539 | Power To A/C Clutch Circuit Overcurrent |
| P1540 | Air Bypass Valve Circuit Malfunction |
| P1549 | IMCC Circuit Malfunction, Bank B |
| P1550 | PSPS Out Of Self Test Range |
| P1565 | Speed Control Command Switch Out of Range High |
| P1566 | Speed Control Command Switch Out of Range Low |
| P1567 | Speed Control Output Circuit Continuity |
| P1568 | Speed Control Unable to Hold Speed |
| P1571 | Brake Switch Malfunction |
| P1572 | Brake Pedal Switch Circuit Malfunction |
| P1573 | Throttle Position Not Available |
| P1574 | Throttle Position Sensor Disagreement btwn Sensors |
| P1575 | Pedal Position Out of Self Test Range |
| P1576 | Pedal Position Not Available |
| P1577 | Pedal Position Sensor Disagreement btwn Sensors |
| P1578 | ETC Power Less Than Demand |
| P1579 | ETC In Power Limiting Mode |
| P1580 | Electronic Throttle Monitor PCM Override |
| P1581 | Electronic Throttle Monitor Malfunction |
| P1582 | Electronic Throttle Monitor Data Available |
| P1583 | Electronic Throttle Monitor Cruise Disable |
| P1584 | TCU Detected IPE Circuit Malfunction |
| P1585 | Throttle Control Unit Malfunction |
| P1586 | Throttle Control Unit Throttle Position Malfunction |
| P1587 | Throttle Control Unit Modulated Command Malfunction |
| P1588 | Throttle Control Unit Detected Loss of Return Spring |
| P1589 | TCU Unable To Control Desired Throttle Angle |
| P1600 | Loss of KAM Power; Open Circuit |
| P1601 | ECM/TCM Serial Communication Error |
| P1602 | Immobilizer/ECM Communication Error |
| P1603 | EEPROM Malfunction |
| P1604 | Code Word Unregestered |
| P1605 | Keep Alive Memory Test Failure |
| P1606 | ECM Control Relay O/P Circuit Malfunction |
| P1607 | MIL O/P Circuit Malfunction |
| P1608 | Internal ECM Malfunction |
| P1609 | Diagnostic Lamp Driver Fault |
| P1610 | SBDS Interactive Codes |
| P1611 | SBDS Interactive Codes |
| P1612 | SBDS Interactive Codes |
| P1613 | SBDS Interactive Codes |
| P1614 | SBDS Interactive Codes |
| P1615 | SBDS Interactive Codes |
| P1616 | SBDS Interactive Codes |
| P1617 | SBDS Interactive Codes |
| P1618 | SBDS Interactive Codes |
| P1619 | SBDS Interactive Codes |
| P1620 | SBDS Interactive Codes |
| P1621 | Control Module Long Term Memory Performance/ Immobilizer Code Words Do Not Match |
| P1622 | Immobilizer ID Does Not Match |
| P1623 | Immobilizer Code Word/ID Number Write Failure |
| P1624 | Anti Theft System |
| P1625 | B+ Supply To VCRM Fan Circuit Malfunction |
| P1626 | Theft Deterrent Fuel Enable Signal Not Received/ B+ Supply To VCRM A/C Circuit Malfunction |
| P1627 | Module Supply Voltage Out Of Range |
| P1628 | Module Ignition Supply Input Malfunction |
| P1629 | Internal Voltage Regulator Malfunction |
| P1630 | Internal Vref Malfunction |
| P1631 | Theft Deterrent Start Enable Signal Not Correct/ Main Relay Malfunction (Power Hold) |
| P1632 | Smart Alternator Faults Sensor/Circuit Malfunction |
| P1633 | KAM Voltage Too Low |
| P1634 | Data Output Link Circuit Failure |
| P1635 | Tire / Axle Ratio Out of Acceptable Range |
| P1636 | Inductive Signature Chip Communication Error |
| P1637 | Can Link ECM/ABSCM Circuit / Network Malfunction |
| P1638 | Can Link ECM/INSTM Circuit / Network Malfunction |
| P1639 | Vehicle ID Block Corrupted or Not Programmed |
| P1640 | Powertrain DTCs Available in Another Module |
| P1641 | Fuel Pump Primary Circuit Failure |
| P1642 | Fuel Pump Monitor Circuit High Input |
| P1643 | Fuel Pump Monitor Circuit Low Input |
| P1644 | Fuel Pump Speed Control Circuit Malfunction |
| P1645 | Fuel Pump Resistor Switch Circuit Malfunction |
| P1650 | PSP Switch Out of Self Test Range |
| P1651 | PSP Switch Input Malfunction |
| P1652 | IAC Monitor Disabled by PSP Switch Failed On |
| P1653 | Power Steering Output Circuit Malfunction |
| P1654 | Recirculation Override Circuit Malfunction |
| P1655 | Starter Disable Circuit Malfunction |
| P1660 | Output Circuit Check Signal High |
| P1661 | Output Circuit Check Signal Low |
| P1662 | IDM_EN Circuit Failure |
| P1663 | Fuel Demand Command Signal Circuit Malfunction |
| P1667 | CI Circuit Malfunction |
| P1668 | PCM - IDM Communications Error |
| P1670 | Electronic Feedback Signal Not Detected |
| P1680 | Metering Oil Pump Malfunction |
| P1681 | Metering Oil Pump Malfunction |
| P1682 | Metering Oil Pump Malfunction |
| P1683 | Metering Oil Pump Temperature Sensor Circuit Malfunction |
| P1684 | Metering Oil Pump Position Sensor Circuit Malfunction |
| P1685 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1686 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1687 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1688 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1689 | Oil Pressure Control Solenoid Circuit Malfunction |
| P1690 | Wastegate Solenoid Circuit Malfunction |
| P1691 | Turbo Pressure Control Solenoid Circuit Malfunction |
| P1692 | Turbo Control Solenoid Circuit Malfunction |
| P1693 | Turbo Charge Control Circuit Malfunction |
| P1694 | Turbo Charge Relief Circuit Malfunction |
| P1700 | Transmission Indeterminate Failure (Failed to Neutral) |
| P1701 | Reverse Engagement Error |
| P1702 | TRS Circuit Intermittent Malfunction |
| P1703 | Brake Switch Out Of Self Test Range |
| P1704 | Digital TRS Failed to Transition States in KOEO / KOER |
| P1705 | Not in P or N During KOEO / KOER |
| P1706 | High Vehicle Speed Observed in Park |
| P1707 | Transfer Case Neutral Indicator Hard Fault Present |
| P1708 | Clutch Switch Circuit Malfunction |
| P1709 | PNP Switch Out Of Self Test Range |
| P1711 | TFT Sensor Out Of Self Test Range |
| P1712 | Trans Torque Reduction Request Signal Malfunction |
| P1713 | TFT Sensor In Range Failure Low Value |
| P1714 | SSA Inductive Signature Malfunction |
| P1715 | SSB Inductive Signature Malfunction |
| P1716 | SSC Inductive Signature Malfunction |
| P1717 | SSD Inductive Signature Malfunction |
| P1718 | TFT Sensor In Range Failure High |
| P1720 | Vehicle Speed (Meter) Circuit Malfunction |
| P1721 | Gear 1 Incorrect Ratio |
| P1722 | Gear 2 Incorrect Ratio |
| P1723 | Gear 3 incorrect Ratio |
| P1724 | Gear 4 Incorrect Ratio |
| P1725 | Insufficient Engine Speed Increase During Self Test |
| P1726 | Insufficient Engine Speed Decrease During Self Test |
| P1727 | Coast Clutch Solenoid Inductive Signature Malfunction |
| P1728 | Transmission Slip Error |
| P1729 | 4x4 Low Switch Error |
| P1730 | Gear Control Malfunction 2,3,5 |
| P1731 | 1-2 Shift Malfunction |
| P1732 | 2-3 Shift Malfunction |
| P1733 | 3-4 Shift Malfunction |
| P1734 | Gear Control Malfunction |
| P1735 | First Gear Switch Circuit Malfunction |
| P1736 | Second Gear Switch Circuit Malfunction |
| P1737 | Lockup Solenoid System |
| P1738 | Shift Time Error |
| P1739 | Slip Solenoid System |
| P1740 | Torque Converter Clutch Inductive Signature Malfunction |
| P1741 | Torque Converter Clutch Control Error |
| P1742 | Torque Converter Clutch Solenoid Failed On |
| P1743 | Torque Converter Clutch Solenoid Failied On |
| P1744 | Torque Converter Clutch System Performance |
| P1745 | Line Pressure Solenoid System |
| P1746 | Pressure Control Solenoid "A" Open Circuit |
| P1747 | Pressure Control Solenoid "A" Short Circuit |
| P1748 | EPC Malfunction |
| P1749 | Pressure Control Solenoid Failed Low |
| P1751 | Shift Solenoid A Performance |
| P1754 | Coast Clutch Solenoid Circuit Malfunction |
| P1755 | Intermediate Speed Sensor (ISS) Malfunction |
| P1756 | Shift Solenoid B Performance |
| P1760 | Pressure Control Solenoid "A" Short Circuit |
| P1761 | Shift Solenoid C Performance |
| P1762 | Overdrive Band Failed Off |
| P1765 | Timing Solenoid Circuit Malfunction |
| P1767 | Torque Converter Clutch Circuit Malfunction |
| P1768 | Performance / Normal / Winter Mode Input Malfunction |
| P1769 | AG4 Transmission Torque Modulation Fault |
| P1770 | Clutch Solenoid Circuit Malfunction |
| P1775 | Transmission System MIL Fault |
| P1776 | Ignition Retard Request Duration Fault |
| P1777 | Ignition Retard Request Circuit Fault |
| P1778 | Transmission Reverse I/P Circuit Malfunction |
| P1779 | TCIL Circuit Malfunction |
| P1780 | Trans Control Switch (O/D Cancel) Out of Self Test Range |
| P1781 | 4X4 Switch Out of Self Test Range |
| P1782 | P/ES Circuit Out Of Self Test Range |
| P1783 | Transmission Overtemperature Condition |
| P1784 | Transmission Mechanical Failure - First And Reverse |
| P1785 | Transmission Mechanical Failure - First And Second |
| P1786 | 3-2 Downshift Error |
| P1787 | 2-1 Downshift Error |
| P1788 | Pressure Control Solenoid "B" Open Circuit |
| P1789 | Pressure Control Solenoid "B" Short Circuit |
| P1790 | TP (Mechanical) Circuit Malfunction |
| P1791 | TP (Electric) Circuit Malfunction |
| P1792 | Barometer Pressure Circuit Malfunction |
| P1793 | Intake Air Volume Circuit Malfunction |
| P1794 | Battery Voltage Circuit Malfunction |
| P1795 | Idle Switch Circuit Malfunction |
| P1796 | Kick Down Switch Circuit Malfunction |
| P1797 | Neutral Switch Circuit Malfunction |
| P1798 | Coolant Temperature Circuit Malfunction |
| P1799 | Hold Switch Circuit Malfunction |
| P1800 | Transmission Clutch Interlock Safety Switch Circuit Failure |
| P1801 | Transmission Clutch Interlock Safety Switch Open Circuit |
| P1802 | Transmission Clutch Interlock Safety Switch Short Circuit To Battery |
| P1803 | Transmission Clutch Interlock Safety Switch Short Circuit To Ground |
| P1804 | Transmission 4-Wheel Drive High Indicator Circuit Failure |
| P1805 | Transmission 4-Wheel Drive High Indicator Open Circuit |
| P1806 | Transmission 4-Wheel Drive High Indicator Short Circuit To Battery |
| P1807 | Transmission 4-Wheel Drive High Indicator Short Circuit To Ground |
| P1808 | Transmission 4-Wheel Drive Low Indicator Circuit Failure |
| P1809 | Transmission 4-Wheel Drive Low Indicator Open Circuit |
| P1810 | TFP Valve Position Switch Circuit/ Transmission 4-Wheel Drive Low Indicator Short Circuit To Battery |
| P1811 | Transmission 4-Wheel Drive Low Indicator Short Circuit To Ground |
| P1812 | Transmission 4-Wheel Drive Mode Select Circuit Failure |
| P1813 | Transmission 4-Wheel Drive Mode Select Open Circuit |
| P1814 | Transmission 4-Wheel Drive Mode Select Short Circuit To Battery |
| P1815 | Transmission 4-Wheel Drive Mode Select Short Circuit To Ground |
| P1816 | Transmission Neutral Safety Switch Circuit Failure |
| P1817 | Transmission Neutral Safety Switch Open Circuit |
| P1818 | Transmission Neutral Safety Switch Short Circuit To Battery |
| P1819 | Transmission Neutral Safety Switch Short Circuit To Ground |
| P1820 | Transmission Transfer Case Clockwise Shift Relay Coil Circuit Failure |
| P1821 | Transmission Transfer Case Clockwise Shift Relay Coil Open Circuit |
| P1822 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Battery |
| P1823 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Ground |
| P1824 | Transmission 4-Wheel Drive Clutch Relay Circuit Failure |
| P1825 | Transmission 4-Wheel Drive Clutch Relay Open Circuit |
| P1826 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Battery |
| P1827 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Ground |
| P1828 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Circuit Failure |
| P1829 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Open Circuit |
| P1830 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Battery |
| P1831 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Ground |
| P1832 | Transmission Transfer Case Differential Lock-Up Solenoid Circuit Failure |
| P1833 | Transmission Transfer Case Differential Lock-Up Solenoid Open Circuit |
| P1834 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Battery |
| P1835 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Ground |
| P1836 | Transmission Transfer Case Front Shaft Speed Sensor Circuit Failure |
| P1837 | Transmission Transfer Case Rear Shaft Speed Sensor Circuit Failure |
| P1838 | Transmission Transfer Case Shift Motor Circuit Failure |
| P1839 | Transmission Transfer Case Shift Motor Open Circuit |
| P1840 | Transmission Transfer Case Shift Motor Short Circuit To Battery |
| P1841 | Transmission Transfer Case Shift Motor Short Circuit To Ground |
| P1842 | Transmission Transfer Case Differential Lock-Up Feedback Switch Circuit Failure |
| P1843 | Transmission Transfer Case Differential Lock-Up Feedback Switch Open Circuit |
| P1844 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Battery |
| P1845 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Ground |
| P1846 | Transmission Transfer Case Contact Plate 'A' Circuit Failure |
| P1847 | Transmission Transfer Case Contact Plate 'A' Open Circuit |
| P1848 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Battery |
| P1849 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Ground |
| P1850 | Transmission Transfer Case Contact Plate 'B' Circuit Failure |
| P1851 | Transmission Transfer Case Contact Plate 'B' Open Circuit |
| P1852 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Battery |
| P1853 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Ground |
| P1854 | Transmission Transfer Case Contact Plate 'C' Circuit Failure |
| P1855 | Transmission Transfer Case Contact Plate 'C' Open Circuit |
| P1856 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Battery |
| P1857 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Ground |
| P1858 | Transmission Transfer Case Contact Plate 'D' Circuit Failure |
| P1859 | Transmission Transfer Case Contact Plate 'D' Open Circuit |
| P1860 | TCC PWM Solenoid Circuit Electrical/ Transmission Transfer Case Contact Plate 'D' Short Circuit To Battery |
| P1861 | Transmission Transfer Case Contact Plate 'D' Short Circuit To Ground |
| P1862 | Transmission Transfer Case Contact Plate Power Circuit Failure |
| P1863 | Transmission Transfer Case Contact Plate Power Open Circuit |
| P1864 | Transmission Transfer Case Contact Plate Power Short To Battery |
| P1865 | Transmission Transfer Case Contact Plate Power Short To Ground |
| P1866 | Transmission Transfer Case System Concern - Servicing Required |
| P1867 | Transmission Transfer Case Contact Plate General Circuit Failure |
| P1868 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Failure |
| P1869 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Short To Battery |
| P1870 | Transmission Component Slipping/ Transmission Mechanical Transfer Case 4x4 Switch Circuit Failure |
| P1871 | Transmission Mechanical Transfer Case 4x4 Switch Circuit Short To Battery |
| P1872 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Failure |
| P1873 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Short To Battery |
| P1874 | Transmission Automatic Hall Effect Sensor Power Circuit Failure |
| P1875 | Transmission Automatic Hall Effect Sensor Power Circuit Short To Battery / 4WD Low Switch Circuit Electrical |
| P1876 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Failure |
| P1877 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Short To Battery |
| P1878 | Transmission Transfer Case Disengaged Solenoid Circuit Failure |
| P1879 | Transmission Transfer Case Disengaged Solenoid Open Circuit |
| P1880 | Transmission Transfer Case Disengaged Solenoid Short to Battery |
| P1881 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1882 | Engine Coolant Level Switch Circuit Short to Ground |
| P1883 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1884 | Engine Coolant Level Lamp Circuit Short to Ground |
| P1885 | Transmission Transfer Case Disengaged Solenoid Short to Ground |
| P1886 | 4X4 Initialization Failure |
| P1890 | Transmission 4WD Mode Select Return Input Circuit Failure |
| P1891 | Transmission Transfer Case Contact Plate Ground Return Open Circuit |
| P1900 | OSS Circuit Intermittent Malfunction |
| P1901 | TSS Circuit Intermittent Malfunction |
| P1902 | Pressure Control Solenoid "B" Intermittent Short |
| P1903 | Pressure Control Solenoid "C" Short Circuit |
| P1904 | Pressure Control Solenoid "C" Open Circuit |
| P1905 | Pressure Control Solenoid "C" Intermittent Short |
| P1906 | Kickdown Pull Relay Open or Short Circuit to Ground |
| P1907 | Kickdown Hold Relay Open or Short Circuit to Ground |
| P1908 | Transmission Pressure Circuit Solenoid Open or Short to Ground |
| P1909 | Trans Temp Sensor Circuit Open or Shorted to Pwr or Gnd |
| P1910 | VFS A Pressure Output Failed Low |
| P1911 | VFS B Pressure Output Failed Low |
| P1912 | VFS C Pressure Output Failed Low |
| P1913 | Pressure Switch A Circuit Malfunction |
| P1914 | Manually Shifted Automatic (MSA) Sw Circuit Malf |
| P1915 | Reverse Switch Circuit Malfunction |
| P1916 | High Clutch Drum Speed Sensor Malfunction |
| P1917 | High Clutch Drum Speed Sensor Intermittent |
| P1918 | Transmission Range Display Circuit Malfunction |

Oι παραπάνω κωδικοί είναι γενικοί για διάφορες κατηγορίες οχημάτων. Έχει ήδη αναφερθεί ότι η δοκιμή του κυκλώματος που σχειδάστηκε έγινε σε όχημα Ford Mondeo 1.8 (2004) βενζίνης και έχουν αναφερθεί και οι κωδικοί σφάλματος που προέκυψαν μετά την τεχνητή βλάβη που προκλήθηκε. Για λόγους πληρότητας ακολουθούν οι βασικοί Ρ – κωδικοί που αφορούν οχήματα της εταιρείας Ford και μέσα στους οποίους περιλαμβάνονται και οι κωδικοί που αναφέρονται στα αποτελέσματα του κεφαλαίου 5:

| P1000 | OBD-II Monitor Testing Incomplete |
| --- | --- |
| P1001 | KOER Test Cannot Be Completed |
| P1039 | Vehicle Speed Signal Missing or Improper |
| P1051 | Brake Switch Signal Missing or Improper |
| P1100 | MAF Sensor Intermittent/ Check of all OBDII Systems Not Complete |
| P1101 | MAF Sensor Out Of Self Test Range |
| P1102 | MAF Sensor In Range But Lower Than Expected |
| P1103 | MAF Sensor In Range But Higher Than Expected |
| P1104 | MAF Ground Malfunction |
| P1105 | Dual Alternator Upper Fault |
| P1106 | Dual Alternator Lower Fault/ Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent High Voltage |
| P1107 | Dual Alternator Lower Circuit Malfunction/ Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent Low Voltage |
| P1108 | Dual Alternator Battery Lamp Circuit Malfunction |
| P1109 | IAT - B Sensor Intermittent |
| P1110 | IAT Sensor (D/C) Open/Short |
| P1111 | Intake Air Temperature (IAT) Sensor Circuit Intermittent High Voltage |
| P1112 | Intake Air Temperature (IAT) Sensor Circuit Intermittent Low Voltage |
| P1113 | IAT Sensor Open/Short |
| P1114 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent Low Voltage/IAT - B Circuit Low Input |
| P1115 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent High Voltage/IAT - B Circuit High Input |
| P1116 | Engine Coolant sensor out of range/ECT Sensor Out Of Self Test Range |
| P1117 | Engine Coolant Sensor intermittent/ECT Sensor Intermittent |
| P1118 | Manifold Absolute Temperature Circuit Low Input |
| P1119 | Manifold Absolute Temperature Circuit High Input |
| P1120 | Throttle position sensor out of range |
| P1121 | Throttle Position (TP) Sensor Circuit Intermittent High Voltage |
| P1122 | Throttle Position (TP) Sensor Circuit Intermittent Low Voltage |
| P1123 | Throttle Position Sensor In Range But Higher Than Expected |
| P1124 | Throttle Position Sensor Out Of Self Test Range |
| P1125 | Throttle position sensor intermittent |
| P1126 | Throttle Position (Narrow Range) Sensor Circuit Malfunction |
| P1127 | Exhaust Not Warm, Downstream O2 Sensor |
| P1128 | Upstream Heated O2 Sensors Swapped |
| P1129 | Downstream Heated O2 Sensors Swapped |
| P1130 | Lack Of HO2S Switch - Adaptive Fuel At Limit |
| P1131 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1132 | Lack Of HO2S Switch - Sensor Indicates Rich |
| P1133 | HO2S Insufficient Switching Sensor 1 |
| P1134 | HO2S Transition Time Ratio Sensor 1 |
| P1135 | Pedal Position Sensor A Circuit Intermittent |
| P1136 | Fan Control Circuit Malfunction |
| P1137 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1138 | Lack Of HO2S12 Switch - Sensor Indicates Rich |
| P1139 | Water In Fuel Indicator Circuit Malfunction |
| P1140 | Water In Fuel Condition |
| P1141 | Fuel Restriction Indicator Circuit Malfunction |
| P1142 | Fuel Restriction Condition |
| P1143 | Air Assist Control Valve Range/Performance |
| P1144 | Air Assist Control Valve Circuit Malfunction |
| P1150 | Lack Of HO2S21 Switch - Adaptive Fuel At Limit |
| P1151 | Lack Of HO2S21 Switch - Sensor Indicates Lean |
| P1152 | Lack Of HO2S21 Switch - Sensor Indicates Rich |
| P1153 | Bank 2 Fuel Control Shifted Lean |
| P1154 | Bank 2 Fuel Control Shifted Rich |
| P1155 | Alternative Fuel Controller |
| P1156 | Fuel Select Switch Malfunction |
| P1157 | Lack Of HO2S22 Switch - Sensor Indicates Lean |
| P1158 | Lack Of HO2S22 Switch - Sensor Indicates Rich |
| P1159 | Fuel Stepper Motor Malfunction |
| P1167 | Invalid Test,throttle not depressed |
| P1168 | Fuel Rail Sensor In-Range Low Failure |
| P1169 | Fuel Rail Sensor In-Range High Failure |
| P1170 | ESO - Engine Shut Off Solenoid Fault |
| P1171 | Rotor Sensor Fault |
| P1172 | Rotor Control Fault |
| P1173 | Rotor Calibration Fault |
| P1174 | Cam Sensor Fault |
| P1175 | Cam Control Fault |
| P1176 | Cam Calibration Fault |
| P1177 | Synchronization Fault |
| P1178 | ( open ) |
| P1180 | Fuel Delivery System Malfunction - Low |
| P1181 | Fuel Delivery System Malfunction - High |
| P1182 | Fuel Shut Off Solenoid Malfunction |
| P1183 | Engine Oil Temperature Circuit Malfunction |
| P1184 | Engine Oil Temperature Out Of Self Test Range |
| P1185 | FTS High - Fuel Pump Temperature Sensor High |
| P1186 | FTS Low - Fuel Pump Temperature Sensor Low |
| P1187 | Variant Selection |
| P1188 | Calibration Memory Fault |
| P1189 | Pump Speed Signal Fault |
| P1190 | Calibration Resistor Out Of Range |
| P1191 | Key Line Voltage |
| P1192 | Voltage External |
| P1193 | EGR Drive Overcurrent |
| P1194 | ECU A/D Converter |
| P1195 | SCP HBCC Failed To Initialize |
| P1196 | Key Off Voltage High |
| P1197 | Key Off Voltage Low |
| P1198 | Pump Rotor Control Underfueling |
| P1199 | Fuel Level Input Circuit Low |
| P1200 | Injector Control Circuit |
| P1201 | Injector Circuit Open / Shorted - Cylinder #1 |
| P1202 | Injector Circuit Open / Shorted - Cylinder #2 |
| P1203 | Injector Circuit Open / Shorted - Cylinder #3 |
| P1204 | Injector Circuit Open / Shorted - Cylinder #4 |
| P1205 | Injector Circuit Open / Shorted - Cylinder #5 |
| P1206 | Injector Circuit Open / Shorted - Cylinder #6 |
| P1209 | Injector Control Pressure System Fault |
| P1210 | Injector Control Pressure Above Expected Level |
| P1211 | Injector Control Pressure Sensor Above / Below Desired |
| P1212 | Injector Control Pressure Not Detected During Crank |
| P1213 | Start Injector Circuit Malfunction |
| P1214 | Pedal Position Sensor B Circuit Intermittent |
| P1215 | Pedal Position Sensor C Circuit Low Input |
| P1216 | Pedal Position Sensor C Circuit High Input |
| P1217 | Pedal Position Sensor C Circuit Intermittent |
| P1218 | CID High |
| P1219 | CID Low |
| P1220 | Series Throttle Control System Malfunction |
| P1221 | Traction Control System Malfunction |
| P1222 | Traction Control Output Circuit Malfunction |
| P1223 | Pedal Demand Sensor B Circuit High Input |
| P1224 | Throttle Position Sensor B Out Of Self Test Range |
| P1225 | Needle Lift Sensor Malfunction |
| P1226 | Control Sleeve Sensor Malfunction |
| P1227 | Wastegate Failed Closed (Over Pressure) |
| P1228 | Wastegate Failed Open (Under Pressure) |
| P1229 | Intercooler Pump Driver Fault |
| P1230 | Fuel Pump Low Speed Malfunction |
| P1231 | Fuel Pump Secondary Circuit Low, High Speed |
| P1232 | Fuel Pump Speed Primary Circuit Malfunction |
| P1233 | Fuel Pump Driver Module Off Line |
| P1234 | Fuel Pump Driver Module Off Line |
| P1235 | Fuel Pump Control Out Of Range |
| P1236 | Fuel Pump Control Out Of Range |
| P1237 | Fuel Pump Secondary Circuit Malfunction |
| P1238 | Fuel Pump Secondary Circuit Malfunction |
| P1239 | Speed Fuel Pump Positive Feed Fault |
| P1240 | Sensor Power Supply Malfunction |
| P1241 | Sensor Power Supply Low Input |
| P1242 | Sensor Power Supply High Input |
| P1243 | Second Fuel Pump Faulty or Ground Fault |
| P1244 | Alternator Load Input Failed High |
| P1245 | Alternator Load Input Failed Low |
| P1246 | Alternator Load Input Failed |
| P1247 | Turbo Boost Pressure Low |
| P1248 | Turbo Boost Pressure Not Detected |
| P1249 | Wastegate Control Valve Performance |
| P1250 | PRC Solenoid Circuit Malfunction |
| P1251 | Air Mixture Solenoid Circuit Malfunction |
| P1252 | Pedal Correlation PDS1 and LPDS High |
| P1253 | Pedal Correlation PDS1 and LPDS Low |
| P1254 | Pedal Correlation PDS2 and LPDS High |
| P1255 | Pedal Correlation PDS2 and LPDS Low |
| P1256 | Pedal Correlation PDS1 and HPDS |
| P1257 | Pedal Correlation PDS2 and HPDS |
| P1258 | Pedal Correlation PDS1 and PDS2 |
| P1259 | Immobilizer to PCM Signal Error |
| P1260 | THEFT Detected, Vehicle Immobilzed |
| P1261 | Cylinder #1 High To Low Side Short |
| P1262 | Cylinder #2 High To Low Side Short |
| P1263 | Cylinder #3 High To Low Side Short |
| P1264 | Cylinder #4 High To Low Side Short |
| P1265 | Cylinder #5 High To Low Side Short |
| P1266 | Cylinder #6 High To Low Side Short |
| P1267 | Cylinder #7 High To Low Side Short |
| P1268 | Cylinder #8 High To Low Side Short |
| P1269 | Immobilizer Code Not Programmed |
| P1270 | Engine RPM Or Speed Limiter Reached |
| P1271 | Cylinder #1 High To Low Side Open |
| P1272 | Cylinder #2 High To Low Side Open |
| P1273 | Cylinder #3 High To Low Side Open |
| P1274 | Cylinder #4 High To Low Side Open |
| P1275 | Cylinder #5 High To Low Side Open |
| P1276 | Cylinder #6 High To Low Side Open |
| P1277 | Cylinder #7 High To Low Side Open |
| P1278 | Cylinder #8 High To Low Side Open |
| P1280 | Injection Control Pressure Out Of Range Low |
| P1281 | Injection Control Pressure Out Of Range High |
| P1282 | Excessive Injection Control Pressure |
| P1283 | IPR Circuit Failure |
| P1284 | Aborted KOER - ICP Failure |
| P1285 | Cylinder head over temp sensed |
| P1286 | Fuel Pulse In Range But Lower Than Expected |
| P1287 | Fuel Pulse In Range But Higher Than Expected |
| P1288 | Cylinder Head Temp Sensor Out Of Self Test Range |
| P1289 | Cylinder Head Temp Sensor High Input |
| P1290 | Cylinder Head Temp Sensor Low Input |
| P1291 | Injector High Side Short To GND Or VBATT - Bank 1 |
| P1292 | Injector High Side Short To GND Or VBATT - Bank 2 |
| P1293 | Injector High Side Open - Bank 1 |
| P1294 | Injector High Side Open - Bank 2/Target idle not reached |
| P1295 | Multi-faults - Bank 1 - With Low Side Shorts |
| P1296 | Multi-faults - Bank 2 - With Low Side Shorts |
| P1297 | Injector High Sides Shorted Together |
| P1298 | IDM Failure |
| P1299 | Cylinder Head Overtemperature Protection Active |
| P1300 | Boost Calibration Fault |
| P1301 | Boost Calibration High |
| P1302 | Boost Calibration Low |
| P1303 | EGR Calibration Fault |
| P1304 | EGR Calibration High |
| P1305 | EGR Calibration Low |
| P1306 | Kickdown Relay Pull - In Circuit Fault |
| P1307 | Kickdown Relay Hold Circuit Fault |
| P1308 | A/C Clutch Circuit Fault |
| P1309 | Misfire Monitor AICE Chip Fault |
| P1313 | Misfire Rate Catalyst Damage Fault - Bank 1 |
| P1314 | Misfire Rate Catalyst Damage Fault - Bank 2 |
| P1315 | Persistent Misfire |
| P1316 | Injector Circuit / IDM Codes Detected |
| P1317 | Injector Circuit / IDM Codes Not Updated |
| P1336 | Crank / Cam Sensor Range / Performance |
| P1340 | Camshaft Position Sensor B Circuit Malfunction |
| P1341 | Camshaft Position Sensor B Range / Performance |
| P1345 | SGC (Cam Position) Sensor Circuit Malfunction/ Crankshaft Position - Camshaft Position Correlation |
| P1346 | Fuel Level Sensor B Circuit Malfunction |
| P1347 | Fuel Level Sensor B Range / Performance |
| P1348 | Fuel Level Sensor B Circuit Low |
| P1349 | Fuel Level Sensor B Circuit High |
| P1350 | Fuel Level Sensor B Intermittent/Bypass Line Monitor |
| P1351 | IDM Input Circuit Malfunction/ Ignition Coil Control Circuit High Voltage |
| P1352 | Ignition Coil A Primary Circuit Malfunction |
| P1353 | Ignition Coil B Primary Circuit Malfunction |
| P1354 | Ignition Coil C Primary Circuit Malfunction |
| P1355 | Ignition Coil D Primary Circuit Malfunction |
| P1360 | Ignition Coil A Secondary Circuit Malfunction |
| P1361 | Ignition Control (IC) Circuit Low Voltage |
| P1362 | Ignition Coil C Secondary Circuit Malfunction |
| P1363 | Ignition Coil D Secondary Circuit Malfunction |
| P1364 | Ignition Coil Primary Circuit Failure |
| P1365 | Ignition Coil Secondary Circuit Failure |
| P1366 | Ignition Spare |
| P1367 | Ignition Spare |
| P1368 | Ignition Spare |
| P1369 | Engine Temperature Light Monitor Failure |
| P1370 | Insufficient RMP Increase During Spark Test |
| P1371 | Ignition Coil - Cylinder 1 - Early Activation Fault |
| P1372 | Ignition Coil - Cylinder 2 - Early Activation Fault |
| P1373 | Ignition Coil - Cylinder 3 - Early Activation Fault |
| P1374 | Crankshaft Position (CKP)/Ignition Coil - Cylinder 4 - Early Activation Fault |
| P1375 | Ignition Coil - Cylinder 5 - Early Activation Fault |
| P1376 | Ignition Coil - Cylinder 6 - Early Activation Fault |
| P1380 | Misfire Detected - Rough Road Data Not Available |
| P1381 | Variable Cam Timing Overadvanced (Bank #1)/ Misfire Detected - No Communication with BCM |
| P1382 | Variable Cam Timing Solenoid #1 Circuit Malfunction |
| P1383 | Variable Cam Timing Overretarded (Bank #1) |
| P1384 | VVT Solenoid A Malfunction |
| P1385 | Variable Cam Timing Solenoid B Malfunction |
| P1386 | Variable Cam Timing Overadvanced (Bank #2) |
| P1387 | Variable Cam Timing Solenoid #2 Circuit Malfunction |
| P1388 | Variable Cam Timing Overretarded (Bank #2) |
| P1389 | Glow Plug Circuit High Side Low Input |
| P1390 | Octane Adjust Pin Out Of Self Test Range |
| P1391 | Glow Plug Circuit Low Input (Bank #1) |
| P1392 | Glow Plug Circuit High Input (Bank #1) |
| P1393 | Glow Plug Circuit Low Input (Bank #2) |
| P1394 | Glow Plug Circuit High Input (Bank #2) |
| P1395 | Glow Plug Monitor Fault (Bank #1) |
| P1396 | Glow Plug Monitor Fault (Bank #2) |
| P1397 | System Voltage Out Of Self Test Range |
| P1398 | VVT Solenoid B Circuit High Input |
| P1399 | Glow Plug Circuit High Side, High Input |
| P1400 | DPFE Circuit Low Input |
| P1401 | DPFE Circuit High Input |
| P1402 | EGR Metering Orifice Restricted |
| P1403 | DPFE Sensor Hoses Reversed |
| P1404 | IAT - B Circuit Malfunction/ Exhaust Gas Recirculation Closed Position Performance |
| P1405 | DPFE Sensor Upstream Hose Off Or Plugged |
| P1406 | Exhaust Gas Recirculation (EGR) Position Sensor Performance |
| P1407 | EGR No Flow Detected |
| P1408 | EGR Flow Out Of Self Test Range |
| P1409 | EVR Control Circuit Malfunction |
| P1411 | SAI System Incorrect Downstream Flow Detected |
| P1413 | SAI System Monitor Circuit Low Input |
| P1414 | SAI System Monitor Circuit High Input |
| P1415 | Air Pump Circuit Malfunction/ (AIR) System Bank 1 |
| P1416 | Port Air Circuit Malfunction/ (AIR) System Bank 2 |
| P1417 | Port Air Relief Circuit Malfunction |
| P1418 | Split Air #1 Circuit Malfunction |
| P1419 | Split Air #2 Circuit Malfunction |
| P1420 | Catalyst Temperature Sensor Failure |
| P1421 | Catalyst Damage |
| P1422 | EGI Temperature Sensor Failure |
| P1423 | EGI Functionality Test Failed |
| P1424 | EGI Glow Plug Primary Failure |
| P1425 | EGI Glow Plug Secondary Failure |
| P1426 | EGI Mini - MAF Failed Out Of Range |
| P1427 | EGI Mini - MAF Failed Short Circuit |
| P1428 | EGI Mini - MAF Failed Open Circuit |
| P1429 | Electric Air Pump Primary Failure |
| P1430 | Electric Air Pump Secondary Failure |
| P1433 | A/C Refrigerant Temperature Circuit Low |
| P1434 | A/C Refrigerant Temperature Circuit High |
| P1435 | A/C Refrigerant Temperature Circuit Range/Performance |
| P1436 | A/C Evaporator Air Temperature Circuit Low |
| P1437 | A/C Evaporator Air Temperature Circuit High |
| P1438 | A/C Evaporator Air Temperature Circuit Range/Performance |
| P1439 | Floor Temperature Switch Circuit Malfunction |
| P1440 | Purge Valve Stuck Open |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Chevrolet Only |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Oldsmobile Only |
| P1442 | Evaporative Emission Control System Leak Detected |
| P1443 | Evaporative Emission Control System Control Valve |
| P1444 | Purge Flow Sensor Circuit Low Input |
| P1445 | Purge Flow Sensor Circuit High Input |
| P1446 | Evaporative Vac Solenoid Circuit Malfunction |
| P1447 | ELC System Closure Valve Flow Fault |
| P1448 | ELC System 2 Fault |
| P1449 | Evaporative Check Solenoid Circuit Malfunction |
| P1450 | Unable To Bleed Up Fuel Tank Vacuum |
| P1451 | Evap Emission Control Sys Vent Control Valve Circuit |
| P1452 | Unable To Bleed - Up Vacuum in Tank |
| P1453 | Fuel Tank Pressure Relief Valve Malfunction |
| P1454 | Evaporative System Vacuum Test Malfunction |
| P1455 | Evap Emission Control Sys Leak Detected (Gross Leak/No Flow) |
| P1456 | Fuel Tank Temperature Sensor Circuit Malfunction |
| P1457 | Unable To Pull Vacuum In Tank |
| P1460 | Wide open throttle A/C cutoff relay circuit |
| P1461 | A/C pressure sensor circuit voltage low |
| P1462 | A/C pressure sensor circuit voltage high |
| P1463 | A/C Pressure Sensor Insufficient Pressure Change |
| P1464 | A/C Demand Out of Self Test Range |
| P1465 | A/C Relay Circuit Malfunction |
| P1466 | A/C Refrigerant Temperature Sensor/Circuit Malfunction |
| P1467 | A/C Compressor Temperature Sensor Malfunction |
| P1468 | SSPOD Open Circuit or Closed Circuit Fault |
| P1469 | Low A/C Cycling Period |
| P1470 | A/C Cycling Period Too Short |
| P1471 | Electrodrive Fan 1 Operational Failure (Driver Side) |
| P1472 | Electrodrive Fan 2 Operational Failure (Passenger Side) |
| P1473 | Fan Secondary High With Fan(s) Off |
| P1474 | Low Fan Control Primary Circuit Malfunction |
| P1475 | Fan Relay (Low) Circuit Malfunction |
| P1476 | Fan Relay (High) Circuit Malfunction |
| P1477 | Additional Fan Relay Circuit Malfunction |
| P1478 | Cooling Fan Driver Fault |
| P1479 | High Fan Control Primary Circuit Malfunction |
| P1480 | Fan Secondary Low with Low Fan On |
| P1481 | Fan Secondary Low With High Fan On |
| P1482 | SCP |
| P1483 | Power To Fan Circuit Overcurrent |
| P1484 | Open Power To Ground VCRM |
| P1485 | EGRV Circuit Malfunction |
| P1486 | EGRA Circuit Malfunction |
| P1487 | EGRCHK Solenoid Circuit Malfunction |
| P1490 | Secondary Air Relief Solenoid Circuit Malfunction |
| P1491 | Secondary Switch Solenoid Circuit Malfunction |
| P1492 | APLSOL Solenoid Circuit Malfunction |
| P1493 | RCNT Solenoid Circuit Malfunction |
| P1494 | SPCUT Solenoid Circuit Malfunction |
| P1495 | TCSPL Solenoid Circuit Malfunction |
| P1500 | Vehicle Speed Sensor Intermittent |
| P1501 | Vehicle Speed Sensor Out Of Self Test Range |
| P1502 | Vehicle Speed Sensor Intermittent Malfunction |
| P1503 | Auxillary Speed Sensor Fault |
| P1504 | Idle Air Control Circuit Malfunction |
| P1505 | Idle Air Control System At Adaptive Clip |
| P1506 | Idle Air Control Overspeed Error |
| P1507 | Idle Air Control Underspeed Error |
| P1508 | Idle Control System Circuit Open |
| P1509 | Idle Control System Circuit Shorted |
| P1510 | Idle Signal Circuit Malfunction |
| P1511 | Idle Switch (Electric Control Throttle) Circuit Malfunction |
| P1512 | Intake Manifold Runner Control (Bank 1) Stuck Closed |
| P1513 | Intake Manifold Runner Control (Bank 2) Stuck Closed |
| P1514 | High Load Neutral/Drive Fault |
| P1515 | Electric Current Circuit Malfunction |
| P1516 | IMRC Input Error (Bank 1) |
| P1517 | IMRC Input Error (Bank 2) |
| P1518 | Intake Manifold Runner Control (Stuck Open) |
| P1519 | Intake Manifold Runner Control (Stuck Closed) |
| P1520 | Intake Manifold Runner Control Circuit Malfunction |
| P1521 | Variable Intake Solenoid #1 Circuit Malfunction |
| P1522 | Variable Intake Solenoid #2 Circuit Malfunction |
| P1523 | IVC Solenoid Circuit Malfunction |
| P1524 | Variable Intake Solenoid System |
| P1525 | Air Bypass Valve System |
| P1526 | Air Bypass System |
| P1527 | Accelerate Warmup Solenoid Circuit Malfunction |
| P1528 | Subsidiary Throttle Valve Solenoid Circuit Malfunction |
| P1529 | SCAIR Solenoid Circuit Malfunction |
| P1530 | A/C Clutch Circuit Malfunction |
| P1531 | Invalid Test - Accelerator Pedal Movement |
| P1532 | IMCC Circuit Malfunction, Bank B |
| P1533 | AAI Circuit Malfunction |
| P1534 | Inertia Switch Activated |
| P1535 | Blower Fan Speed Circuit Range/Performance |
| P1536 | Parking Brake Switch Circuit Failure |
| P1537 | Intake Manifold Runner Control (Bank 1) Stuck Open |
| P1538 | Intake Manifold Runner Control (Bank 2) Stuck Open |
| P1539 | Power To A/C Clutch Circuit Overcurrent |
| P1540 | Air Bypass Valve Circuit Malfunction |
| P1549 | IMCC Circuit Malfunction, Bank B |
| P1550 | PSPS Out Of Self Test Range |
| P1565 | Speed Control Command Switch Out of Range High |
| P1566 | Speed Control Command Switch Out of Range Low |
| P1567 | Speed Control Output Circuit Continuity |
| P1568 | Speed Control Unable to Hold Speed |
| P1571 | Brake Switch Malfunction |
| P1572 | Brake Pedal Switch Circuit Malfunction |
| P1573 | Throttle Position Not Available |
| P1574 | Throttle Position Sensor Disagreement btwn Sensors |
| P1575 | Pedal Position Out of Self Test Range |
| P1576 | Pedal Position Not Available |
| P1577 | Pedal Position Sensor Disagreement btwn Sensors |
| P1578 | ETC Power Less Than Demand |
| P1579 | ETC In Power Limiting Mode |
| P1580 | Electronic Throttle Monitor PCM Override |
| P1581 | Electronic Throttle Monitor Malfunction |
| P1582 | Electronic Throttle Monitor Data Available |
| P1583 | Electronic Throttle Monitor Cruise Disable |
| P1584 | TCU Detected IPE Circuit Malfunction |
| P1585 | Throttle Control Unit Malfunction |
| P1586 | Throttle Control Unit Throttle Position Malfunction |
| P1587 | Throttle Control Unit Modulated Command Malfunction |
| P1588 | Throttle Control Unit Detected Loss of Return Spring |
| P1589 | TCU Unable To Control Desired Throttle Angle |
| P1600 | Loss of KAM Power; Open Circuit |
| P1601 | ECM/TCM Serial Communication Error |
| P1602 | Immobilizer/ECM Communication Error |
| P1603 | EEPROM Malfunction |
| P1604 | Code Word Unregestered |
| P1605 | Keep Alive Memory Test Failure |
| P1606 | ECM Control Relay O/P Circuit Malfunction |
| P1607 | MIL O/P Circuit Malfunction |
| P1608 | Internal ECM Malfunction |
| P1609 | Diagnostic Lamp Driver Fault |
| P1610 | SBDS Interactive Codes |
| P1611 | SBDS Interactive Codes |
| P1612 | SBDS Interactive Codes |
| P1613 | SBDS Interactive Codes |
| P1614 | SBDS Interactive Codes |
| P1615 | SBDS Interactive Codes |
| P1616 | SBDS Interactive Codes |
| P1617 | SBDS Interactive Codes |
| P1618 | SBDS Interactive Codes |
| P1619 | SBDS Interactive Codes |
| P1620 | SBDS Interactive Codes |
| P1621 | Control Module Long Term Memory Performance/ Immobilizer Code Words Do Not Match |
| P1622 | Immobilizer ID Does Not Match |
| P1623 | Immobilizer Code Word/ID Number Write Failure |
| P1624 | Anti Theft System |
| P1625 | B+ Supply To VCRM Fan Circuit Malfunction |
| P1626 | Theft Deterrent Fuel Enable Signal Not Received/ B+ Supply To VCRM A/C Circuit Malfunction |
| P1627 | Module Supply Voltage Out Of Range |
| P1628 | Module Ignition Supply Input Malfunction |
| P1629 | Internal Voltage Regulator Malfunction |
| P1630 | Internal Vref Malfunction |
| P1631 | Theft Deterrent Start Enable Signal Not Correct/ Main Relay Malfunction (Power Hold) |
| P1632 | Smart Alternator Faults Sensor/Circuit Malfunction |
| P1633 | KAM Voltage Too Low |
| P1634 | Data Output Link Circuit Failure |
| P1635 | Tire / Axle Ratio Out of Acceptable Range |
| P1636 | Inductive Signature Chip Communication Error |
| P1637 | Can Link ECM/ABSCM Circuit / Network Malfunction |
| P1638 | Can Link ECM/INSTM Circuit / Network Malfunction |
| P1639 | Vehicle ID Block Corrupted or Not Programmed |
| P1640 | Powertrain DTCs Available in Another Module |
| P1641 | Fuel Pump Primary Circuit Failure |
| P1642 | Fuel Pump Monitor Circuit High Input |
| P1643 | Fuel Pump Monitor Circuit Low Input |
| P1644 | Fuel Pump Speed Control Circuit Malfunction |
| P1645 | Fuel Pump Resistor Switch Circuit Malfunction |
| P1650 | PSP Switch Out of Self Test Range |
| P1651 | PSP Switch Input Malfunction |
| P1652 | IAC Monitor Disabled by PSP Switch Failed On |
| P1653 | Power Steering Output Circuit Malfunction |
| P1654 | Recirculation Override Circuit Malfunction |
| P1655 | Starter Disable Circuit Malfunction |
| P1660 | Output Circuit Check Signal High |
| P1661 | Output Circuit Check Signal Low |
| P1662 | IDM_EN Circuit Failure |
| P1663 | Fuel Demand Command Signal Circuit Malfunction |
| P1667 | CI Circuit Malfunction |
| P1668 | PCM - IDM Communications Error |
| P1670 | Electronic Feedback Signal Not Detected |
| P1680 | Metering Oil Pump Malfunction |
| P1681 | Metering Oil Pump Malfunction |
| P1682 | Metering Oil Pump Malfunction |
| P1683 | Metering Oil Pump Temperature Sensor Circuit Malfunction |
| P1684 | Metering Oil Pump Position Sensor Circuit Malfunction |
| P1685 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1686 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1687 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1688 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1689 | Oil Pressure Control Solenoid Circuit Malfunction |
| P1690 | Wastegate Solenoid Circuit Malfunction |
| P1691 | Turbo Pressure Control Solenoid Circuit Malfunction |
| P1692 | Turbo Control Solenoid Circuit Malfunction |
| P1693 | Turbo Charge Control Circuit Malfunction |
| P1694 | Turbo Charge Relief Circuit Malfunction |
| P1700 | Transmission Indeterminate Failure (Failed to Neutral) |
| P1701 | Reverse Engagement Error |
| P1702 | TRS Circuit Intermittent Malfunction |
| P1703 | Brake Switch Out Of Self Test Range |
| P1704 | Digital TRS Failed to Transition States in KOEO / KOER |
| P1705 | Not in P or N During KOEO / KOER |
| P1706 | High Vehicle Speed Observed in Park |
| P1707 | Transfer Case Neutral Indicator Hard Fault Present |
| P1708 | Clutch Switch Circuit Malfunction |
| P1709 | PNP Switch Out Of Self Test Range |
| P1711 | TFT Sensor Out Of Self Test Range |
| P1712 | Trans Torque Reduction Request Signal Malfunction |
| P1713 | TFT Sensor In Range Failure Low Value |
| P1714 | SSA Inductive Signature Malfunction |
| P1715 | SSB Inductive Signature Malfunction |
| P1716 | SSC Inductive Signature Malfunction |
| P1717 | SSD Inductive Signature Malfunction |
| P1718 | TFT Sensor In Range Failure High |
| P1720 | Vehicle Speed (Meter) Circuit Malfunction |
| P1721 | Gear 1 Incorrect Ratio |
| P1722 | Gear 2 Incorrect Ratio |
| P1723 | Gear 3 incorrect Ratio |
| P1724 | Gear 4 Incorrect Ratio |
| P1725 | Insufficient Engine Speed Increase During Self Test |
| P1726 | Insufficient Engine Speed Decrease During Self Test |
| P1727 | Coast Clutch Solenoid Inductive Signature Malfunction |
| P1728 | Transmission Slip Error |
| P1729 | 4x4 Low Switch Error |
| P1730 | Gear Control Malfunction 2,3,5 |
| P1731 | 1-2 Shift Malfunction |
| P1732 | 2-3 Shift Malfunction |
| P1733 | 3-4 Shift Malfunction |
| P1734 | Gear Control Malfunction |
| P1735 | First Gear Switch Circuit Malfunction |
| P1736 | Second Gear Switch Circuit Malfunction |
| P1737 | Lockup Solenoid System |
| P1738 | Shift Time Error |
| P1739 | Slip Solenoid System |
| P1740 | Torque Converter Clutch Inductive Signature Malfunction |
| P1741 | Torque Converter Clutch Control Error |
| P1742 | Torque Converter Clutch Solenoid Failed On |
| P1743 | Torque Converter Clutch Solenoid Failied On |
| P1744 | Torque Converter Clutch System Performance |
| P1745 | Line Pressure Solenoid System |
| P1746 | Pressure Control Solenoid "A" Open Circuit |
| P1747 | Pressure Control Solenoid "A" Short Circuit |
| P1748 | EPC Malfunction |
| P1749 | Pressure Control Solenoid Failed Low |
| P1751 | Shift Solenoid A Performance |
| P1754 | Coast Clutch Solenoid Circuit Malfunction |
| P1755 | Intermediate Speed Sensor (ISS) Malfunction |
| P1756 | Shift Solenoid B Performance |
| P1760 | Pressure Control Solenoid "A" Short Circuit |
| P1761 | Shift Solenoid C Performance |
| P1762 | Overdrive Band Failed Off |
| P1765 | Timing Solenoid Circuit Malfunction |
| P1767 | Torque Converter Clutch Circuit Malfunction |
| P1768 | Performance / Normal / Winter Mode Input Malfunction |
| P1769 | AG4 Transmission Torque Modulation Fault |
| P1770 | Clutch Solenoid Circuit Malfunction |
| P1775 | Transmission System MIL Fault |
| P1776 | Ignition Retard Request Duration Fault |
| P1777 | Ignition Retard Request Circuit Fault |
| P1778 | Transmission Reverse I/P Circuit Malfunction |
| P1779 | TCIL Circuit Malfunction |
| P1780 | Trans Control Switch (O/D Cancel) Out of Self Test Range |
| P1781 | 4X4 Switch Out of Self Test Range |
| P1782 | P/ES Circuit Out Of Self Test Range |
| P1783 | Transmission Overtemperature Condition |
| P1784 | Transmission Mechanical Failure - First And Reverse |
| P1785 | Transmission Mechanical Failure - First And Second |
| P1786 | 3-2 Downshift Error |
| P1787 | 2-1 Downshift Error |
| P1788 | Pressure Control Solenoid "B" Open Circuit |
| P1789 | Pressure Control Solenoid "B" Short Circuit |
| P1790 | TP (Mechanical) Circuit Malfunction |
| P1791 | TP (Electric) Circuit Malfunction |
| P1792 | Barometer Pressure Circuit Malfunction |
| P1793 | Intake Air Volume Circuit Malfunction |
| P1794 | Battery Voltage Circuit Malfunction |
| P1795 | Idle Switch Circuit Malfunction |
| P1796 | Kick Down Switch Circuit Malfunction |
| P1797 | Neutral Switch Circuit Malfunction |
| P1798 | Coolant Temperature Circuit Malfunction |
| P1799 | Hold Switch Circuit Malfunction |
| P1800 | Transmission Clutch Interlock Safety Switch Circuit Failure |
| P1801 | Transmission Clutch Interlock Safety Switch Open Circuit |
| P1802 | Transmission Clutch Interlock Safety Switch Short Circuit To Battery |
| P1803 | Transmission Clutch Interlock Safety Switch Short Circuit To Ground |
| P1804 | Transmission 4-Wheel Drive High Indicator Circuit Failure |
| P1805 | Transmission 4-Wheel Drive High Indicator Open Circuit |
| P1806 | Transmission 4-Wheel Drive High Indicator Short Circuit To Battery |
| P1807 | Transmission 4-Wheel Drive High Indicator Short Circuit To Ground |
| P1808 | Transmission 4-Wheel Drive Low Indicator Circuit Failure |
| P1809 | Transmission 4-Wheel Drive Low Indicator Open Circuit |
| P1810 | TFP Valve Position Switch Circuit/ Transmission 4-Wheel Drive Low Indicator Short Circuit To Battery |
| P1811 | Transmission 4-Wheel Drive Low Indicator Short Circuit To Ground |
| P1812 | Transmission 4-Wheel Drive Mode Select Circuit Failure |
| P1813 | Transmission 4-Wheel Drive Mode Select Open Circuit |
| P1814 | Transmission 4-Wheel Drive Mode Select Short Circuit To Battery |
| P1815 | Transmission 4-Wheel Drive Mode Select Short Circuit To Ground |
| P1816 | Transmission Neutral Safety Switch Circuit Failure |
| P1817 | Transmission Neutral Safety Switch Open Circuit |
| P1818 | Transmission Neutral Safety Switch Short Circuit To Battery |
| P1819 | Transmission Neutral Safety Switch Short Circuit To Ground |
| P1820 | Transmission Transfer Case Clockwise Shift Relay Coil Circuit Failure |
| P1821 | Transmission Transfer Case Clockwise Shift Relay Coil Open Circuit |
| P1822 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Battery |
| P1823 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Ground |
| P1824 | Transmission 4-Wheel Drive Clutch Relay Circuit Failure |
| P1825 | Transmission 4-Wheel Drive Clutch Relay Open Circuit |
| P1826 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Battery |
| P1827 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Ground |
| P1828 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Circuit Failure |
| P1829 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Open Circuit |
| P1830 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Battery |
| P1831 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Ground |
| P1832 | Transmission Transfer Case Differential Lock-Up Solenoid Circuit Failure |
| P1833 | Transmission Transfer Case Differential Lock-Up Solenoid Open Circuit |
| P1834 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Battery |
| P1835 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Ground |
| P1836 | Transmission Transfer Case Front Shaft Speed Sensor Circuit Failure |
| P1837 | Transmission Transfer Case Rear Shaft Speed Sensor Circuit Failure |
| P1838 | Transmission Transfer Case Shift Motor Circuit Failure |
| P1839 | Transmission Transfer Case Shift Motor Open Circuit |
| P1840 | Transmission Transfer Case Shift Motor Short Circuit To Battery |
| P1841 | Transmission Transfer Case Shift Motor Short Circuit To Ground |
| P1842 | Transmission Transfer Case Differential Lock-Up Feedback Switch Circuit Failure |
| P1843 | Transmission Transfer Case Differential Lock-Up Feedback Switch Open Circuit |
| P1844 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Battery |
| P1845 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Ground |
| P1846 | Transmission Transfer Case Contact Plate 'A' Circuit Failure |
| P1847 | Transmission Transfer Case Contact Plate 'A' Open Circuit |
| P1848 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Battery |
| P1849 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Ground |
| P1850 | Transmission Transfer Case Contact Plate 'B' Circuit Failure |
| P1851 | Transmission Transfer Case Contact Plate 'B' Open Circuit |
| P1852 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Battery |
| P1853 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Ground |
| P1854 | Transmission Transfer Case Contact Plate 'C' Circuit Failure |
| P1855 | Transmission Transfer Case Contact Plate 'C' Open Circuit |
| P1856 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Battery |
| P1857 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Ground |
| P1858 | Transmission Transfer Case Contact Plate 'D' Circuit Failure |
| P1859 | Transmission Transfer Case Contact Plate 'D' Open Circuit |
| P1860 | TCC PWM Solenoid Circuit Electrical/ Transmission Transfer Case Contact Plate 'D' Short Circuit To Battery |
| P1861 | Transmission Transfer Case Contact Plate 'D' Short Circuit To Ground |
| P1862 | Transmission Transfer Case Contact Plate Power Circuit Failure |
| P1863 | Transmission Transfer Case Contact Plate Power Open Circuit |
| P1864 | Transmission Transfer Case Contact Plate Power Short To Battery |
| P1865 | Transmission Transfer Case Contact Plate Power Short To Ground |
| P1866 | Transmission Transfer Case System Concern - Servicing Required |
| P1867 | Transmission Transfer Case Contact Plate General Circuit Failure |
| P1868 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Failure |
| P1869 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Short To Battery |
| P1870 | Transmission Component Slipping/ Transmission Mechanical Transfer Case 4x4 Switch Circuit Failure |
| P1871 | Transmission Mechanical Transfer Case 4x4 Switch Circuit Short To Battery |
| P1872 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Failure |
| P1873 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Short To Battery |
| P1874 | Transmission Automatic Hall Effect Sensor Power Circuit Failure |
| P1875 | Transmission Automatic Hall Effect Sensor Power Circuit Short To Battery / 4WD Low Switch Circuit Electrical |
| P1876 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Failure |
| P1877 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Short To Battery |
| P1878 | Transmission Transfer Case Disengaged Solenoid Circuit Failure |
| P1879 | Transmission Transfer Case Disengaged Solenoid Open Circuit |
| P1880 | Transmission Transfer Case Disengaged Solenoid Short to Battery |
| P1881 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1882 | Engine Coolant Level Switch Circuit Short to Ground |
| P1883 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1884 | Engine Coolant Level Lamp Circuit Short to Ground |
| P1885 | Transmission Transfer Case Disengaged Solenoid Short to Ground |
| P1886 | 4X4 Initialization Failure |
| P1890 | Transmission 4WD Mode Select Return Input Circuit Failure |
| P1891 | Transmission Transfer Case Contact Plate Ground Return Open Circuit |
| P1900 | OSS Circuit Intermittent Malfunction |
| P1901 | TSS Circuit Intermittent Malfunction |
| P1902 | Pressure Control Solenoid "B" Intermittent Short |
| P1903 | Pressure Control Solenoid "C" Short Circuit |
| P1904 | Pressure Control Solenoid "C" Open Circuit |
| P1905 | Pressure Control Solenoid "C" Intermittent Short |
| P1906 | Kickdown Pull Relay Open or Short Circuit to Ground |
| P1907 | Kickdown Hold Relay Open or Short Circuit to Ground |
| P1908 | Transmission Pressure Circuit Solenoid Open or Short to Ground |
| P1909 | Trans Temp Sensor Circuit Open or Shorted to Pwr or Gnd |
| P1910 | VFS A Pressure Output Failed Low |
| P1911 | VFS B Pressure Output Failed Low |
| P1912 | VFS C Pressure Output Failed Low |
| P1913 | Pressure Switch A Circuit Malfunction |
| P1914 | Manually Shifted Automatic (MSA) Sw Circuit Malf |
| P1915 | Reverse Switch Circuit Malfunction |
| P1916 | High Clutch Drum Speed Sensor Malfunction |
| P1917 | High Clutch Drum Speed Sensor Intermittent |
| P1918 | Transmission Range Display Circuit Malfunction |

Τέλος, θα πρέπει να αναφερθεί και η ύπαρξη των B, C και U Trouble Codes οι οποίοι έχουν να κάνουν με Body, Chassis και Network προβλήματα αντίστοιχα αλλά δεν υποστηρίζονται από το ηλεκτρονικό κύκλωμα που σχεδιάστηκε και περιγράφηκε παραπάνω και συνεπώς δεν παρατίθενται στο Παράρτημα Β.

Βιβλιογραφία – Ιστοσελίδες

- ‘Implementing the J1850 Protocol’ – D. John Oliver, Intel Corporation

- ‘Class B Data Communications Network Interface’ - SAE Vehicle Network for Multiplexing and Data Communications Standards Committee, SAE J1850 Standard, May 1994

- ‘Introduction to Vehicle Multiplexing’ – University Consortium for Continuing Education (UCCE), November 1-3, 1995 Training Program

- ‘SAE Standard J18550, Class B Data Communication Network Interface’ – 15 February 1994

- ‘Class B Data Communication Network Messages: Message Definitions for Three Byte Headers’- SAE Vehicle Network for Multiplexing and Data Communications Standards Committee, SAE J2178/4, Issued 1995-02

- ‘General Motors Class 2 Communications Requirements’, Delco Electronics, May 1, 1993

- Wikipedia (http://www.en.wikipedia.org)